/*

* Revision Control Information

*

* $Source: /vol/opua/opua2/sis/sis-1.1/common/src/sis/genlib/RCS/genlib.doc,v $

* $Author: sis $

* $Revision: 1.2 $

* $Date: 1992/05/06 18:53:42 $

*

*/

/* file @(#)genlib.doc	1.1                  */

/* last modified on 5/29/91 at 12:36:49  */

genlib.doc

Richard Rudell

10/28/87

12/29/87 Rev. 1

Cho Moon

12/14/88 Sequential Extension

2/26/90 Revisions

COMBINATIONAL GATE SPECIFICATION

A cell is specified in the following format:

GATE <cell-name> <cell-area> <cell-logic-function>

<pin-info>

.

.

<pin-info>

<cell-name> is the name of the cell in the cell library.  The resulting

net-list will be in terms of these names.

<cell-area> defines the relative area cost of the cell.  It is a floating

point number, and may be in any unit system convenient for the user.

<cell-logic-function> is an equation written in conventional algebraic

notation using the operators '+' for OR, '*' or nothing (space) for AND,

'!' or ''' (post-fixed) for NOT, and parentheses for grouping.  The

names of the literals in the equation define the input pin names

for the cell; the name on the left hand side of the equation defines

the output of the cell.  The equation terminates with a semicolon.

Only single-output cells may be specified.  The '!' operator may only

be used on the input literals, or on the final output; it is not

allowed internal to an expression.  (This constraint may disappear in

the future).

Also, the actual factored form is significant when a logic function has

multiple factored forms.  In principle, all factored forms could be

derived for a given logic function automatically; this is not yet

implemented, so each must be specified separately.  Note that factored

forms which differ by a permutation of the input variables (or by De

Morgan's law) are not considered unique.

Each <pin-info> has the format:

PIN <pin-name> <phase> <input-load> <max-load>

<rise-block-delay> <rise-fanout-delay>

<fall-block-delay> <fall-fanout-delay>

<pin-name> must be the name of a pin in the <cell-logic-function>,

or it can be * to specify identical timing information for all pins.

<phase> is INV, NONINV, or UNKNOWN corresponding to whether the

logic function is negative-unate, positive-unate, or binate in

this input variable respectively.  This is required for the separate

rise-fall delay model.  (In principle, this information is easily

derived from the logic function; this field may disappear in the future).

<input-load> gives the input load of this pin.  It is a floating point

value, in arbitrary units convenient for the user.

<max-load> specifies a loading constraint for the cell.  It is a

floating point value specifying the maximum load allowed on the output.

<rise-block-delay> and <rise-fanout-delay> are the rise-time parameters

for the timing model.  They are floating point values, typically in

the units nanoseconds, and nanoseconds/unit-load repsectively.

<fall-block-delay> and <fall-fanout-delay> are the fall-time parameters

for the timing model.  They are floating point values, typically in

the units nanoseconds, and nanoseconds/unit-load repsectively.

All of the delay information is specified on a pin-by-pin basis.  The

meaning is the delay information for the most critical pin is used

to determine the delay for the gate.

LATCH SPECIFICATION

Latches are specified as follows:

LATCH <cell-name> <cell-area> <cell-logic-function>

<pin-info>

.

.

<pin-info>

<latch-spec>

[<clock-delay-info>]

[<constraint-info>]

<cell-name>, <cell-area>, <cell-logic-function> and <pin-info>

are the same as in the combinational case.

<latch-spec> is required  but <constraint-info> is optional.

<clock-delay-info> is necessary for synchronous latches and

flip-flops.

<latch-spec> has the following format:

SEQ <latch-input> <latch-output> <latch-type>

<latch-input> must be the name of the output of the cell.

Thus, <latch-input> is described as a function of <latch-output>

in the <cell-logic-function>.

A special name "ANY" can be used for <latch-output> to specify

to the mapper that <latch-output> needs not be a signal name in the

<cell-logic-function>.    This is useful for describing D-type flip-flops

and latches.  <latch-type> can be ACTIVE_HIGH or ACTIVE_LOW for transparent latches;

RISING_EDGE or FALLING_EDGE for edge-triggered flip-flops; or ASYNCH

for asychronous latches.

<clock-delay-info> gives the propagation delay from the clock

pin to the output.   Its format is as follows:

CONTROL <clock-pin-name> <input-load> <max-load>

<rise-block-delay> <rise-fanout-delay>

<fall-block-delay> <fall-fanout-delay>

<constraint-info> gives the setup and hold time constraints

with respect to the clock signal.   The format is:

CONSTRAINT <pin-name> <setup-time> <hold-time>

If not specified, the values of 0.0 are assumed.

---------------------------------------------------------------------------

Example 0 of latch specification

LATCH "d-latch"	80	Q=D;

PIN D NONINV 1 999 1 .2 1 .2

SEQ Q ANY ACTIVE_HIGH

CONTROL CLK 1 999 1 .2 1 .2

CONSTRAINT D 0.2 0.2

---------------------------------------------------------------------------

Example 1 of <cell-logic-function>

O = !(!I1 * I2);

is legal and defines a NAND gate with 1 inverted input; it could also

be specified as

O = I1 + !I2;

There is no advantage or disadvantage to either representation; only 1

need be given.

---------------------------------------------------------------------------

Example 2 of <cell-logic-function>

O = !((I1 * I2) + !(I3 + I4));

is incorrectly specified.  It must be re-written as either:

O = !((I1 * I2) + (!I3 * !I4));

or

O = (!I1 + !I2)*(I3 + I4);

Again, there is no advantage or disadvantage to either representation as

they differ only by De Morgan's law.  Only 1 need be given.  Note that

there are no other factored forms for this equation.

---------------------------------------------------------------------------

Example 3 of <cell-logic-function>

O = a*b + a*c + b*c;

is one specification of a majority gate.  Another possible specification

(which IS considered different) is:

O = a*(b + c) + b*c;

Any permutation of the input variables does not provide a new specification;

hence, these are the only two useful representations for a majority

gate logic function.  Both should be provided to the technology mapping

program, until further notice.