/*

* Revision Control Information

*

* $Source: /vol/opua/opua2/sis/sis-1.2/common/src/sis/map/RCS/map.doc,v $

* $Author: sis $

* $Revision: 1.3 $

* $Date: 1993/05/20 13:43:12 $

*

*/

/* file @(#)map.doc	1.2 */

/* last modified on 5/1/91 at 15:52:15 */

network_t *

map_network(network, library, mode, inv_optz, fanout_flag)

network_t *network;

library_t *library;

double mode;

int inv_optz;

int fanout_flag;

Implement 'network' using gates from library.  Uses the

tree-based technology mapping algorithm.  Returns NIL(network_t)

if any error, in which case error_string() contains a description

of what went wrong.

If 'mode' is '0', the area will be minimized.

If 'mode' is '1', the delay will be minimized.

If 'mode' is '2', the delay will be minimized on an estimate of the

critical path obtained from a trivial 2-input nand mapping.

If 'mode' is between '0' and '1', then a linear combination of the area

and delay is minimized.

Setting 'inv_optz' to '1' ('0'), enables (disables) the

inverter optimization

'fanout_flag' is a mask of 4 flags: [b0,b1,b2,b3]. b0 is the least

significant bit of the word. For example, if fanout_flag equals 3,

this means that b0 and b1 are on, b2 and b3 off.

Setting 'b0' means that the area at a fanout point is estimated

as being the area of the subtree divided by the number of fanouts.

Otherwise, the area is assumed to be 0 at a multiple fanout point.

Setting 'b1' means that the mapper is allowed to introduce overlaps

(i.e. logic duplication).

'b2' is unused.

Setting 'b3' allows multiple fanouts within gates (i.e. for XOR's, MUX'es).

The default value is 11 in area mode and 9 in delay mode.

void

map_add_inverter(network, add_at_pipo)

network_t *network;

int add_at_pipo;

Add extra inverters nodes to the network such that every pair

of nand gates is separated by 2 or more inverters.

Setting 'add_at_pipo' to 1 enables the insertion also at primary inputs

and outputs.

void

map_remove_inverter(network, report_data)

network_t *network;

VoidFn report_data;

Remove redundant inverters from the network while keeping the

network mapped.  If report_data is not NIL, the routine reports

debugging information by calling the report_data function with the

network as an argument.