/*

* Revision Control Information

*

* $Source: /vol/opua/opua2/sis/sis-1.1/common/src/sis/decomp/RCS/decomp.doc,v $

* $Author: sis $

* $Revision: 1.2 $

* $Date: 1992/05/06 18:51:59 $

*

*/

Summary:

decomp_quick_network();

decomp_good_network();

decomp_disj_network();

decomp_quick_node();

decomp_good_node()

decomp_disj_node()

decomp_quick();

decomp_good();

decomp_disj();

void

decomp_quick_network(network)

network_t *network;

Decompose all functions in the network  using quick

decomposition algorithm (QD).  See MIS paper for the

complete description of the algorithm.  The new

functions are inserted into the network.

void

decomp_good_network(network)

network_t *network;

Decompose all functions in the network  using good

decomposition algorithm (GD).  See MIS paper for the

complete description of the algorithm.  The new

functions are inserted into the network.

void

decomp_disj_network(network)

network_t *network;

Decompose all functions in the network  using disjoint

decomposition algorithm (DD).  See decomp_disj for

for information about the algorithm.  The new functions

are inserted into the network.

void

decomp_tech_network(network, and_limit, or_limit)

network_t *network;

Decomposes the network into AND and OR gates where the

number of fanins of AND gates is less than or equal to

and_limit and that of OR gates is less than or equal to

or_limit.  If and_limit (or_limit) is less than or equal

to 0, the resulting network only consists of OR (AND)

gates.  Calling this routine with both and_limit and

or_limit less than or equal to 0 is a series error.

THIS ROUTINE DOES NOT DO KERNEL DECOMPOSITION.

void

decomp_quick_node(network, f)

network_t *network;

node_t *f;

Decompose f using quick decomposition algorithm (QD).

See MIS paper for the complete description of the

algorithm.  The new functions are inserted into the

network.

void

decomp_good_node(network, f)

network_t *network;

node_t *f;

Decompose f using good decomposition algorithm (GD).

See MIS paper for the complete description of the

algorithm.  The new functions are inserted into the

network.

void

decomp_disj_node(network, f)

network_t *network;

node_t *f;

Decompose f using disjoint decomposition algorithm (DD).

See decomp_disj for more information about the

algorithm.  The new functions are inserted into the

network.

array_t *

decomp_quick(f)

node_t *f;

Returns an array of nodes which forms the decomposition

of f using quick decomposition algorithm (QD).  See MIS

paper for the complete description of the algorithm.

f does not have to be in the network.  The root of the

decomposition is guaranteed to be the first element in

the result.

array_t *

decomp_good(f)

node_t *f;

Returns an array of nodes which forms the decomposition

of f using good decomposition algorithm (GD).  See MIS

paper for the complete description of the algorithm.

f does not have to be in the network. The root of the

decomposition is guaranteed to be the first element in

the result.

array_t *

decomp_disj(f)

node_t *f;

Returns an array of nodes which forms the decomposition

of f using disjoint decomposition algorithm (DD).

It partitions the cubes of f into sets of cubes having

disjoint variable support and creates a node for each

partition of cubes and a node (the new f) which is the

OR of all the partitions.  f does not have to be in the

network.  The root of the decomposition is guaranteed to

be the first element in the result.