/*

* Revision Control Information

*

* $Source$

* $Author$

* $Revision$

* $Date$

*

*/

This is a package that is used for clock verification and optimal clock

computation. The circuit is assumed to have memory elements that can

i) flip-flops (falling or rising edge triggered)

ii) level-sensitive latches (active high or active low).

The gate model is a pin-to-pin delay model. Timing analysis uses static delay

computation. Each gate can have only one output. The clocking scheme has the

following restrictions

i) all phases must have the same frequency. In other words in a time interval

equal to the clock cycle, each phase must rise and fall only once.

ii) signals to the control inputs of memory elements have no logic associated

, i.e. they are one of the clock phases

iii) Clock skews if any must be provided by the user using a pre-defined

routine in the package.

The package solve 2 problem associated with clocking

i) Clock verification- given a circuit and a proposed clocking scheme (i.e.

the clock cycle + rise /fall times of all the phases) check if the set-up and

hold constraints are met at every memory element.

All primary inputs are assumed to arrive at t= 0 by default, relative to

the clock schedule; if the arrival time for a PI is set then it is used (i.e.

it gets tagged onto the edge weights).

ii) Optimal clocking- given a circuit, relative ordering of the phase and

possible duty cycle constraints, find the minimum clock cycle and times for

the rise/fall of clock phases so that the circuit operates without any

set-up/hold errors.

An issue yet to be resolved is the delay model for gates. The pin-to-pin model

is used to come up with min and max delays from a latch to another. The issues

of how to handle CLK pin to DATA output pin, DATA in to DATA output pin are

still open. You are welcome to email your suggestions to me. Please note that

I may agree with what you say but the fixes will be only after making sure

that the model does not contradict any other assumptions in SIS over which I

have no control!

The timing package currently does not export any routines since they

assume a specific clocking scheme and specific circuit.

The code for optimal clock computation can be speeded up considerably

if you have the octtools package and have the priority queue package (pq).

This enables certain heuristics to be used efficiently and the clock lower

bound computation often finishes much faster.

BUGS: send mail to shenoy@ic.berkeley.edu