/*

* Revision Control Information

*

* $Source: /vol/opua/opua2/sis/sis-1.2/common/src/sis/resub/RCS/resub.doc,v $

* $Author: sis $

* $Revision: 1.3 $

* $Date: 1993/05/11 22:22:48 $

*

*/

Summary:

resub_alge_node();

resub_alge_network();

int

resub_alge_node(f, use_complement)

node_t *f;

int use_complement;

Algebraic re-substitution of f into all other nodes in the network.

It returns 1 if f is substituted into some other notes, 0 otherwise.

If use_complement is 1, the substitution uses both f and its complement.

Because it uses algebraic division, only the immediate fanouts of the

immediate fanins of f can possibly be simplified.  No action is taken

if f is a primary input, a primary output, or a

constant function.

void

resub_alge_network(network, use_complement)

network_t *network;

int use_complement;

Does resub_alge_node on each function in the network until no more

improvement can be made.