TD <21TD1xx>

ETSI TS 101 321-3 v3.0.3 (2002-04)

Technical Specification

Telecommunications and Internet Protocol

Harmonization Over Networks (TIPHON);

Open Settlement Protocol (OSP) for

Inter-Domain pricing, authorization,

and usage exchange

Reference

RTS/TIPHON-03004.3

Keywords

internet, network, interoperability, protocol, telephony, IP

ETSI

650 Route des Lucioles

F-06921 Sophia Antipolis Cedex - FRANCE

Tel.: +33 4 92 94 42 00   Fax: +33 4 93 65 47 16

Siret N° 348 623 562 00017 - NAF 742 C

Association à but non lucratif enregistrée à la

Sous-Préfecture de Grasse (06) N° 7803/88

Important notice

Individual copies of the present document can be downloaded from:
http://www.etsi.org

The present document may be made available in more than one electronic version or in print. In any case of existing or perceived difference in contents between such versions, the reference version is the Portable Document Format (PDF). In case of dispute, the reference shall be the printing on ETSI printers of the PDF version kept on a specific network drive within ETSI Secretariat.

Users of the present document should be aware that the document may be subject to revision or change of status. Information on the current status of this and other ETSI documents is available at http://www.etsi.org/tb/status/

If you find errors in the present document, send your comment to:
editor@etsi.fr

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© European Telecommunications Standards Institute 2000.

All rights reserved.

Contents

Intellectual Property Rights	7

Foreword	7

Introduction	7

1	Scope	8

2	References	8

3	Definitions and abbreviations	10

3.1	Abbreviations	10

4	Open Settlement Protocol Architecture	10

4.1	Communication Protocols	10

4.1.1	Secure Sockets Layer/Transport Layer Security	11

4.1.2	Hypertext Transfer Protocol	11

4.2	Message Format	11

4.2.1	Multipurpose Internet Mail Extensions	12

4.2.2	Extensible Markup Language	12

4.2.3	Secure MIME	12

5	Protocol Profiles	13

5.1	Secure Sockets Layer/Transport Layer Security	13

5.1.1	Protocol Version	13

5.1.2	Client/Server Roles	13

5.1.3	CipherSuites	13

5.2	Hypertext Transfer Protocol	13

5.2.1	Protocol Version	13

5.2.2	Client/Server Roles	13

5.2.3	TCP Port	13

5.2.4	HTTP Methods	14

5.2.5	Uniform Resource Identifier	14

5.2.6	HTTP Headers	14

5.2.7	HTTP Entity Body	14

6	XML Content	14

6.1	Document Structure	14

6.1.1	Multipurpose Internet Mail Extensions Conformance	14

6.1.1.1	Content-Type	14

6.1.1.2	Content-Length	14

6.1.1.3	Transfer Encoding	15

6.1.2	XML Conformance	15

6.1.2.1	XML Version	15

6.1.2.2	Well-Formed Constraint	15

6.1.2.3	Character Encoding	15

6.1.3	XML Framework	15

6.1.3.1	Root Entity	16

6.1.3.2	Random Attribute	16

6.1.3.3	Identifier Attribute	16

6.1.3.4	Critical Attribute	16

6.1.3.5	Extensions	16

6.2	Components	17

6.2.1	PricingIndication	17

6.2.2	PricingConfirmation	17

6.2.3	AuthorizationRequest	18

6.2.4	AuthorizationResponse	18

6.2.5	AuthorizationIndication	19

6.2.6	AuthorizationConfirmation	19

6.2.7	UsageIndication	19

6.2.8	UsageConfirmation	19

6.2.9	ReauthorizationRequest	20

6.2.10	ReauthorizationResponse	20

6.2.11	SubscriberAuthenticationRequest	20

6.2.12	SubscriberAuthenticationResponse	20

6.2.13	CapabilitiesIndication	20

6.2.14	CapabilitiesConfirmation	21

6.3	Elements	21

6.3.1	Amount	21

6.3.2	AuthorityURL	21

6.3.3	CallId	21

6.3.4	Code	22

6.3.5	Currency	23

6.3.6	Description	23

6.3.7	Destination	23

6.3.8	DestinationAlternate	23

6.3.9	DestinationInfo	23

6.3.10	DestinationSignalAddress	24

6.3.11	Increment	24

6.3.12	MaximumDestinations	25

6.3.13	Role	25

6.3.14	Service	25

6.3.15	SourceAlternate	25

6.3.16	SourceInfo	26

6.3.17	SourceSignalAddress	26

6.3.18	Status	26

6.3.19	Timestamp	26

6.3.20	Token	27

6.3.21	TransactionId	27

6.3.22	Unit	27

6.3.23	UsageDetail	27

6.3.24	ValidAfter	28

6.3.25	ValidUntil	28

6.3.26	EndTime	28

6.3.27	StartTime	28

6.3.28	TCCode	28

6.3.29	TerminationCause	30

6.3.30	Certificate	30

6.3.31	CertificateChain	30

6.3.32	OSPCapability	31

6.3.33	OSPService	31

6.3.34	OSPServiceURL	31

6.3.35	OSPSignatureRequired	31

6.3.36	OSPVersion	32

6.3.37	SubscriberAuthenticationInfo	32

6.3.38	DeviceInfo	32

6.3.39	DeviceId	32

6.3.40	Resources	32

6.3.41	DataRate	32

6.3.42	NumberOfChannels	33

6.3.43	Bandwidth	33

6.3.44	AlmostOutOfResources	33

6.3.45	DestinationProtocol	33

6.3.46	QualityOfService	33

6.3.47	QoSClass	33

6.3.48	QoSParameters	34

6.3.49	Group	34

6.3.50	GroupId	34

6.3.51	SessionId	34

6.3.52	MultiMediaCallId	35

6.3.53	MMCId	35

6.3.54	ServiceType	35

7	Signature Format	35

7.1	Canonical Form	35

7.2	Signature Algorithms	36

7.3	Transfer Encoding	36

8	Protocol Behaviour	36

8.1	Message Sequencing	36

8.2	Exception Handling	36

8.2.1	Transmission Control Protocol	38

8.2.2	Secure Socket Layer/Transport Layer Security	38

8.2.3	Hypertext Transfer Protocol	39

8.2.4	Status Element	39

8.3	Transaction types	39

8.3.1	Group behaviour	40

8.3.2	Group identifiers	42

Annex A (normative): Document Type Definition	44

Annex B (normative): Cryptographic Algorithms	47

B.1	SSL/TLS CipherSuites	47

B.2	S/MIME Signatures	47

B.3	Tokens	47

Annex C (normative): Enhanced Usage Reports	48

C.1	Enhanced Usage Elements	48

C.1.1	Statistics	48

C.1.2	LossSent	48

C.1.3	Packets	48

C.1.4	Fraction	48

C.1.5	LossReceived	48

C.1.6	OneWayDelay	48

C.1.7	Minimum	49

C.1.8	Mean	49

C.1.9	Variance	49

C.1.10	Samples	49

C.1.11	RoundTripDelay	49

C.1.12	Maximum	49

C.1.13	Delay	49

C.1.14	Jitter	49

C.1.15	PackLoss	50

Annex D (informative): Token Formats	51

D.1	Cryptographic Encoding	51

D.2	Token Content	52

D.2.1	ASN.1 Format	52

D.2.2	XML Format	53

D.2.3	Binary XML Format	53

D.3	Token Carriage	53

D.4	Sample Token	55

Annex E (informative): Example Messages	56

E.1	Pricing Exchange	56

E.2	Authorization Exchange	58

E.3	Usage Exchange	60

E.4	Subscriber Authentication Exchange	61

E.5	Capabilities Exchange	62

Annex F (informative): Billing Format Conversion	64

Annex G (informative): XML Overview	68

G.1	Document Definition	68

G.2	Element Declaration	68

G.3	Attribute Declaration	69

Annex H (informative): Binary XML Content Format for OSP	70

H.1	Global Extension Tokens	71

H.2	Example Application	73

H.2.1	Standard XML Format (505 bytes)	73

H.2.2	Binary XML Content Format (160 bytes)	73

Annex J (informative): OSP Applications and Implementations	93

J.1	Call Control Protocols	93

J.1.1	Peer-to-Peer Architecture	93

J.1.1.1	H.323    Gateways	93

J.1.1.1.1	Call Routing and Authorization	94

J.1.1.1.2	Usage Reports	96

J.1.1.2	Session Initiation Protocol Gateways	97

J.1.1.2.1	Call Routing and Authorization	98

J.1.1.2.2	Usage Reports	100

J.1.2	Tightly Controlled Distributed Architecture	101

J.1.2.1	H.323    Gatekeeper Routed Calls	102

J.1.2.1.1	Call Routing and Authorization	102

J.1.2.1.2	Usage Reports	104

J.1.2.2	Session Initiation Protocol Proxy Servers	105

J.1.2.2.1	Call Routing and Authorization	106

J.1.2.2.2	Usage Reports	108

J.1.3	Loosely Controlled Distributed Architecture	109

J.1.3.1	H.323    Direct Routed Calls (with Gatekeepers)	110

J.1.3.1.1	Call Routing and Authorization	110

J.1.3.1.2	Usage Reports	112

J.1.3.2	Session Initiation Protocol Redirect Servers	114

J.1.3.2.1	Call Routing and Authorization	115

J.1.3.2.2	Usage Reports	117

J.2	Prepaid Calling Card and Roaming User Support	120

J.2.1	Call Routing and Authorization	121

J.2.2	Reauthorization	122

Annex K (informative):	History of OSP codes	126

K.1	Code 2XX	126

K.2	Code 200	126

K.3	Code 4XX	126

K.4	Code 400	127

K.5	Code 401	127

K.6	Code 402	127

K.7	Code 403	128

K.8	Code 404	128

K.9	Code 405	128

K.10	Code 428	129

K.11	Code 441	129

K.12	Code 447	129

K.13	Code 449	129

K.14	Code 450	130

K.15	Code 463	130

K.16	Code 481	130

K.17	Code 488	130

K.18	Code 495	131

Bibliography	132

History	133

# Intellectual Property Rights

IPRs essential or potentially essential to the present document may have been declared to ETSI. The information pertaining to these essential IPRs, if any, is publicly available for ETSI members and non-members, and can be found in ETSI SR 000 314: "Intellectual Property Rights (IPRs); Essential, or potentially Essential, IPRs notified to ETSI in respect of ETSI standards", which is available from the ETSI Secretariat. Latest updates are available on the ETSI Web server (http://www.etsi.org/ipr).

Pursuant to the ETSI IPR Policy, no investigation, including IPR searches, has been carried out by ETSI. No guarantee can be given as to the existence of other IPRs not referenced in ETSI SR 000 314 (or the updates on the ETSI Web server) which are, or may be, or may become, essential to the present document.

# Foreword

This Technical Specification (TS) has been produced by ETSI Project Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON).

# Introduction

The contents of the present document are the result of contributions and discussions in Working Group 3.

# 1	Scope

The present document shall be called the Open Settlement Protocol (OSP). The present document specifies a set of protocols and associated profiles to permit the exchange of inter-domain pricing, authorization, and settlement information between internet telephony operators. The protocols specified fulfil the essential requirements of such services, by providing appropriate functionality between multiple administrative domains in a secure manner. The specification also provides for non-standard extensions that permit co-operating parties to augment or replace the basic functionality.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.

- A non-specific reference to an ETS shall also be taken to refer to later versions published as an EN with the same number.

[1]	American National Standards Institute. Accredited Standards Committee X9 Working Draft: American National Standard X9.42-1993: Public Key Cryptography for the Financial Services Industry: Management of Symmetric Algorithm Keys Using Diffie-Hellman. American Bankers Association, September 21, 1994.

[2]	RFC 1945 (1996): Hypertext Transfer Protocol - HTTP/1.0. Berners Lee, T., R. Fielding, and H. Frystyk.

[3]	Bray, Tim, Jean Paoli, and C. M. Sperberg-McQueen. Extensible Markup Language (XML) 1.0. World Wide Web Consortium (W3C): 10 February 1998. [http://www.w3.org/TR/REC-xml].

[4]	RFC 2068 (1997): Hypertext Transfer Protocol - HTTP/1.1. Fielding R., J. Gettys, J. Mogul, H. Frystyk, and T. Berners-Lee.

[5]	RFC 2045 (1996): Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies. Freed, N. and N. Borenstein.

[6]	Freier, Alan O., Philip Karlton, and Paul C. Kocher. The SSL Protocol Version 3.0 [http://www.netscape.com/eng/ssl3/ssl-toc.html]. Netscape Communications Corporation: March 1996. As amended by SSL 3.0 Errata of August 26, 1996 [http://www.netscape.com/eng/ssl3/ssl-errata.html].

[7]	ISO 4217 (1995): "Codes for the representation of currencies and funds".

[8]	ISO 8601 (1988): "Data elements and interchange formats -- Information
interchange -- Representation of dates and times".

[9]	ITU-T Recommendation H.225.0 (1998): "Call signalling protocols and media stream packetization for packet-based multimedia communication systems".

[10]	ITU-T Recommendation H.245 (1998): "Control protocol for multimedia communication".

[11]	ITU-T Recommendation X.691 (1995): "Information technology - ASN.1 encoding rules - Specification of Packed Encoding Rules (PER)".

[12]	ITU-T Recommendation E.164 (1997): "The international public telecommunication numbering plan".

[13]	ITU-T Recommendation H.323 (1998): "Packet-based multimedia communications systems".

[14]	ITU-T Recommendation H.235 (1998): "Security and encryption for H-Series (H.323 and other H.245-based) multimedia terminals".

[15]	National Institute of Standards and Technology, U.S. Department of Commerce NIST FIPS PUB 46-1 (January 1988): "Data Encryption Standard".

[16]	National Institute of Standards and Technology, U.S. Department of Commerce NIST FIPS PUB 186 (18 May 1994): "Digital Signature Standard"

[17]	National Institute of Standards and Technology, U.S. Department of Commerce NIST FIPS PUB 180-1 (31 May 1994): "Secure Hash Standard".

[18]	RFC 2311 (1998): S/MIME Version 2 Message Specification. S. Dusse, P. Hoffman, B. Ramsdell, L. Lundblade, and L. Repka.

[19]	RFC 2268 (1998): Description of the RC2® Encryption Algorithm. R. Rivest.

[20]	RFC 1321 (1992): The MD5 Message-Digest Algorithm. R. Rivest.

[21]	RSA Laboratories. PKCS #1: RSA Encryption Standard. Version 1.5, November 1993.

[22]	RSA Laboratories. PKCS #7: Cryptographic Message Syntax Standard. Version 1.5, November 1993.

[23]	The Unicode Consortium. The Unicode Standard. Version 2.0.

[24]	Dierks, Tim and Christopher Allen. The TLS Protocol Version 1.0. Work in progress.

[25]	The Open Trading Protocol Consortium. Internet Open Trading Protocol Part 2: Specification. Version 0.9, 12 January 1998.

[26]	ISO/IEC 9646-7 (1995): "Information technology -- Open Systems Interconnection -- Conformance testing methodology and framework -- Part 7: Implementation Conformance Statements".

[27]	ISO/IEC 10646 (1993): "Information technology -- Universal Multiple-Octet Coded Character Set (UCS)".

[28]	RFC 2630 (1999): Cryptographic Message Syntax. Housley, R.

[29]	WAP-154, Binary XML Content Format Specification

[30]	ISO/IEC 7812-1: "Identification cards -- Identification of issuers -- Part 1: Numbering system".

[31]	ETSI TS 101 329-2: "End-to-end Quality of Service in TIPHON systems-- Part 2: Definition of speech Quality of Service".

[32]	ETSI TS 101 336: “Open Settlement Protocol” Conformance Test Specifications; Part 1: Profile Implementation Conformance Statement (ICS) Proforma.

[33]	RFC 3261: “Session Initiation Protocol”, June 2002.

[34]	DTS-06010-1: “Open Settlement Protocol” Conformance Test Specifications; Part 1: Profile Implementation Conformance Statement (ICS) Proforma.

[35]	RFC 2327: “Session Description Protocol”, April 1998.

# 3	Definitions and abbreviations

## 3.1	Abbreviations

For the purposes of the present document, the following abbreviations apply:

CMS	Cryptographic Message Syntax

DES	Data Encryption Standard

DSA	Digital Signature Algorithm

DTD	Document Type Definition

HTML	Hypertext Markup Language

HTTP	Hypertext Transfer Protocol

IETF	Internet Engineering Task Force

IP	Internet Protocol

IPSEC	Internet Protocol Security

MD5	Message Digest 5

MIME	Multipurpose Internet Mail Extensions

OSP	Open Settlement Protocol

PIN	Personal Identification Number (e.g. for automated teller machines)

PKCS	Public Key Cryptography Standard

RAS	Registration Admission and Status

RSA	Rivest Shamir Adleman

SHA	Secure Hash Algorithm

S/MIME	Secure Multipurpose Internet Mail Extensions

SSL	Secure Socket Layer

TCP	Transmission Control Protocol

TLS	Transport Layer Security

URL	Uniform Resource Locator

UTC	Universal Time Co-ordinated

UTF	Universal Text Format

XML	Extensible Markup Language

# 4	Open Settlement Protocol Architecture

This clause introduces the protocol architecture for the OSP specification. It identifies the major protocols used by communicating parties, and it outlines their relationship to each other. The clause also describes the overall format of messages exchanged by the protocols. The intent of this clause is to outline the framework for the standard’s protocols and message formats; later clauses detail specific profiles for these protocols and the specific message content.

## 4.1	Communication Protocols

As figure 1 shows, systems conforming to the OSP specification use a combination of the Hypertext Transfer Protocol (HTTP), and either the Secure Sockets Layer (SSL) or Transport Layer Security (TLS) to transfer pricing, authorization, and usage information. As the figure indicates, these protocols are layered on top of the Transmission Control Protocol (TCP) for communication across Internet Protocol (IP) networks.

Figure 1: Open Settlement Protocol Architecture for Pricing,
 Authorization, and Usage Exchange

### 4.1.1	Secure Sockets Layer/Transport Layer Security

The Secure Sockets Layer and Transport Layer Security protocols add authentication and privacy to TCP connections. SSL is the standard protocol for securing web browsing. As such, it is widely deployed on the Internet and is distinguished by considerable operational experience. SSL also enjoys near universal support from firewalls and proxy servers. TLS is an updated version of SSL currently being developed within the Internet Engineering Task Force (IETF). TLS is heavily based on SSL and, although it is not strictly backwards compatible with SSL, systems supporting both TLS and SSL can automatically recognize either protocol and adapt as required to ensure interoperability.

NOTE:	As other industry standard mechanisms for IP-based security (for example, IPSEC) reach maturity, later revisions to the present document may incorporate support for those mechanisms in addition to SSL/TLS. Such revisions to the security mechanisms may also permit the use of an unreliable transport such as UDP.

### 4.1.2	Hypertext Transfer Protocol

The Hypertext Transfer Protocol (HTTP) is the standard protocol for web-based communications. HTTP has been adopted for a wide variety of purposes including proxy services, bi-directional content delivery, database access, network management, and metering information. HTTP is by far the most widely used application protocol on the Internet, and is supported by all significant firewalls and proxy servers.

## 4.2	Message Format

To illustrate the overall format of OSP messages, figure 2 shows an example message. As the figure indicates, the content within the HTTP message is formatted according to the standard for Multipurpose Internet Mail Extensions (MIME). The individual components of the message are a document conforming to the Extensible Markup Language (XML) specification and a Secure MIME (S/MIME) digital signature.

NOTE:	The digital signature is optional and, if omitted, the message content consists solely of a XML document.

| HTTP Header | POST scripts/settlements HTTP/1.0 content-type: multipart/signed; protocol="application/pkcs7-signature"; micalg=sha1; boundary=bar content-length: 844 |
| --- | --- |
| Message Content | --bar Content-Type: text/plain Content-Length: 524 <?xml version='1.0'?> <Message messageId="123454321" random="12345678"> <AuthorizationRequest componentId="9876567890"> <Timestamp> 1998-04-24T17:03:00Z </Timestamp> <CallId> 1234432198766789 </CallId> <SourceInfo type="e164"> 81458811202 </SourceInfo> <DestinationInfo type="e164"> 4766841360 </DestinationInfo> <Service/> <MaximumDestinations> 5 </MaximumDestinations> </AuthorizationRequest> </Message> |
| Digital Signature | --bar Content-Type: application/pkcs7-signature Content-Length: 191 GhyHhHUujhJhjH77n8HHGTrfvbnj756tbB9HG4VQpfyF467GhIG fHfYT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t bB9HGTrfvbnjn8HHGTrfvhJhjH776tbB9HG4VQbnj7567GhIGfH fYT6ghyHhHUujpfyF47GhIGfHfYT64VQbnj756 --bar-- |

Figure 2: Example Message Showing Overall Format

### 4.2.1	Multipurpose Internet Mail Extensions

All messages exchanged as part of this OSP specification conform to the Multipurpose Internet Mail Extensions (MIME) specification. The MIME specification defines mechanisms to combine individual components of arbitrary format (e.g. text, graphics, audio information, binary data, etc.) into a single message. Originally designed for electronic mail, the MIME specification has been adapted for a variety of communication applications, including web browsing. MIME format is widely supported by existing firewalls and proxy servers.

### 4.2.2	Extensible Markup Language

The first part of each MIME message is a document conforming to the Extensible Markup Language (XML) standard. As an extension of the widely deployed Hypertext Markup Language (HTML), XML can be readily parsed by firewalls and proxy servers. Unlike HTML, though, XML is readily extensible and can easily support rich, structured data such as pricing and usage information.

### 4.2.3	Secure MIME

The second part of each MIME message, if present, is a digital signature conforming to the Secure Multipurpose Internet Mail Extensions (S/MIME). S/MIME format includes support for multiple digest and signing algorithms and for variable cryptographic strength (e.g. key lengths). S/MIME format is also self-identifying with respect to these parameters, so that a recipient can derive the necessary information for verifying the signature from the signature data.

NOTE:	This does not imply that the recipient is guaranteed to be able to verify the signature, only that the recipient can tell what it needs to perform the verification. (So that, for example, the recipient may identify a signing algorithm that it does not support).

# 5	Protocol Profiles

This clause specifies the profiles for the protocols required by this OSP specification. It identifies the normative references to those protocols, as well as the specific versions, options, and extensions that the present document requires. The specific protocols described in this clause are the Secure Sockets Layer (SSL) and Transport Layer (TLS) protocols and the Hypertext Transfer Protocol (HTTP). The clause concludes by specifying the overall format of the messages conveyed through these protocols. The following clauses describe the message content in detail.

## 5.1	Secure Sockets Layer/Transport Layer Security

If secure authentication of the server is desired, or if confidentiality of the information exchanged between client and server is desired, the communication between the devices shall be secured using the Secure Sockets Layer (SSL) or Transport Layer Security (TLS) as described in this clause.

### 5.1.1	Protocol Version

Conforming systems shall support version 3.0 of the Secure Sockets Layer protocol [36] to secure their communications.

NOTE:	As an implementation option, systems may support version 1.0 of the Transport Layer Security protocol [37] or later versions.

### 5.1.2	Client/Server Roles

When initiating a communication as part of the present document, the initiating system shall act as an SSL/TLS client while the responding system shall act as an SSL/TLS server.

### 5.1.3	CipherSuites

Annex B documents the cryptographic algorithms required and recommended by the present document, including SSL/TLS ciphersuites.

## 5.2	Hypertext Transfer Protocol

### 5.2.1	Protocol Version

Conforming systems shall support version 1.0 of the Hypertext Transfer Protocol [2] as the base transfer protocol for their messages.

NOTE:	As an implementation option, systems may support HTTP version 1.1 [4].

### 5.2.2	Client/Server Roles

When initiating a communication as part of the present document, the initiating system shall act as an HTTP client, while the responding system shall act as an HTTP server.

### 5.2.3	TCP Port

Clients shall support sending their requests to TCP port 443 if SSL/TLS is being used, and to TCP port 80 otherwise. As an implementation option, communicating parties may agree to communicate via other TCP ports.

### 5.2.4	HTTP Methods

Requests from clients to a server shall be in the form of HTTP request messages using the POST method. Responses from a server shall consist of valid HTTP response messages.

### 5.2.5	Uniform Resource Identifier

The uniform resource identifier included in the POST request is not specified in the present document, but rather is subject to prior agreement between the communicating parties.

### 5.2.6	HTTP Headers

The HTTP header of the POST method shall minimally consist of the request-line. All request-header and general-header fields are optional. If present, they shall conform to the HTTP standard [38]. The status-line for the HTTP responses shall be present in those responses, and it shall conform to the HTTP standard, including status-code and reason-phrase values. All response-header and general-header fields are optional. If present, they shall conform to the HTTP standard.

### 5.2.7	HTTP Entity Body

Each message (i.e. HTTP entity body) conveyed as part of the present document shall conform to the Multipurpose Internet Mail Extensions standard [39], and shall, if signed, consist of exactly two parts, an Extensible Markup Language document and a Secure Multipurpose Internet Mail Extensions digital signature, as specified in the following two clauses. The highest level structure for each message shall conform to the multipart/signed syntax defined in S/MIME [40]. The message’s media type shall be "multipart/signed" with appropriate parameters (e.g. protocol of "application/pkcs7-signature" and micalg of "sha1.") The entity shall indicate the correct content-length value, as defined in the HTTP standard [41].

If not signed, each message shall simply consist of a single, text/plain part.

If bulk transfer is used, eachthe entire message shallould be signed.

# 6	XML Content

This clause specifies the actual message format used by the OSP to exchange pricing, authentication and authorization, and usage information. It outlines the overall XML document structure, lists the individual XML elements, and describes how those elements are combined into exchanges.

## 6.1	Document Structure

### 6.1.1	Multipurpose Internet Mail Extensions Conformance

As the first part of a Multipurpose Internet Mail Extensions (MIME) message, each message content shall conform to the MIME standard [42] as indicated below.

#### 6.1.1.1	Content-Type

The message’s content-type shall be designated text/plain.

NOTE:	It is anticipated that the Internet Engineering Task Force (IETF) will eventually define a MIME content-type for XML documents (e.g. text/xml). When such a definition is available, subsequent revisions of the present document may specify the use of that content-type instead of text/plain.

#### 6.1.1.2	Content-Length

All messages shall indicate the correct content-length value as defined in the MIME standard.

#### 6.1.1.3	Transfer Encoding

As XML documents can be carried within the HTTP protocol in their native format, no transfer encoding (e.g. quoted-printable or base-64) shall be used.

NOTE:	Since XML content is carried in its native encoding, that encoding will not conform to the recommendations for the text/plain content-type. In particular, the XML encoding specifies the use of a single line-feed character (#xA) to indicate line ending rather than the return/line-feed pair. Although not the default encoding for text/plain, such an encoding does not violate the MIME standard.

### 6.1.2	XML Conformance

The actual message content itself shall conform to the XML standard [43]. As part of that conformance, systems shall follow the well-formedness and character encoding requirements as follows.

#### 6.1.2.1	XML Version

Message content shall conform to version 1.0 of the XML standard, and shall indicate that version with the required XML prologue of <?xml version=‘1.0’?>.

#### 6.1.2.2	Well-Formed Constraint

All messages shall be well-formed XML documents, as defined in the [44] standard. Messages may be valid XML documents as well, by referencing the appropriate XML document type definitions (DTDs). Strict validity (as defined by [45]) is not required, however.

NOTE:	The terms "well-formed" and "valid" have specific meanings with the XML standard, and are used to indicate specific degrees of conformance to the standard.

#### 6.1.2.3	Character Encoding

Messages may use any character set permitted by the XML standard. As specified in that standard, however, all implementations shall be capable of generating and interpreting UTF-8 and UTF-16 encodings. In the absence of explicit knowledge that the receiving system can support other character encodings, sends shall use UTF-8 or UTF-16 encoding [46].

### 6.1.3	XML Framework

All messages shall conform to the overall framework illustrated in figure 3. As the figure shows, messages consist of a single root entity, which contains one or more components, each of which consists in turn of one or more elements. These elements may include XML attributes.

Figure 3: Overall XML Framework

#### 6.1.3.1	Root Entity

<!DOCTYPE Message [

<!ELEMENT Message ( ( PricingIndication | PricingConfirmation | AuthorizationRequest |

AuthorizationResponse | AuthorizationIndication | AuthorizationConfirmation | UsageIndication |

UsageConfirmation | ReauthorizationRequest | ReauthorizationResponse |
    SubscriberAuthenticationRequest | SubscriberAuthenticationResponse | CapabilitiesIndication |
    CapabilitiesConfirmation )+ )>

<!ATTLIST Message messageId ID    #REQUIRED

random    CDATA #REQUIRED>

...

]>

The root entity for each message shall be a Message element. It shall contain the components documented above, as well as a unique identifier attribute and a random attribute.

#### 6.1.3.2	Random Attribute

Each Message element shall include a random attribute. This attribute’s value shall be a random number, encoded as a decimal character string. The attribute ensures that each message contains some random content, and it therefore increases the security of the S/MIME digital signature.

NOTE:	Systems should use a cryptographically strong random number generator as the source for this attribute’s value. Standard library random number functions (such as from the ANSI C standard) are generally not cryptographically strong, and should not be used.

#### 6.1.3.3	Identifier Attribute

Each message and each component within a message shall include a unique identifier for that element. The system that initiates communication (through either a request or an indication) shall ensure that the identifier is unique. The system that replies (through either a response or a confirmation) shall use the identifier value to associate messages and components in its response or confirmation with the corresponding elements in the request or indication. The identifier attribute consists of an arbitrary character string, and is indicated by the attribute names messageId or componentId, as appropriate.

#### 6.1.3.4	Critical Attribute

All XML elements shall include a special attribute with the name critical that takes values of "true" or "false". This attribute indicates whether or not processing of the message can safely proceed if the particular element is not supported by the receiver.

If a system receives a component containing a critical element that it does not support, that system shall not accept or process the component.

If the critical attribute is omitted from an element (and its parents), that element shall be treated as if the critical attribute was present with the value "true".

The value of the critical attribute (whether explicit or implied) for an element shall apply to all sub-elements of the element unless a sub-element explicitly indicates otherwise.

#### 6.1.3.5	Extensions

Organizations may define additional elements or components beyond those documented in the present document. To ensure that no two organizations use the same named element, all private extensions shall have, as a prefix to their name, an officially registered Internet domain name belonging to the defining party. That domain name may be separated from the element name by a colon. If, for example, the organization that owns the Internet domain name acme.com wishes to add an element named PrivateOption, the tags delineating that element will be <acme.com:PrivateOption> and </acme.com:PrivateOption>. As noted above, the critical attribute may be used to indicate to a recipient whether or not it can safely ignore a private extension that it does not understand or support.

## 6.2	Components

Components are the main elements within each message. The <Message> element shall contain at least one and may contain more than one component. The components defined in this revision of the standard include pairs to effect pricing exchange (PricingIndication and PricingConfirmation), obtain authorization (AuthorizationRequest and AuthorizationResponse), verify authorization (AuthorizationIndication and AuthorizationConfirmation), refresh authorization (ReauthorizationRequest and ReauthorizationResponse), report usage (UsageIndication and UsageConfirmation), authentication subscribers (SubscriberAuthenticationRequest and SubscriberAuthenticationResponse) and exchange capabilities (CapabilitiesIndication and CapabilitiesConfirmation).

If not otherwise stated the components are sent on a per-call basis.

6.2.1	PricingIndication

<!ELEMENT PricingIndication ( Timestamp, SourceInfo, DestinationInfo, Currency, Amount, Increment,

Unit, Service, ValidAfter, ValidUntil )>

<!ATTLIST PricingIndication componentId ID #REQUIRED>

A PricingIndication component identifies the price for a particular service. It is composed of several elements, as indicated above.

For example, the following XML content states that the cost of basic telephone calls from the United States (country code 1) to France (country code 33) is US$ 0,50 per minute, effective immediately and indefinitely.

<PricingIndication componentId="1234567890">

<Timestamp>

1997-05-02T19:03:00Z

</Timestamp>

<SourceInfo type="e164prefix">

1

</SourceInfo>

<DestinationInfo type="e164prefix">

33

</DestinationInfo>

<Currency>

USD

</Currency>

<Amount>

0.5

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

<Service/>

<ValidAfter/>

<ValidUntil/>

</PricingIndication>

NOTE:	The PricingIndication component may be sent in bulk transfer. If bulk transfer is used, then the digital signature shallould be mandatory. The digital signature is described in clause 5.2.7.

### 6.2.2	PricingConfirmation

<!ELEMENT PricingConfirmation ( Timestamp, Status )>

<!ATTLIST PricingConfirmation componentId ID #REQUIRED>

A PricingConfirmation component indicates acceptance or rejection of the corresponding PricingIndication. The componentId attribute associates the confirmation with the indication. The only elements within the PricingConfirmation are the Timestamp and Status elements.

NOTE:	The PricingConfirmation component may be sent in bulk transfer. If bulk transfer is used, then the digital signature shallould be mandatory. The digital signature is described in clause 5.2.7.

### 6.2.3	AuthorizationRequest

<!ELEMENT AuthorizationRequest ( Timestamp, CallId+, SourceInfo, SourceAlternate*,

DestinationInfo, DestinationAlternate*, Service, MaximumDestinations, Token*,

SubscriberAuthenticationInfo*, Group?, MultiMediaCallId*, SessionId* )>

<!ATTLIST AuthorizationRequest componentId ID #REQUIRED>

An AuthorizationRequest asks for authorization to use resources. In the context of basic Internet telephony service, it asks for authorization to complete a phone call. The call, for example, may be identified by ITU-T Recommendation E.164 [12] numbers in the SourceInfo and DestinationInfo elements. The requesting system may leave the choice of peer endpoints up to the authorizing server, or it may specify the peer endpoint itself in a DestinationAlternate element.

The client shall provide one or more CallId elements in the request. A single CallId element implies that the client wishes to use that call identifier for all possible destinations returned in the AuthorizationResponse. Multiple CallId elements imply that the client wishes each potential destination to have its own call identifier. If the client wishes to specify multiple call identifiers, the number of CallId elements shall be equal to the value of the MaximumDestinations element.

The AuthorizationRequest message may request either authorization information, call routing information, or both. When requesting authorization information only, clients shall specify a MaximumDestinations value of zero (0). When requesting call routing information only, clients may omit any SourceAlternate elements that identify a subscriber, and they may include a Token received as the result of a prior authorization. In addition, clients may include previously received SubscriberAuthenticationInfo elements.

In order to authorize a group, OSP client A might indicate in the Group element the parameter on which authorization shall be based.

If a Group element is included in the AuthorizationRequest component, then the DestinationInfo element shall identify the range of valid destination domain addresses. The SourceInfo element should not be included. The SourceAlternate element should identify the source domain.

NOTE 1:	The use of the AuthorizationRequest message is not intended to be a replacement for, or supersede any ITU-T Recommendation H.323 [13] Registration Admission and Status (RAS) messages. Although the elements of the AuthorizationRequest are similar to information elements in RAS messages, AuthorizationRequest is intended for use in the bulk transfer of authorization tokens or other scenarios in which RAS signalling would not be appropriate.

NOTE 2:	The present document permits either party in an authorization exchange to determine the call routing information (e.g. identifying the peer endpoint for the call). If the client wishes to explicitly specify call routing, it does so by including one or more DestinationAlternate elements (e.g. of type "transport" containing the IP address and port number of the peer endpoint) in the AuthorizationRequest. If the server is performing call routing, it returns the information in the AuthorizationResponse. The DestinationAlternate elements are advisory during the request. The server should attempt to use one of the endpoints specified, but it is free to substitute a different set of endpoints if it is unable to settle the call using those specified.

If no routing is required, this shall be explicitly indicated by setting MaximumDestinations to "0".

### 6.2.4	AuthorizationResponse

<!ELEMENT AuthorizationResponse ( Timestamp, Status, TransactionId, Token*, Destination*, Service?, Group* )>

<!ATTLIST AuthorizationResponse componentId ID #REQUIRED>

An AuthorizationResponse component returns authorization information corresponding to an AuthorizationRequest. It includes a Timestamp, a Status element, a TransactionId, optional tokens, zero or more Destination elements, optional a QualityOfService element and optional a Group element. Any tokens present are assumed to apply to all destinations. The response includes a componentId attribute to associate it with the appropriate AuthorizationRequest.

If the requested QoS cannot be granted then the Service element should indicate the highest level of QoS available.

The use of the Group element is defined in clause 8.3.1.

NOTE:	If a client has expressed its own call routing preferences in the AuthorizationResponse (e.g. with DestinationAlternate elements of type transport), then the server should make every attempt to honour those preferences by returning appropriate Destination elements. The server may also include alternative destinations in its response.

### 6.2.5	AuthorizationIndication

<!ELEMENT AuthorizationIndication ( Timestamp, Role, CallId, SourceInfo, SourceAlternate*,

DestinationInfo, DestinationAlternate*, Service, Token* )>

<!ATTLIST AuthorizationIndication componentId ID #REQUIRED>

An AuthorizationIndication asks for verification of previously issued authorization, typically by asking for verification of an authorization token. Because tokens may be opaque to the terminating endpoint, that endpoint may not be able to determine the originator of a particular token. It is therefore acceptable to pass an entire sequence of tokens from a setup message in this component, and it is acceptable to send simultaneous AuthorizationIndication messages to multiple servers. The server shall be capable of recognizing authorization tokens in addition to validating them.

NOTE:	Even though the present document defines messages for validating authorization tokens, it does not require their use. In particular, some tokens may be constructed so that they can be completely verified in the peer system (e.g. through the use of digital signatures).

### 6.2.6	AuthorizationConfirmation

<!ELEMENT AuthorizationConfirmation ( Timestamp, Status, ValidAfter, ValidUntil )>

<!ATTLIST AuthorizationConfirmation componentId ID #REQUIRED>

An AuthorizationConfirmation component indicates whether or not an authorization is valid. It includes a Timestamp, a Status element and validation time limits. The confirmation also includes a componentId attribute to associate it with the appropriate AuthorizationIndication.

### 6.2.7	UsageIndication

<!ELEMENT UsageIndication ( Timestamp, Role, TransactionId, CallId, SourceInfo, SourceAlternate*,

DestinationInfo, DestinationAlternate*, UsageDetail*)>

<!ATTLIST UsageIndication componentId ID #REQUIRED>

The UsageIndication component reports resource usage. In the context of basic Internet telephony service, this is typically call duration.

NOTE 1:	The UsageIndication component may be sent in bulk transfer. If bulk transfer is used, then the digital signature shallould be mandatory. The digital signature is described in clause 5.2.7.

NOTE 2:	The CallId, the SourceInfo and the DestinationInfo element form an unique identifier for each call. This unique identifier might be used to compare the reported usage details of originating and terminating endpoint.

### 6.2.8	UsageConfirmation

<!ELEMENT UsageConfirmation ( Timestamp, Status )>

<!ATTLIST UsageConfirmation componentId ID #REQUIRED>

A UsageConfirmation component indicates acceptance or rejection of the corresponding UsageIndication. The componentId attribute associates the confirmation with the indication. The only elements within the UsageConfirmation are the Timestamp and Status elements.

NOTE:	The UsageConfirmation component may be sent in bulk transfer. If bulk transfer is used, then the digital signature shallould be mandatory. The digital signature is described in clause 5.2.7.

### 6.2.9	ReauthorizationRequest

<!ELEMENT ReauthorizationRequest ( Timestamp, Role, CallId, SourceInfo?, SourceAlternate*,

DestinationInfo?, DestinationAlternate*, TransactionId, UsageDetail*, Token* )>

<!ATTLIST ReauthorizationRequest componentId ID #REQUIRED>

A ReauthorizationRequest component requests a reauthorization of a previously authorized service. A client may use this message, for example, if a previous authorization has expired.

### 6.2.10	ReauthorizationResponse

<!ELEMENT ReauthorizationResponse ( Timestamp, Status, TransactionId, Token*, Destination* )>

<!ATTLIST ReauthorizationResponse componentId ID #REQUIRED>

A ReauthorizationResponse component indicates acceptance or rejection of the corresponding ReauthorizationRequest. Any tokens present are assumed to apply to all destinations. The componentId attribute associates the response with the request.

### 6.2.11	SubscriberAuthenticationRequest

<!ELEMENT SubscriberAuthenticationRequest (Timestamp, SourceInfo, SourceAlternate*,

DestinationInfo?, Service?)>

<!ATTLIST SubscriberAuthenticationRequest componentId ID #REQUIRED>

A SubscriberAuthenticationRequest asks for authentication of a subscriber’s credentials, typically in the form of a ISO/IEC 7812-1 [30] identification card number and associated personal identification number.

### 6.2.12	SubscriberAuthenticationResponse

<!ELEMENT SubscriberAuthenticationResponse (Timestamp, Status, SubscriberAuthenticationInfo*)>
<!ATTLIST SubscriberAuthenticationResponse componentId ID #REQUIRED>

A SubscriberAuthenticationResponse component returns an indication of whether the subscriber’s credentials are considered authentic.

### 6.2.13	CapabilitiesIndication

<!ELEMENT CapabilitiesIndication (DeviceInfo*, OSPVersion, OSPCapability*, Resources?)>

<!ATTLIST CapabilitiesIndication componentId ID                #REQUIRED
                                 critical    ( true | false )  #FIXED    "false">

To identify the OSP features it should use with a particular server, a client sends that server a CapabilitiesIndication message. That message indicates the highest version of OSP that the client is willing to support, as well as the specific capabilities it wishes to use. The server responds with a CapabilitiesConfirmation message. This message indicates the version of OSP that the two parties should use for their communication, and it provides the client details about how to use the services it requires.

If the server responds to a CapabilitiesIndication message with anything other than a CapabilitiesConfirmation message (e.g. an error status), the client should assume that the server is only capable of support for version 1.4.3 of the specification.

### 6.2.14	CapabilitiesConfirmation

<!ELEMENT CapabilitiesConfirmation (Timestamp, Status, OSPVersion, OSPService*, CertificateChain*,
    DeviceId?)>

<!ATTLIST CapabilitiesConfirmation componentId ID                #REQUIRED
                                   critical (true | false)       #FIXED    "false">

The CapabilitiesConfirmation message allows a server to specify the OSP capabilities that clients should use to communicate with it. It is sent in response to a CapabilitiesIndication. The information consists of a version number, an optional list of services supported, and an optional set of certificate chains. The version element specifies the version of OSP to which future communications shall conform. This version number shall be equal to or less than the version number proposed in the client’s CapabilitiesIndication. The services list provides the client with information it needs to access various OSP services. The CertificateChain elements provide certificate chains for public keys that the server will use to sign any authorization tokens it supplies.

## 6.3	Elements

This subclause defines the individual elements that make up each message component.

### 6.3.1	Amount

<!ELEMENT Amount (#PCDATA)>

<!ATTLIST Amount critical (true | false) "true">

The Amount element identifies a numeric value and is often associated with the Increment and Unit elements, as well as the Currency element. Amounts are expressed using the period (.) as a decimal separator and with no punctuation as the thousands separator. The following excerpt, for example, expresses a rate of 50 cents (US) per minute.

<Currency>

USD

</Currency>

<Amount>

0.5

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

### 6.3.2	AuthorityURL

<!ELEMENT AuthorityURL (#PCDATA)>

<!ATTLIST AuthorityURL critical (true | false) "true">

The AuthorityURL element identifies a uniform resource locator (URL) by which authorization may be verified or refreshed.

### 6.3.3	CallId

<!ELEMENT CallId (#PCDATA)>

<!ATTLIST CallId encoding (cdata | base64) "cdata"

critical (true | false)   "true">

The CallId element contains a call’s ITU-T Recommendation H.323 [13] CallId value, and is thus used to uniquely identify individual calls. To convey its binary value, a call identifier may either be encoded using XML CDATA format and appropriate escape sequences, or it may be encoded with base64 encoding as per the [47] standard. An encoding attribute indicates the method selected, and the default value for that attribute is XML CDATA.

### 6.3.4	Code

<!ELEMENT Code (#PCDATA)>

<!ATTLIST Code critical (true | false) "true">

The Code element contains the numeric value that uniquely and unambiguously indicates the sender’s response to a request or indication. It is usually paired with a Description element within a Status element. The Code content consists of three numeric digits in the form NNN. The first (most significant) digit indicates the success or failure of the operation, subsequent digits provide greater detail. Values defined by the present document include the following:

NOTE:	Subsequent revisions to the present document may add other code values, but will always preserve the meaning of a most significant 2 as success, and any other most significant digit as failure.

2xx  =  operation successful

200  =  success (no other information)

201  =  information created (no previous values)

210  =  updated information accepted (previous values replaced)

4xx  = request failure

400  =  bad request (generic problem interpreting message)

401  =  unauthorized (user authentication failed)

402  =  payment required

403  =  forbidden (route blocked)

404  =  not found (no route to destination)

405  =  source may not originate call to destination

410  =  character encoding not supported

411  =  parsing unsuccessful

412  =  critical element not supported

420  =  generic security problem (no other information available)

421  =  signature invalid

422  =  cryptographic algorithm not supported

423  =  certificate invalid

424  =  certificate revoked

425  =  encryption required

428  =  SourceInfo invalid or missing

441  =  temporary failure

447  =  resource unavailable (route to destination may not terminate)

449  =  quality of service unavailable

450  =  requested facility is not subscribed

463  =  service or option not available

481  =  call ID invalid or missing

488  =  incompatible destination

495  =  invalid message

5xx  =  server error

500  =  internal server error

501  =  not implemented

503  =  service not available

510  =  transient problem in server

520  =  long term problem in server

530  =  time problem

531  =  valid time too soon

532  =  time interval too small

999  =  generic failure (no other information available)

### 6.3.5	Currency

<!ELEMENT Currency (#PCDATA)>

<!ATTLIST Currency critical (true | false) "true">

The Currency element defines the financial currency in use for the parent element. It is represented according to the notation of ISO 4217 [7]. In addition, the following definitions not included in ISO 4217 [7] may be used:

ECU	European Currency Unit

EUR	Euro

SDR	Special Drawing Rights

### 6.3.6	Description

<!ELEMENT Description (#PCDATA)>

<!ATTLIST Description critical (true | false) "false">

The Description element provides a textual description for a response, and is typically paired with a Code element as part of a Status parent element. The Description element is for informational purposes only, as the Code element defines the behaviour. Suggested values for the Description element are the descriptions given above for each Code value.

### 6.3.7	Destination

<!ELEMENT Destination ( DestinationInfo?, DestinationAlternate*, DestinationSignalAddress, Token*,

ValidAfter?, ValidUntil?, UsageDetail*, AuthorityURL*, DestinationProtocol?, CallId, MultiMediaCallId+, SessionId+ )>

<!ATTLIST Destination critical (true | false) "true">

The Destination element is the parent element for call routing information, and it is returned by servers in an AuthorizationResponse messages. As the above definition shows, as many as ten different types of subelements may comprise a Destination element.

### 6.3.8	DestinationAlternate

<!ELEMENT DestinationAlternate (#PCDATA)>

<!ATTLIST DestinationAlternate type     ( e164 | h323 | url | email | transport |

international | national | network |

subscriber | abbreviated | e164prefix )    #REQUIRED

critical ( true | false )                             "true"       >

The DestinationAlternate element contains secondary identification of the destination. This information provides an alternative to the DestinationInfo element. DestinationAlternate uses the same notation as DestinationInfo.

### 6.3.9	DestinationInfo

<!ELEMENT DestinationInfo (#PCDATA)>

<!ATTLIST DestinationInfo type     ( e164 | h323 | url | email | transport |

international | national | network |

subscriber | abbreviated | e164prefix )         #REQUIRED

critical ( true | false )                                  "true"       >

The DestinationInfo element gives the primary identification of the destination, or called party, for a call. The element includes a type attribute, and can take one of several forms depending on the value of that attribute.

The following list indicates the contents of the element, given each possible attribute type.

e164		full ITU-T Recommendation E.164 [12] telephone number containing numeric digits 	only 	(i.e. no punctuation)

h323		ITU-T Recommendation H.323 [13] identifier

url		Uniform Resource Locator [48]

email		electronic mail address [49]

transport			transport address is the form of name:nn where name is the domain name (or IP address 				enclosed in square brackets) and :nn is an (optional) TCP or UDP port number,

(e.g. [172.16.1.1]:112)

international		international party number [50]

national		national party number [51]

network		network specific party number [52]

subscriber		subscriber party number [53]

abbreviated		abbreviated party number [54]

e164prefix	initial (most significant) digits of an ITU-T Recommendation E.164 [12] number with no punctuation

### 6.3.10	DestinationSignalAddress

<!ELEMENT DestinationSignalAddress (#PCDATA)>

<!ATTLIST DestinationSignalAddress critical (true | false) "true">

The DestinationSignalAddress element identifies the call signalling address for the destination. It is represented as name:nn, where name is a domain name or an IP address enclosed in square brackets. The :nn is optional and indicates a TCP port number.

For example, call signalling to device gateway.operator.com at TCP port number 112 is represented as follows:

<DestinationSignalAddress>

gateway.operator.com:112

</DestinationSignalAddress>

For example, call signalling to a VISIONng ITSP is represented as follows:

<DestinationSignalAddress>

visionng.com/contactid

</DestinationSignalAddress>

The VISIONng contactid contains 9 digits. The first 5 digits identify the ITSP, digits 6 to 9 are internal numbers and identify gatekeeper zones.

| Digit 1 to 5 | ITSP Name |
| --- | --- |
| 00001 | Tele Denmark |
| 00002 | Telefonica |
| 00003 | Belgacom |
| 00004 | Telekom Austria |
| 00005 | Matav |
| 00006 | Omnitel |
| 00007 | Pantel |

### 6.3.11	Increment

<!ELEMENT Increment (#PCDATA)>

<!ATTLIST Increment critical (true | false) "true">

The Increment element indicates the number of units being accounted. It is typically used in combination with the Amount and Unit elements. The following excerpt, for example, expresses a duration of 5½ minutes.

<Amount>

5.5

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

### 6.3.12	MaximumDestinations

<!ELEMENT MaximumDestinations (#PCDATA)>

<!ATTLIST MaximumDestinations critical (true | false) "true">

The MaximumDestinations element appears in the AuthorizationRequest component to indicate the maximum number of potential destinations the client wishes to receive in the response.

### 6.3.13	Role

<!ELEMENT Role (#PCDATA)>

<!ATTLIST Role critical (true | false) "true">

The Role element indicates the role of the system generating a message. It shall contain one of the following values:

source	message generated by source;

destination	message generated by destination;

other	message generated by systems other than source or destination.

### 6.3.14	Service

<!ELEMENT Service ( ServiceType?, Bandwidth?, QualityOfService? )>

<!ATTLIST Service critical (true | false) "true">

The Service element indicates a type of service being priced, authorized, or reported. If the ServiceElement is not present: voice. If the Bandwidth element is present, it indicates the bandwidth required for or consumed by the service. The QualityOfService element indicates 5 different speech QoS classes or QoS parameters according to ETSI TS 101 329-2 [55].

### 6.3.15	SourceAlternate

<!ELEMENT SourceAlternate (#PCDATA)>

<!ATTLIST SourceAlternate type     ( e164 | h323 | url | email | transport |

international | national | network |

subscriber | abbreviated | e164prefix | iso7812 |

pin | epin | deviceId ) #REQUIRED

critical ( true | false )                                    "true"     >

The SourceAlternate element contains secondary identification of the source of a call. It conforms to the same notation as the DestinationInfo element.

The additional types specific to source information are iso7812, pin, and epin. They are defined as follows:

iso7812	ISO/IEC standard 7812-1 [30] identification card number containing numeric digits only (i.e. no punctuation)

pin	Personal identification number containing numeric digits only (i.e. no punctuation)

epin	Encrypted personal identification number, base64 encoded. The encryption procedure is as follows:

1. 	Pad the PIN, represented as ASCII decimal digits, with nulls (binary zeroes) to a multiple of 16 octets.

2. 	Concatenate a 128-bit random number to the shared secret (shared by the OSP server and client).

3. 	Calculate a one-way hash of the resulting concatenation using the Message Digest 5 algorithm.

4. 	Exclusive-OR the result with the padded PIN.

5.	Concatenate the resulting value after the original 128-byte random and base64 encode the resulting 256-byte quantity.

deviceId	server-assigned identifier.

### 6.3.16	SourceInfo

<!ELEMENT SourceInfo (#PCDATA)>

<!ATTLIST SourceInfo type     ( e164 | h323 | url | email | transport |

international | national | network | subscriber |

abbreviated | e164prefix | iso7812 | pin | epin | deviceId )

#REQUIRED

critical ( true | false )                                       "true"       >

The SourceInfo element contains the primary identification of the source of a call. It uses the same notation as the DestinationInfo element, along with the additional types defined for SourceAlternate.

### 6.3.17	SourceSignalAddress

<!ELEMENT SourceSignalAddress (#PCDATA)>

<!ATTLIST SourceSignalAddress critical (true | false) "true">

The SourceSignalAddress element identifies the call signalling address of the source of a call. It uses the same notation as the DestinationSignalAddress element.

### 6.3.18	Status

<!ELEMENT Status ( Code, Description? )>

<!ATTLIST Status critical (true | false) "true">

The Status element reports the results of a response or confirmation. It is composed of a Code element and an optional Description element.

For example, the following excerpt indicates a successful response or confirmation:

<Status>

<Code>

200

</Code>

<Description>

success (no other information)

</Description>

</Status>

### 6.3.19	Timestamp

<!ELEMENT Timestamp (#PCDATA)>

<!ATTLIST Timestamp critical (true | false) "true">

The Timestamp element indicates the time at which the component was generated. It is represented by a restricted form of ISO 8601 [8] format. In particular, time is always represented in co-ordinated universal time (UTC) using the notation YYYY-MM-DDThh:mm:ssZ where:

YYYY =  four-digit year (for example, 1998);

MM   =  two-digit month (01=January, etc.);

DD   =  two-digit day of month (01 through 31);

T    =  indicates division between date and time;

hh   =  two-digit hour (00 through 23);

mm   =  two-digit minute (00 through 59);

ss   =  two-digit second (00 through 59);

Z    =  indicates co-ordinated universal time.

For example, exactly 3:03 P.M. on May 2, 1997, Eastern Daylight Time in the United States, is represented as:

<Timestamp>

1997-05-02T19:03:00Z

</Timestamp>

### 6.3.20	Token

<!ELEMENT Token (#PCDATA)>

<!ATTLIST Token encoding (cdata | base64) "cdata"

critical (true | false)   "true">

The Token element conveys a security token. To convey its binary value, a token may either be encoded using XML CDATA format and appropriate escape sequences, or it may be encoded with base64 encoding as per the [56] standard. An encoding attribute indicates the method selected, and the default value for that attribute is XML CDATA.

### 6.3.21	TransactionId

<!ELEMENT TransactionId (#PCDATA)>

<!ATTLIST TransactionId critical (true | false) "true">

The TransactionId element contains an integer, decimal valued identifier assigned to a specific authorized transaction. It is represented without any punctuation (e.g. no thousands separator).

### 6.3.22	Unit

<!ELEMENT Unit (#PCDATA)>

<!ATTLIST Unit critical (true | false) "true">

The Unit element indicates the units by which pricing is measured or usage recorded. It shall contain one of the following values:

s		seconds;

pkt	packets (datagrams);

byte	bytes,

page	fax pages

call	calls

6.3.23	UsageDetail

<!ELEMENT UsageDetail (Service, Amount, Increment, Unit, StartTime?, EndTime?, TerminationCause?,
    Statistics? )>

<!ATTLIST UsageDetail critical (true | false) "true">

The UsageDetail element collects information describing the usage of a service. Individual transactions may combine multiple UsageDetail elements as part of their usage report. This capability supports both parallel services (e.g. audio and video streams during a video conference) and serial services (e.g. low bit-rate codec switching to a higher quality codec as network conditions deteriorate).

The UsageDetail element may also be present in either an AuthorizationResponse or a ReauthorizationResponse. In that case, it indicates a limit to the authorization. For example, an AuthorizationResponse that includes a UsageDetail of (amount = 3, increment = 60, unit = s) indicates that the authorization is valid for no more than 3 minutes of service.

### 6.3.24	ValidAfter

<!ELEMENT ValidAfter (#PCDATA)>

<!ATTLIST ValidAfter critical (true | false) "true">

The ValidAfter element identifies the time and date after which the component’s information shall be effective or valid. It is encoded using the same notation as the Timestamp element. If this element is empty, component information is assumed to be valid as soon as it is received.

### 6.3.25	ValidUntil

<!ELEMENT ValidUntil (#PCDATA)>

<!ATTLIST ValidUntil critical (true | false) "true">

The ValidUntil element identifies the time and date after which the component’s information is no longer effective or valid. It is encoded using the same notation as the Timestamp element. If this element is empty, component information is assumed to be effective indefinitely, or until it is explicitly modified with new information.

### 6.3.26	EndTime

<!ELEMENT EndTime (#PCDATA)>

<!ATTLIST EndTime critical (true | false) #FIXED "false">

The EndTime element indicates the time at which the service ended. It is encoded using the same notation as the Timestamp element.

### 6.3.27	StartTime

<!ELEMENT StartTime (#PCDATA)>

<!ATTLIST StartTime critical (true | false) #FIXED "false">

The StartTime element indicates the time at which the service started. It is encoded using the same notation as the Timestamp element.

### 6.3.28	TCCode

<!ELEMENT TCCode (#PCDATA)>

<!ATTLIST TCCode critical (true | false) #FIXED "false">

The TCCode element contains the numeric value that uniquely and unambiguously indicates the internet telephony transaction’s status code. It is usually paired with a Description element within a TerminationCause element. The TCCode content consists of four numeric digits in the form NNNN. The first (most significant) digit indicates the success (1) or failure (0) of the operation, subsequent digits provide greater detail. Values defined by the present document include the following:

0001   =  unallocated (unassigned) number;

0002   =  no route to specified transit network;

0003   =  no route to destination;

0004   =  send special information tone;

0005   =  misdialed trunk prefix;

0006   =  channel unacceptable;

0007   =  call awarded and being delivered in an established channel;

0008   =  preemption;

0009   =  preemption – circuit reserved for re-use;

1016   =  normal call clearing;

0017   =  user busy;

0018   =  no user responding;

0019   =  no answer from user (user alerted);

0020   =  subscriber absent;

0021   =  call rejected;

0022   =  number changed;

0026   =  non-selected user clearing;

0027   =  destination out of order;

0028   =  invalid number format (address incomplete);

0029   =  facility rejected;

0030   =  response to STATUS ENQUIRY;

0031   =  normal, unspecified;

0032   =  no circuit/channel unavailable;

0038   =  network out of order;

0039   =  permanent frame mode connection out of service;

0040   =  permanent frame mode connection operational;

0041   =  temporary failure;

0042   =  switching equipment congestion;

0043   =  access information discarded;

0044   =  requested circuit/channel not available;

0046   =  precedence call blocked;

0047   =  resource unavailable, unspecified;

0049   =  quality of service unavailable;

0050   =  requested facility not subscribed;

0053   =  outgoing calls barred within CUG;

0055   =  incoming calls barred with CUG;

0057   =  bearer capability not authorized;

0058   =  bearer capability not presently available;

0062   =  inconsistency in designated outgoing access information and subscriber class;

0063   =  service or option not available, unspecified;

0065   =  bearer capability not implemented;

0066   =  channel type not implemented;

0069   =  requested facility not implemented;

0070   =  only restricted digital information bearer capability is available;

0079   =  service or option nit implemented, unspecified;

0081   =  invalid call reference value;

0082   =  identified channel does not exist;

0083   =  a suspended call exists, but this call identity does not;

0084   =  call identity in use;

0085   =  no call suspended;

0086   =  call having the requested call identity has been cleared;

0087   =  user not member of CUG;

0088   =  incompatible destination;

0090   =  non-existent CUG;

0091   =  invalid transit network selection;

0095   =  invalid message, unspecified;

0096   =  mandatory information element is missing;

0097   =  message type non-existent or not implemented;

0098   =  message not compatible with call state or message type non-existent or not implemented;

0099   =  information element/parameter non-existent or not implemented;

0100   =  invalid information element contents;

0101   =  message not compatible with call state;

0102   =  recovery on timer expiry;

0103   =  parameter non-existent or not implemented, passed on;

0110   =  message with unrecognized parameter, discarded;

0111   =  protocol error, unspecified;

0127   =  interworking, unspecified.

### 6.3.29	TerminationCause

<!ELEMENT TerminationCause (TCCode, Description?)>

<!ATTLIST TerminationCause critical (true | false) #FIXED "false">

The TerminationCause element reports the results of an internet telephony transaction. It is composed of the TCCode element and an optional Description element.

For example, the following excerpt indicates a successful termination cause:

<TerminationCause>

<TCCode>

1O16

</TCCode

<Description>

normal call clearing

</Description>

</TerminationCause>

### 6.3.30	Certificate

<!ELEMENT Certificate (#PCDATA)>

<!ATTLIST Certificate encoding (cdata | base64) "base64"

critical (true | false) #FIXED "false">

This element contains a public key certificate. To convey its binary value, a certificate may either be encoded using XML CDATA format and appropriate escape sequences, or it may be encoded with base64 encoding as per the [5] standard. An encoding attribute indicates the method selected, and the default value for that attribute is base64.

### 6.3.31	CertificateChain

<!ELEMENT CertificateChain (Certificate*)>

<!ATTLIST CertificateChain critical (true | false) #FIXED "false">

This element contains a certificate chain. The first certificate is the subject’s certificate. It is followed by any intermediate certificate authorities (in order) and concludes with the certificate for the root authority.

### 6.3.32	OSPCapability

<!ELEMENT OSPCapability (#PCDATA)>

<!ATTLIST OSPCapability critical (true | false) #FIXED "false">

This element identifies a particular OSP capability that a server can provide to a client. Capabilities are identified by the name of the component that the client sends to the server to invoke them. A client that wishes to send AuthorizationRequest messages to a server, for example, would include the following XML in its CapabilitiesIndication message.

<OSPCapability critical="false">
    AuthorizationRequest
<OSPCapability>

### 6.3.33	OSPService

<!ELEMENT OSPService (OSPCapability, OSPServiceURL*, OSPSignatureRequired)>

<!ATTLIST OSPService critical (true | false) #FIXED "false">

The OSPService element provides information a client needs to obtain a particular service from an OSP server. The specific service is identified by the OSPCapability element. OSPServiceURL elements, if present, indicate the URL(s) the client should use to request the service. They appear in order of decreasing priority (e.g. the primary URL appears first), and, if none are present in the OSPService element, then the client may rely on other means to obtain the information. (It may, for example, simply use the same URL it used for the CapabilitiesIndication. Message.) The final element, OSPSignatureRequired, is a Boolean value that indicates whether or not the server requires that request for the specified service to the identified URLs to be digitally signed. This explicitly allows for a server to indicate one set of URLs for signed messages of a particular type and a different set for unsigned messages of that same type. When such an option is available, the decision as to whether or not to sign messages is an implementation choice for the client. One possible strategy would be to first attempt to use the unsigned option, falling back to the signed option if the server responds with a security error.

### 6.3.34	OSPServiceURL

<!ELEMENT OSPServiceURL (#PCDATA)>

<!ATTLIST OSPServiceURL critical (true | false) #FIXED "false">

The OSPServiceURL element identifies the uniform resource locator (URL) a client should use for a particular OSP service.

### 6.3.35	OSPSignatureRequired

<!ELEMENT OSPSignatureRequired (#PCDATA)>

<!ATTLIST OSPSignatureRequired critical (true | false) #FIXED "false">

The OSPSignatureRequired element indicates whether or not a server requires that requests for the indicated service be digitally signed. It shall contain one of the following values:

true		signatures are required;

false	signatures are not required.

### 6.3.36	OSPVersion

<!ELEMENT OSPVersion (#PCDATA)>

<!ATTLIST OSPVersion critical (true | false) #FIXED "false">

The OSPVersion element identifies the highest version of TS 101 321 that the sender is willing to support. The sender is assumed to support all publicly released versions of the specification up to and including the indicated value. For example, the following XML fragment indicates that the sender can support OSP versions 1.4.2, 1.4.3, and 2.1.1.

<OSPVersion critical = "false">
    2.1.1
</OSPVersion>

### 6.3.37	SubscriberAuthenticationInfo

<!ELEMENT SubscriberAuthenticationInfo (#PCDATA)>

<!ATTLIST SubscriberAuthenticationInfo encoding (cdata | base64) "cdata"

critical (true | false)   #FIXED "false">

Opaque information that a server creates to confirm the authentication of a subscriber. The server returns this element as part of the SubscriberAuthenticationResponse message, and clients should include it in subsequent AuthorizationRequest messages for the subscriber.

### 6.3.38	DeviceInfo

<!ELEMENT DeviceInfo (#PCDATA) >

<!ATTLIST DeviceInfo type ( e164 | h323 | url | email |

transport | serialnumber | customerId ) #REQUIRED

critical    ( true | false )                   #FIXED    "false" >

The DeviceInfo element allows a device to identify itself to a server when sending a CapabilitiesIndication message. In addition to the standard SourceInfo types, clients may use the following types to identify themselves:

serialnumber	manufacturer’s serial number or equivalent for the device

customerId 		user-assigned value for the device.

### 6.3.39	DeviceId

<!ELEMENT DeviceId (#PCDATA) >

<!ATTLIST DeviceId critical (true | false) "false" >

The DeviceId element allows a server to provide an identifier to a client in a CapabilitiesConfirmation message. The client may then use that identifier in subsequent messages to the server.

### 6.3.40	Resources

<!ELEMENT Resources ( DataRate?, AlmostOutOfResources? ) >

<!ATTLIST Resources critical    ( true | false )  #FIXED "false" >

The Resources element allows a client to indicate its resources and their current status, as part of a CapabilitiesIndication message.

### 6.3.41	DataRate

<!ELEMENT DataRate ( NumberOfChannels?, Bandwidth ) >

<!ATTLIST DataRate critical    ( true | false )  #FIXED "false" >

DataRate describes the load that the client can handle.

### 6.3.42	NumberOfChannels

<!ELEMENT NumberOfChannels (#PCDATA) >

<!ATTLIST NumberOfChannels critical (true | false) #FIXED "false" >

The NumberOfChannels element is positive integer, indicating the number of channels available on the client to service calls for a particular protocol.

### 6.3.43	Bandwidth

<!ELEMENT Bandwidth (#PCDATA) >

<!ATTLIST Bandwidth critical (true | false) #FIXED "false" >

The Bandwidth element is a positive integer.  When the NumberOfChannels element is present, it indicates the available bandwidth (in bits/sec) of each channel on the client to service calls for a particular protocol. When the NumberOfChannels element is not present, Bandwidth represents the total bandwidth (in bits/sec).

### 6.3.44	AlmostOutOfResources

<!ELEMENT AlmostOutOfResources (#PCDATA) >

<!ATTLIST AlmostOutOfResources critical (true | false) #FIXED "false" >

The AlmostOutOfResources element is a flag from the client to the server.  It shall contain one of the following values:

true		if the client might run out of resources shortly to accept new calls

false	if the client has sufficient resources to accept new calls.

6.3.45	DestinationProtocol

<!ELEMENT DestinationProtocol (#PCDATA) >

<!ATTLIST DestinationProtocol critical ( true | false ) #FIXED "false">

The DestinationProtocol element indicates the session, or call, control protocol of the destination, or called party, for a call. It contains one of the following values :

h323		destination supports ITU-T Recommendation H.323

sip		destination supports IETF RFC3261 Session Initiation Protocol

Editors Note: 21TD088

### 6.3.46	QualityOfService

<!ELEMENT QualityOfService (QoSClass? | QoSParameters? ) >

<!ATTLIST QualityOfService critical (true | false) "true" >

The QoSClass element indicates either one of 256 speech QoS classes or the QoS parameters.

### 6.3.47	QoSClass

<!ELEMENT QoSClass (QoSClass? | QoSParameters? ) >

<!ATTLIST QoSClass critical (true | false) "true" >

The QoSClass element indicates one of 256 speech QoS classes or the TC classes.

| Name | Meaning |
| --- | --- |
| T0 | Predefined |
| T1 | Best (ETSI TS 101 329-2 [57]) |
| T2 | High (ETSI TS 101 329-2 [58]) |
| T3 | Medium (ETSI TS 101 329-2 [59]) |
| T4 | Acceptable (ETSI TS 101 329-2 [60]) |
| T5 | Best effort (ETSI TS 101 329-2 [61]) |
| T6 | for future TIPHON standardization |
| To |  |
| T16 |  |
| T17 | User defined |
| To |  |
| T255 |  |
| TCx | To be defined by ITU-T SG13 |
| To |  |
| TCxx |  |

Editors Note:	QoS signalling is not yet finalized. Review ITU, IPCableCom, 3GPP

### 6.3.48	QoSParameters

<!ELEMENT QoSParameters (Delay, Jitter, PackLoss)>

<!ATTLIST QoSParameters critical (true | false) "true">

The QoSParameters element contains the Delay, Jitter and PackLoss element.

### 6.3.49	Group

<!ELEMENT Group ( GroupID?, (Amount, Unit)?, (ValidAfter, ValidUntil)?, ValidUntil? )>

<!ATTLIST Group critical (true | false) "true">

The Group element is used in order to authorize a group of calls between 2 domains.

The Group element may contain a GroupId element:

The Group element shall contain at least one of the following parameter combinations:

- Amount and Unit element to indicate a maximum number of calls or

- ValidAfter and ValidUntil element or

- ValidUntil element to indicate a maximum time period for calls.

### 6.3.50	GroupId

<!ELEMENT GroupId ( #PCDATA )>

<!ATTLIST GroupId critical (true | false) "true">

The GroupId element shall contain an integer, decimal valued identifier assigned to a specific group. It is represented without any punctuation (e.g. no thousands separator).

### 6.3.51	SessionId

<!ELEMENT SessionId (#PCDATA)>

<!ATTLIST SessionId encoding (cdata | base64) "cdata"

critical (true | false)   "true">

The SessionId element shall contain a session’s SessionId value (SIP [62], SDP [63]).

### 6.3.52	MultiMediaCallId

<!ELEMENT MultiMediaCallId (MMCId, CallId+, SessionId+)>

<!ATTLIST MultiMediaCallId encoding (cdata | base64) "cdata"

critical (true | false)   "true">

The MultiMediaCallId element shall contain the multi media call’s identifier in the MMCId element. The MultiMediaCallId element shall contain at least one CallId element and one SessionId element.

### 6.3.53	MMCId

<!ELEMENT MMCId (#PCDATA)>

<!ATTLIST MMCId encoding (cdata | base64) "cdata"

critical (true | false)   "true">

The MMCId element shall contain in the #PCDATA field the multi media call’s identifier. The MMCId element shall contain an integer, decimal valued identifier assigned to a specific multi media call. It is represented without any punctuation (e.g. no thousands separator).

### 6.3.54	ServiceType

<!ELEMENT ServiceType (#PCDATA) >

<!ATTLIST ServiceType critical (true | false) #FIXED "false" >

The ServiceType element indicates the requested type of service (e.g. video streaming).

# 7	Signature Format

If present, the digital signature within OSP shall conform to the application/pkcs7-signature format specified in the Secure Multipurpose Internet Mail Extensions (S/MIME) standard [64]. This clause specifies how that signature is created, including the canonicalization procedure, signature algorithm, and transfer encoding.

## 7.1	Canonical Form

Digital signatures described in the present document are signatures of the entire, transmitted XML document, beginning with (and including) the leftmost "<" of the XML declaration and ending with (and including) the rightmost ">" of the end tag of the root XML entity. Furthermore, conforming implementations should construct their XML documents using the following procedures to arrive at a canonical form. Such a form will enable the reconstruction of an XML document (and the verification of a digital signature) from the abstract information of the document.

NOTE 1:	The following procedure borrows heavily from the Internet Open Trading Protocol (IOTP) specification [65].

1)	Isolate the element to be signed. For the present document, this consists of the entire XML document, beginning with the leftmost "<" of the XML declaration and ending with (and including) the rightmost ">" of the end tag of the root XML entity;

2)	convert all characters in the element to canonical form for the character encoding;

3)	apply all external XML entities and all character and entity references in the element so that they are completely resolved;

4)	exclude comments and processing instructions;

5)	reduce all attributes to their canonical form using the attribute type in the Document Type Definition (DTD). Replace all single and double quotes present in attributes with &#39; and &#34; respectively so that attributes can be enclosed in double quotes;

6)	create attributes, using their default value, which are not present in the original but have default values in the DTD;

7)	sort the original and generated attributes in ascending attribute name order according to character encoding of the attribute name;

8)	for whitespace inside markup but not inside attribute values, generate it as minimally as possible. Specifically, (1) remove non-essential whitespace, and (2) represent required whitespace by a single space character;

9)	generate the content of all start tags using only the element name and the attributes as described above. If the element is an empty element, then generate it using the single empty tag format (i.e. a trailing slash). Generate end tags using only element name with no added whitespace;

10)	remove all whitespace in the element content;

11)	assemble start tags, end tags, empty tags, CDATA sections, and text sections in the same order as the original document.

NOTE 2:	The above procedure results in a canonicalized XML document rather than a canonicalized MIME text part. In particular, the line ending is encoded as a single line-feed character (#xA) rather than the S/MIME convention of a return/line-feed pair.

## 7.2	Signature Algorithms

Annex B provides a list of cryptographic algorithms required and recommended by the present document, including S/MIME signature algorithms.

## 7.3	Transfer Encoding

Unlike some electronic mail protocols, HTTP is capable of transferring raw binary data. Consequently, no transfer encoding (e.g. quoted-printable or base-64) shall be used for the signature.

# 8	Protocol Behaviour

This clause specifies the relationship between the OSP message components. It defines message sequencing, interdependence of messages, and exception handling.

## 8.1	Message Sequencing

The present document specifies a simple client/server protocol. All protocol exchanges shall be initiated by clients, who send one or more of the eight client components in a single message. The server shall reply with its own single message, which contains one server component for each client component. Figure 4 shows the seven different component pairs.

When multiple components are combined in a single message, each component shall be treated independently of all others in the message. Logically, this treatment shall have the same effect as if each component is carried in its own message.

In addition, each component exchange shall be considered independent of all others; there shall be no dependence between message exchanges. This is true even of Authorization exchanges. Specifically, both AuthorizationIndication/Confirmation and ReauthorizationRequest/Response exchanges may refer to authorization previously obtained through means other than an AuthorizationRequest and AuthorizationResponse.

## 8.2	Exception Handling

If the OSP server cannot accept the authorization request because

- OSP client A or OSP Proxy cannot be authenticated, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “401 unauthorized (user authentication failed)”.

- no routes can be returned, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “404 not found (no route to destination)”.

- the source may not originate the call to the destination, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “405 source may not originate call to destination”.

- the SourceInfo is invalid or missing, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “428 SourceInfo invalid or missing”.

- the route is known but not available, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “447 resource unavailable (route to destination may not terminate)”.

- the requested QoS cannot be granted, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “449 quality of service unavailable” and containing a <<Service/QualityOfService>> element indicating the highest level of QoS available.

- the routes suggested by OSP client A or OSP Proxy are unknown, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “450 requested facility is not subscribed”.

- the requested service cannot be granted, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “463 service or option not available”.

- requested group call cannot be granted, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “463 service or option not available” and containing a <<Group>> element indicating the quantity or duration available on a group call basis.

- source and destination are incompatible, then the OSP server shall send an <AuthorizationResponse> component containing a <<Code>> element indicating “488 incompatible destinations”.

If the OSP server cannot accept a request because

- of malformed syntax, then the OSP server shall send a response containing a <<Code>> element indicating “400 bad request”.

- OSP client A or OSP Proxy is required to pay, then the OSP server shall send a response containing a <<Code>> element indicating “402 payment required”.

- the route is blocked for OSP client A or OSP Proxy, then the OSP server shall send a response containing a <<Code>> element indicating “403 route blocked”.

- of decoding errors, then the OSP server shall send a response containing a <<Code>> element indicating “410 character encoding not supported”.

- of parsing errors, then the OSP server shall send a response containing a <<Code>> element indicating “411 parsing unsuccessful”.

- of an unsupported critical element, then the OSP server shall send a response containing a <<Code>> element indicating “412 critical element not supported”.

- of a security problem, then the OSP server shall send a response containing a <<Code>> element indicating “420 generic security problem (no other information available)”.

- of an invalid signature, then the OSP server shall send a response containing a <<Code>> element indicating “421 signature invalid”.

- of an unsupported crypthographic algorithm, then the OSP server shall send a response containing a <<Code>> element indicating “422 cryptographic algorithm not supported”.

- of an invalid certificate, then the OSP server shall send a response containing a <<Code>> element indicating “423 certificate invalid”.

- of an revoked certificate, then the OSP server shall send a response containing a <<Code>> element indicating “424 certificate revoked”.

- encryption is required, then the OSP server shall send a response containing a <<Code>> element indicating “425 encryption required”.

- of a temporary failure, then the OSP server shall send a response containing a <<Code>> element indicating “441 temporary failure”.

- TransactionId or CallId does not match any existing TranscationId or CallId, then the OSP server shall send a response containing a <<Code>> element indicating “481 callId invalid or missing”.

- the value of the <<Timestamp>> element is wrong, then the OSP server shall send a response containing a <<Code>> element indicating “495 invalid message”.

- it’s not valid in a certain state, then the OSP server shall send a response containing a <<Code>> element indicating “495 invalid message”.

### 8.2.1	Transmission Control Protocol

If TCP indicates that communication cannot be established, or that communication has failed during a transmission, the communicating parties should not assume the delivery of any partial information.

### 8.2.2	Secure Socket Layer/Transport Layer Security

If SSL or TLS indicates that compatible encryption parameters cannot be established, the communicating parties should not assume the delivery of any partial information. In addition, if SSL/TLS is unable to successfully authenticate the server, the client should not proceed with the transfer of any information.

### 8.2.3	Hypertext Transfer Protocol

Communicating parties should treat HTTP status codes as defined in RFC 1945 [2]. In particular, unless the status code is in the range 200-299, the client should not assume delivery of any information.

Figure 4: Component Pairs

### 8.2.4	Status Element

XML Code elements within responses and confirmations should be treated appropriately according to the definitions of subclause 6.3.3. The associated Description element should be used solely for informational purposes.

## 8.3	Transaction types

Transactions may be completed by sending messages containing components on a

- per-call basis:
the message exchange is related to a single call with a unique call identifier or a single session with a session identifier.

- group basis:
in a group transaction an Authorization token will be given for a specified number of calls between 2 domains or for a specified duration between 2 domains. Each call shall contain a common group identifier and a unique call identifier.

- bulk transfer basis:
bulk transfer: multiple components related to a number of calls are sent within a single message.

- multi media call basis:
the multi media call is an aggregation of calls and sessions. It should contain at least 1 call or 1 session.

Possible implementations are described in XXX (location to be determined (e.g. 2007))

### Group behaviour

As shown in figure 4a, the OSP client requests authorization of a group. No GroupId is used because the OSP server manages the GroupId.

Alternative 1: The requested group is granted. The OSP client can now place calls by using the same group identifier.

Alternative 2: The requested group cannot be granted (group failure). The OSP server indicates the available quantity or duration in a new Group element. No group identifier is returned because the new group does not yet exist. The new group will be created, if the OSP clients request authorization for the new group.

Alternative 3: The requested group cannot be granted (group failure). The OSP server indicates the available quantity or duration in a new Group element. A group identifier is returned because this group does already exist (invitation to join a group). The OSP client will join this group, if it requests authorization for this group. Only in this case the AuthorizationReuqest will contain a Group element with GroupId.

NOTE 1:	Rather than granting 1000 calls for one group, the OSP server could grant 500 calls for group1 and 500 calls for group2.

NOTE 2:	The group feature implies that the terminating gatekeeper is capable of counting the number of inbound calls.

NOTE 3:	For group calls the token is calculated using the GroupId, not the CallId or SessionId.

Figure 4a: Group request 1

As shown in figure 4b, the OSP client requests authorization of a single call.

Alternative 1: The requested call is granted.

Alternative 2: The requested call is granted. If the OSP server wishes to invite a single call to a group, then a Group element should be included. No group identifier is returned because the new group does not yet exist. The new group will be created, if the OSP clients request authorization for the new group.

Alternative 3: The requested call is granted. If the OSP server wishes to invite a single call to a group, then a Group element should be included. A group identifier is returned because this group does already exist (invitation to join a group). The OSP client will join this group, if it requests authorization for this group.

Figure 4b: Call request 1

### 8.3.2	Group identifiers

The following picture shows a possible implementation of group identifiers. A multi media call (MultiMediaCallId 1) could consist of multiple H.323 calls (CallId 1,2,3) and multiple SIP sessions (SessionId 1,2). The border proxy could map this multi media call to different groups. Within group 2 (GroupId 2) a H.323 call would be identified by its multi media call identifier (MMCId 1) and its H.323 call identifier (CallId 1). Even different multi media calls could be mapped to group 2.

NOTE 1:	The terminology has to be aligned with 3016-R5 draft.

Figure 4c: Group identifiers

Annex A (normative):
Document Type Definition

This annex contains the complete XML document type definition for the messages described in the present document. The information is repeated from clause 6 and annex C, but is collected in one place for convenience of reference.

<!ELEMENT Message ( ( PricingIndication | PricingConfirmation | AuthorizationRequest | AuthorizationResponse | AuthorizationIndication | AuthorizationConfirmation | UsageIndication | UsageConfirmation | ReauthorizationRequest | ReauthorizationResponse | SubscriberAuthenticationRequest | SubscriberAuthenticationResponse | CapabilitiesIndication | CapabilitiesConfirmation )+ )>

<!ATTLIST Message messageId ID #REQUIRED

random CDATA #REQUIRED>

<!ELEMENT PricingIndication ( Timestamp, SourceInfo, DestinationInfo, Currency, Amount, Increment, Unit, Service, ValidAfter, ValidUntil )>

<!ATTLIST PricingIndication componentId ID #REQUIRED>

<!ELEMENT PricingConfirmation ( Timestamp, Status )>

<!ATTLIST PricingConfirmation componentId ID #REQUIRED>

<!ELEMENT AuthorizationRequest ( Timestamp, CallId+, SourceInfo, SourceAlternate*, DestinationInfo, DestinationAlternate*, Service, MaximumDestinations, Token*, SubscriberAuthenticationInfo*, Group?, MultiMediaCallId*, SessionId* )>

<!ATTLIST AuthorizationRequest componentId ID #REQUIRED>

<!ELEMENT AuthorizationResponse ( Timestamp, Status, TransactionId, Token*, Destination*, Service?, Group? )>

<!ATTLIST AuthorizationResponse componentId ID #REQUIRED>

<!ELEMENT AuthorizationIndication ( Timestamp, Role, CallId, SourceInfo, SourceAlternate*, DestinationInfo, DestinationAlternate*, Service, Token* )>

<!ATTLIST AuthorizationIndication componentId ID #REQUIRED>

<!ELEMENT AuthorizationConfirmation ( Timestamp, Status, ValidAfter, ValidUntil )>

<!ATTLIST AuthorizationConfirmation componentId ID #REQUIRED>

<!ELEMENT UsageIndication ( Timestamp, Role, TransactionId, CallId, SourceInfo, SourceAlternate*, DestinationInfo, DestinationAlternate*, UsageDetail*)>

<!ATTLIST UsageIndication componentId ID #REQUIRED>

<!ELEMENT UsageConfirmation ( Timestamp, Status )>

<!ATTLIST UsageConfirmation componentId ID #REQUIRED>

<!ELEMENT ReauthorizationRequest ( Timestamp, Role, CallId, SourceInfo?, SourceAlternate*, DestinationInfo?, DestinationAlternate*, TransactionId, UsageDetail*, Token* )>

<!ATTLIST ReauthorizationRequest componentId ID #REQUIRED>

<!ELEMENT ReauthorizationResponse ( Timestamp, Status, TransactionId, Token*, Destination* )>

<!ATTLIST ReauthorizationResponse componentId ID #REQUIRED>

<!ELEMENT SubscriberAuthenticationRequest (Timestamp, SourceInfo, SourceAlternate*, DestinationInfo?, Service?)>

<!ATTLIST SubscriberAuthenticationRequest componentId ID #REQUIRED>

<!ELEMENT SubscriberAuthenticationResponse (Timestamp, Status, SubscriberAuthenticationInfo*)>

<!ATTLIST SubscriberAuthenticationResponse componentId ID #REQUIRED>

<!ELEMENT CapabilitiesIndication (DeviceInfo*, OSPVersion, OSPCapability*, Resources*)>

<!ATTLIST CapabilitiesIndication componentId ID #REQUIRED

critical ( true | false ) #FIXED "false">

<!ELEMENT CapabilitiesConfirmation (Timestamp, Status, OSPVersion, OSPService*, CertificateChain*, DeviceId?)>

<!ATTLIST CapabilitiesConfirmation componentId ID #REQUIRED

critical (true | false) #FIXED "false">

<!ELEMENT Amount (#PCDATA)>

<!ATTLIST Amount critical (true | false) "true">

<!ELEMENT AuthorityURL (#PCDATA)>

<!ATTLIST AuthorityURL critical (true | false) "true">

<!ELEMENT CallId (#PCDATA)>

<!ATTLIST CallId encoding (cdata | base64) "cdata"

critical (true | false) "true">

<!ELEMENT Code (#PCDATA)>

<!ATTLIST Code critical (true | false) "true">

<!ELEMENT Currency (#PCDATA)>

<!ATTLIST Currency critical (true | false) "true">

<!ELEMENT Description (#PCDATA)>

<!ATTLIST Description critical (true | false) "false">

<!ELEMENT Destination ( DestinationInfo?, DestinationAlternate*, DestinationSignalAddress, Token*, ValidAfter?, ValidUntil?, UsageDetail*, AuthorityURL*, DestinationProtocol?,CallId, MultiMediaCallId+, SessionId+ )>

<!ATTLIST Destination critical (true | false) "true">

<!ELEMENT DestinationAlternate (#PCDATA)>

<!ATTLIST DestinationAlternate type ( e164 | h323 | url | email | transport | international | national | network | subscriber | abbreviated | e164prefix ) #REQUIRED

critical ( true | false ) "true" >

<!ELEMENT DestinationInfo (#PCDATA)>

<!ATTLIST DestinationInfo type ( e164 | h323 | url | email | transport | international | national | network | subscriber | abbreviated | e164prefix ) #REQUIRED

critical ( true | false ) "true" >

<!ELEMENT DestinationSignalAddress (#PCDATA)>

<!ATTLIST DestinationSignalAddress critical (true | false) "true">

<!ELEMENT Increment (#PCDATA)>

<!ATTLIST Increment critical (true | false) "true">

<!ELEMENT MaximumDestinations (#PCDATA)>

<!ATTLIST MaximumDestinations critical (true | false) "true">

<!ELEMENT Role (#PCDATA)>

<!ATTLIST Role critical (true | false) "true">

<!ELEMENT Service ( ServiceType?, Bandwidth?, QualityOfService? )>

<!ATTLIST Service critical (true | false) "true">

<!ELEMENT SourceAlternate (#PCDATA)>

<!ATTLIST SourceAlternate type ( e164 | h323 | url | email | transport | international | national | network | subscriber | abbreviated | e164prefix | iso7812 | pin | epin | deviceId ) #REQUIRED

critical ( true | false ) "true" >

<!ELEMENT SourceInfo (#PCDATA)>

<!ATTLIST SourceInfo type ( e164 | h323 | url | email | transport | international | national | network | subscriber | abbreviated | e164prefix | iso7812 | pin | epin | deviceId ) #REQUIRED

critical ( true | false ) "true" >

<!ELEMENT SourceSignalAddress (#PCDATA)>

<!ATTLIST SourceSignalAddress critical (true | false) "true">

<!ELEMENT Status ( Code, Description? )>

<!ATTLIST Status critical (true | false) "true">

<!ELEMENT Timestamp (#PCDATA)>

<!ATTLIST Timestamp critical (true | false) "true">

<!ELEMENT Token (#PCDATA)>

<!ATTLIST Token encoding (cdata | base64) "cdata"

critical (true | false) "true">

<!ELEMENT TransactionId (#PCDATA)>

<!ATTLIST TransactionId critical (true | false) "true">

<!ELEMENT Unit (#PCDATA)>

<!ATTLIST Unit critical (true | false) "true">

<!ELEMENT UsageDetail (Service, Amount, Increment, Unit, StartTime?, EndTime?, TerminationCause?, Statistics? )>

<!ATTLIST UsageDetail critical (true | false) "true">

<!ELEMENT ValidAfter (#PCDATA)>

<!ATTLIST ValidAfter critical (true | false) "true">

<!ELEMENT ValidUntil (#PCDATA)>

<!ATTLIST ValidUntil critical (true | false) "true">

<!ELEMENT EndTime (#PCDATA)>

<!ATTLIST EndTime critical (true | false) #FIXED "false">

<!ELEMENT StartTime (#PCDATA)>

<!ATTLIST StartTime critical (true | false) #FIXED "false">

<!ELEMENT TCCode (#PCDATA)>

<!ATTLIST TCCode critical (true | false) #FIXED "false">

<!ELEMENT TerminationCause (TCCode, Description?)>

<!ATTLIST TerminationCause critical (true | false) #FIXED "false">

<!ELEMENT Certificate (#PCDATA)>

<!ATTLIST Certificate encoding (cdata | base64) "base64"

critical (true | false) #FIXED "false">

<!ELEMENT CertificateChain (Certificate*)>

<!ATTLIST CertificateChain critical (true | false) #FIXED "false">

<!ELEMENT OSPCapability (#PCDATA)>

<!ATTLIST OSPCapability critical (true | false) #FIXED "false">

<!ELEMENT OSPService (OSPCapability, OSPServiceURL*, OSPSignatureRequired)>

<!ATTLIST OSPService critical (true | false) #FIXED "false">

<!ELEMENT OSPServiceURL (#PCDATA)>

<!ATTLIST OSPServiceURL critical (true | false) #FIXED "false">

<!ELEMENT OSPSignatureRequired (#PCDATA)>

<!ATTLIST OSPSignatureRequired critical (true | false) #FIXED "false">

<!ELEMENT OSPVersion (#PCDATA)>

<!ATTLIST OSPVersion critical (true | false) #FIXED "false">

<!ELEMENT SubscriberAuthenticationInfo (#PCDATA)>

<!ATTLIST SubscriberAuthenticationInfo encoding (cdata | base64) "cdata"

critical (true | false) #FIXED "false">

<!ELEMENT DeviceInfo (#PCDATA) >

<!ATTLIST DeviceInfo type ( e164 | h323 | url | email | transport | serialnumber | customerId ) #REQUIRED

critical ( true | false ) #FIXED "false" >

<!ELEMENT DeviceId (#PCDATA) >

<!ATTLIST DeviceId critical (true | false) "false" >

<!ELEMENT Resources ( DataRate?, AlmostOutOfResources? ) >

<!ATTLIST Resources critical ( true | false ) #FIXED "false" >

<!ELEMENT DataRate ( NumberOfChannels?, Bandwidth ) >

<!ATTLIST DataRate critical ( true | false ) #FIXED "false" >

<!ELEMENT NumberOfChannels (#PCDATA) >

<!ATTLIST NumberOfChannels critical (true | false) #FIXED "false" >

<!ELEMENT Bandwidth (#PCDATA) >

<!ATTLIST Bandwidth critical (true | false) #FIXED "false" >

<!ELEMENT AlmostOutOfResources (#PCDATA) >

<!ATTLIST AlmostOutOfResources critical (true | false) #FIXED "false" >

<!ELEMENT Statistics ( LossSent?, LossReceived?, OneWayDelay?, RoundTripDelay?, Delay?, Jitter?, PackLoss? )>

<!ATTLIST Statistics critical (true | false) #FIXED "false">

<!ELEMENT LossSent ( Packets, Fraction )>

<!ATTLIST LossSent critical (true | false) #FIXED "false">

<!ELEMENT Packets (#PCDATA)>

<!ATTLIST Packets critical (true | false) #FIXED "false">

<!ELEMENT Fraction (#PCDATA)>

<!ATTLIST Fraction critical (true | false) #FIXED "false">

<!ELEMENT LossReceived ( Packets, Fraction )>

<!ATTLIST LossReceived critical (true | false) #FIXED "false">

<!ELEMENT OneWayDelay ( Minimum, Mean, Variance, Samples )>

<!ATTLIST OneWayDelay critical (true | false) #FIXED "false">

<!ELEMENT Minimum (#PCDATA)>

<!ATTLIST Minimum critical (true | false) #FIXED "false">

<!ELEMENT Maximum (#PCDATA)>

<!ATTLIST Maximum critical (true | false) #FIXED "false">

<!ELEMENT Mean (#PCDATA)>

<!ATTLIST Mean critical (true | false) #FIXED "false">

<!ELEMENT Variance (#PCDATA)>

<!ATTLIST Variance critical (true | false) #FIXED "false">

<!ELEMENT Samples (#PCDATA)>

<!ATTLIST Samples critical (true | false) #FIXED "false">

<!ELEMENT RoundTripDelay ( Minimum, Mean, Variance, Samples )>

<!ATTLIST RoundTripDelay critical (true | false) #FIXED "false">

<!ELEMENT DestinationProtocol (#PCDATA) >

<!ATTLIST DestinationProtocol critical ( true | false ) #FIXED "false">

<!ELEMENT ServiceType (#PCDATA)>

<!ATTLIST ServiceType critical (true | false) "true">

<!ELEMENT QualityOfService (QoSClass? | QoSParameters?)>

<!ATTLIST QualityOfService critical (true | false) "true">

<!ELEMENT QoSClass (#PCDATA)>

<!ATTLIST QoSClass critical (true | false) "true">

<!ELEMENT QoSParameters (Delay, Jitter, PackLoss)>

<!ATTLIST QoSParameters critical (true | false) "true">

<!ELEMENT Delay (Minimum?, Mean?, Maximum?, Variance?, Samples?)>

<!ATTLIST Delay critical (true | false) "true">

<!ELEMENT Jitter (Minimum?, Mean?, Maximum?, Variance?, Samples?)>

<!ATTLIST Jitter critical (true | false) "true">

<!ELEMENT PackLoss (Minimum?, Mean?, Maximum?, Variance?, Samples?)>

<!ATTLIST PackLoss critical (true | false) "true">

<!ELEMENT Group (GroupId?, (Amount, Unit)?, (ValidAfter, ValidUntil)?, ValidUntil? )>

<!ATTLIST Group critical (true | false) "true">

<!ELEMENT GroupId (#PCDATA)>

<!ATTLIST GroupId critical (true | false) "true">

<!ELEMENT SessionId (#PCDATA)>

<!ATTLIST SessionId critical (true | false) "true">

<!ELEMENT MultiMediaCallId (MMCId, CallId+, SessionId+ )>

<!ATTLIST MultiMediaCallId critical (true | false) "true">

<!ELEMENT MMCId (MMCId, CallId+, SessionId+ )>

<!ATTLIST MMCId critical (true | false) "true">

Annex B (normative):
Cryptographic Algorithms

For convenience and ease of reference, this annex provides the specification of cryptographic algorithms referenced in the standard. Cryptographic algorithms are essential to SSL/TLS communications, S/MIME signatures, and token formats.

# B.1	SSL/TLS CipherSuites

Systems conforming to the present document may negotiate any mutually supported SSL/TLS CipherSuite using the standard SSL/TLS handshake mechanisms. To ensure maximum interoperability, all compliant systems should support the SSL_RSA_WITH_3DES_EDE_CBC_SHA CipherSuite. In environments where triple DES data encryption is not desired or is otherwise unavailable, systems should support the SSL_RSA_EXPORT_WITH_DES40_CBC_SHA CipherSuite. If no data encryption is desired or available, systems should use the SSL_RSA _WITH_NULL_SHA CipherSuite.

# B.2	S/MIME Signatures

Communicating systems may use any digest and signing algorithms defined by the S/MIME standard [66]. To ensure interoperability, all compliant systems shall support the Secure Hash Algorithm (SHA) [67] for message digests, and the Digital Signature Algorithm (DSA) [68] for signing.

NOTE:	To take maximum advantage of deployed cryptographic software, systems should also support the Message Digest 5 (MD5) digest algorithm [69] and the Rivest Shamir Adleman (RSA) signature algorithm [70].

# B.3	Tokens

Tokens used for authorization, call progress, and call completion may be signed and encrypted. For message digest, signing, and encryption, implementations may use any algorithm defined by the Public Key Cryptography Standard #7 (PKCS7) [71]. To ensure interoperability, systems should support Secure Hash Algorithm [72], Diffie-Hellman key exchange [73], Digital Signature Algorithm [74], and the Data Encryption Standard [75].

NOTE:	To take maximum advantage of deployed cryptographic software, systems should also support the Message Digest 5 [76], Rivest Shamir Aldeman [77], and Rivest Cipher 2 [78] algorithms.

Annex C (normative):
Enhanced Usage Reports

The following optional Enhanced Usage elements may be added to the UsageDetail element of UsageIndication messages.

# C.1	Enhanced Usage Elements

The definition of Enhanced Usage elements, and the definition of sub-elements, follows:

## C.1.1	Statistics

<!ELEMENT Statistics ( LossSent?, LossReceived?, OneWayDelay?, RoundTripDelay?, Delay?, Jitter?, PackLoss? )>
<!ATTLIST Statistics critical (true | false) #FIXED "false">

The Statistics element collects network performance statistics for the call. It may include packet loss statistics (in either direction) and delay statistics (one-way or round trip.) The entire element is non-critical, and may thus be safely ignored by systems that do not support it.

## C.1.2	LossSent

<!ELEMENT LossSent ( Packets, Fraction )>
<!ATTLIST LossSent critical (true | false) #FIXED "false">

The LossSent element contains packet loss information for datagrams transmitted by the reporting system that were not received by its peer, as reported in the peer's RTCP sender and receiver reports. It includes the two sub-elements indicated above and described in the following subsections.

## C.1.3	Packets

<!ELEMENT Packets (#PCDATA)>
<!ATTLIST Packets critical (true | false) #FIXED "false">

The Packets element contains a count of the total number of packets. The value is formatted as a decimal number without punctuation.

## C.1.4	Fraction

<!ELEMENT Fraction (#PCDATA)>
<!ATTLIST Fraction critical (true | false) #FIXED "false">

The Fraction element contains a value for a fraction of packets, expressed as an integer number from 0 (no packets) to 255 (all packets), and it is formatted as a decimal number without punctuation.

## C.1.5	LossReceived

<!ELEMENT LossReceived ( Packets, Fraction )>
<!ATTLIST LossReceived critical (true | false) #FIXED "false">

The LossReceived element contains packet loss information for datagrams that should have been received by the reporting system receive but were not, as reported in the system's RTCP sender and receiver reports. It includes the two sub-elements indicated above and described in the previous subsections.

## C.1.6	OneWayDelay

<!ELEMENT OneWayDelay ( Minimum, Mean, Variance, Samples )>
<!ATTLIST OneWayDelay critical (true | false) #FIXED "false">

The OneWayDelay element reports measurements of one way delay to the reporting system from its peer, as measured during the communication. It is suggested that the measurement be made by comparing the network time protocol (NTP) timestamp included in RTCP messages sent by the peer with the local NTP time. The element consists of the following four sub-elements.

## C.1.7	Minimum

<!ELEMENT Minimum (#PCDATA)>
<!ATTLIST Minimum critical (true | false) #FIXED "false">

The Minimum element reports the minimum measured value, expressed in milliseconds. It is formatted as an integer decimal number without punctuation.

## C.1.8	Mean

<!ELEMENT Mean (#PCDATA)>
<!ATTLIST Mean critical (true | false) #FIXED "false">

The Mean element reports the statistical mean of all measured values. It is expressed in milliseconds, and it is formatted as an integer decimal number without punctuation.

## C.1.9	Variance

<!ELEMENT Variance (#PCDATA)>
<!ATTLIST Variance critical (true | false) #FIXED "false">

The Variance element reports the statistical variance of all measured values. It is expressed in squared milliseconds, and it is formatted as an integer decimal number without punctuation.

## C.1.10	Samples

<!ELEMENT Samples (#PCDATA)>
<!ATTLIST Samples critical (true | false) #FIXED "false">

The Samples element reports the number of samples measured by the reporting system. It is formatted as a decimal number without punctuation.

## C.1.11	RoundTripDelay

<!ELEMENT RoundTripDelay ( Minimum, Mean, Variance, Samples )>
<!ATTLIST RoundTripDelay critical (true | false) #FIXED "false">

The RoundTripDelay element reports measurements of round trip delay between the reporting system and its peer, as measured during the communication. Such measurements may be made, for example, by
ITU-T Recommendation H.245 [10] round trip delay exchanges during the call. The element consists of the four
sub-elements described above.

## C.1.12	Maximum

<!ELEMENT Maximum (#PCDATA)>
<!ATTLIST Maximum critical (true | false) #FIXED "false">

The Maximum element reports the maximum measured value, expressed in milliseconds. It is formatted as an integer decimal number without punctuation.

## C.1.13	Delay

<!ELEMENT Delay ( Minimum?, Mean?, Maximum?, Variance?, Samples? ) >

<!ATTLIST Delay critical ( true | false ) "true">

The Delay element requests or reports the delay of a packet.

## C.1.14	Jitter

<!ELEMENT Jitter ( Minimum?, Mean?, Maximum?, Variance?, Samples? ) >

<!ATTLIST Jitter critical ( true | false ) "true">

The Jitter element requests or reports the jitter of a packet.

## C.1.15	PackLoss

<!ELEMENT PackLoss ( Minimum?, Mean?, Maximum?, Variance?, Samples? ) >

<!ATTLIST PackLoss critical ( true | false ) "true">

The PackLoss element requests or reports the packet loss.

Annex D (informative):
Token Formats

This annex presents suggested formats for authorization tokens included in the authorization information defined in the present document. Once obtained by an endpoint, such tokens may be passed as part of the call signalling exchange. The annex describes suggested cryptographic encodings as well as suggested contents for these authorization tokens, and it identifies methods for carrying tokens within call signalling protocols. The final section shows a complete, sample token.

# D.1	Cryptographic Encoding

Depending on the business roles and network environment, tokens may be digitally signed and/or encrypted. If digitally signed, tokens should conform to Cryptographic Message Syntax (CMS) specification [28] for signed-data content. If encrypted, tokens should conform to the CMS specification for enveloped-data content. If tokens are both signed and encrypted, they should be constructed in two phases. First, the token should be digitally signed, resulting in CMS signed-data content. The resulting signed content should then be encrypted, resulting in CMS enveloped-data content.

If the token syntax and semantics are understood by all relevant parties, then authorization tokens may use the generic id-data OBJECT IDENTIFER (1.2.480.113549.1.7.1) as the data content type. If a token contents are consistent with one of the formats described in this annex, and if the token wishes to explicitly identify that format, it may use one of the following object identifiers as the data content type:

osp-token-contents-asn1 OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token-contents(2) 1 }

osp-token-contents-xml OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token-contents(2) 2 }

osp-token-contents-bxml OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token-contents(2) 3 }

Annex B documents the cryptographic algorithms recommended by the present document, including digest, signing, symmetric encryption, and asymmetric encryption algorithms.

To minimize the size of resulting tokens, appropriate certificates may be conveyed to endpoints as part of the CapabilitiesConfirmaion message, and the optional CMS CertificateSet information element may be omitted from individual tokens. In such cases, the most efficient way to identify the signer is to use the SubjectKeyIdentifier, where its contents are a SHA-1 hash of the signer’s public key. An example of such a token is:

| SignedData ::= SEQUENCE { |  |
| --- | --- |
| version CMSVersion, | version 3 |
| digestAlgorithms DigestAlgorithmIdentifiers, | e.g. md5 (1.2.840.113594.2.5) |
| encapContentInformation EncapsulatedContentInfo, | e.g. osp-token-contents-bxml |
| signerInfos SignerInfos } | see below |

Where, the SignerInfos consists of the following single element:

| SignerInfo ::= SEQUENCE { |  |
| --- | --- |
| version CMSVersion, | version 3 |
| sid SignerIdentifier, | see below |
| digestAlgorithm DigestAlgorithmIdentifier, | e.g. md5 (1.2.840.113594.2.5) |
| signatureAlgorithm SignatureAlgorithmIdentifier, | e.g. rsa (1.2.840.113594.1.1.1) |
| signature SignatureValue } | 128-octet signature |

And the SignerIdentifier element is:

| SignerIdentifier ::= CHOICE { |  |
| --- | --- |
| SubjectKeyIdentifier [0] } | 20-octet SHA-1 hash of signer’s public key |

This encoding will result in a token whose size is about 250 octets plus the size of the signed contents.

NOTE:	The SubjectKeyIdentifier element may be used even when the signer’s public key certificate does not explicitly include that specific extended attribute, as the recipient will be able to compute the appropriate value from the certificate’s public key.

# D.2	Token Content

This clause suggests two different token formats. The ASN.1 format relies on information elements defined in the ITU-T Recommendation H.323 [13] standards, and is appropriate for native H.323 [13] systems. The XML format is consistent with the body of the present document, and may be used where ASN.1 encoding/decoding is not available or desired. The Binary XML format is a more compact representation of the XML format.

## D.2.1	ASN.1 Format

The actual content of the token may conform to the following ASN.1 specifications, as augmented by ASN.1 constructs from the ITU-T Recommendation H.323 [13] standards [79], [80], [81], and [82]. The tokens should be encoded using the Packed Encoding Rules (Aligned) of ITU-T Recommendation X.691 [11].

ETSI-TIPHON-TOKENS DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

AliasAddress,

CallIdentifier,

ReleaseCompleteReason,

TimeStamp

FROM H323-MESSAGES;

ServiceDescription :: = CHOICE

{

basicTelephony      NULL,

vendorSpecific      NonStandardParameter,

...

}

MeasureUnits :: CHOICE

{

seconds             NULL,

packets             NULL,

bytes               NULL,

vendorSpecific      NonStandardParameter,

...

}

UsageData :: = SEQUENCE

{

amount              INTEGER (0..4294967295)),

units               MeasureUnits,

increment           INTEGER (1..4294967295),

vendorSpecific      NonStandardParameter,

...

}

AuthorizationToken :: = SEQUENCE

{
	    random              INTEGER (1..4294967295),

sourceInfo          SEQUENCE OF AliasAddress,

destinationInfo     SEQUENCE OF AliasAddress,

callId              CallIdentifier OPTIONAL,

validAfter          TimeStamp OPTIONAL,

validUntil          TimeStamp OPTIONAL,

transactionId       OCTET STRING (SIZE(16)) OPTIONAL,

serviceAuthorized   SEQUENCE OF SEQUENCE

{

service         ServiceDescription,

limit           UsageData OPTIONAL

}                   OPTIONAL,
        authorityURL        SEQUENCE OF IA5String (SIZE(1..512)) OPTIONAL,

vendorSpecific      NonStandardParameter OPTIONAL,

...

}

END		-- of ASN.1

## D.2.2	XML Format

Authorization tokens may also be constructed in XML as a standalone XML document. The root element of such a document shall be TokenInfo, and it shall conform to the following construction:

<!ELEMENT TokenInfo ( SourceInfo?, SourceAlternate*, DestinationInfo?, DestinationAlternate*,

CallId*, ValidAfter?, ValidUntil?, TransactionId?, UsageDetail*,

AuthorityURL*, Group?, MultiMediaCallId*, SessionId* )>

<!ATTLIST TokenInfo random #REQUIRED>

The individual elements of the document, including any private or future extensions, shall conform to the body of the present document. The following text illustrates an example XML token:

<TokenInfo random="12345678">

<SourceInfo type="e164">81458811202</SourceInfo>

<DestinationInfo type="e164">4766841360</DestinationInfo>

<CallId encoding=base64>Aadf981lasdfA8adooif7c3nnsa89</CallId>

<ValidAfter>1998-04-24T17:01:01Z</ValidAfter>

<ValidUntil>1998-04-24T17:11:01Z</ValidUntil>

<TransactionId>6772374</TransactionId>

<UsageDetail>

<Service/>

<Amount>24</Amount>

<Increment>3600</Increment>

<Unit>s</Unit>

</UsageDetail>

</TokenInfo>

## D.2.3	Binary XML Format

Binary XML (see annex H) provides a more compact representation of XML documents, and may be applied to authorization token contents. The contents of the token of D.2.2, when represented as Binary XML according to the rules of annex H, are as follows:

02 01 03 00

F6 0B 03 '12345678' 00

EE 0F 01 03 '81458811202' 00 01

D6 0F 01 03 '4766841360' 00 01

CD 08 01 03

'Aadf981lasdfA8adooif7c3nnsa89' 00 01

7D 03 '1998-04-24T17:01:01Z' 00 01

7E 03 '1998-04-24T17:11:01Z' 00 01

78 03 '6772374' 00 01

7B

2C

46 03 '24' 00 01

5C 03 '3600' 00 01

79 03 's' 00 01

01

01

# D.3	Token Carriage

When carried as part of a call signalling message of an ASN.1-based protocol, authorization tokens may be identified by one of the following object identifiers. The first value is for tokens with self-identifying or otherwise unspecified formats.

osp-token OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token (1) 0 }

osp-token-asn1-format OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token (1) 1 }

osp-token-xml-format OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token(1) 2 }

osp-token-bxml-format OBJECT IDENTIFER ::= { itu-t(0) identified-organization(4) etsi(0) ts-101-321(1321) token(1) 3 }

When carries as part of a call signalling message of a MIME-based protocol, authorization tokens may be identified by the following MIME type:

MIME media type name: x-application

MIME subtype name: x-osp-token

Required parameters: none

Optional parameters:

x-osp-token-format: a value of "asn.1" indicates the token contents use the ASN.1 format defined in annex D, section D.2.1 of the OSP specification; a value of "xml" indicates the token contents use the XML format defined in annex D, section D.2.2 of the OSP specification, and a value of "bxml" indicates the token contents use the Binary XML format defined in annex D, section D.2.3 of the OSP specification. In the absence of any value for this parameter, the token contents shall be self identifying or otherwise understood by appropriate parties.

x-osp-token-version: a character string indicating the earliest revision of the OSP specification to which the token contents conform. In the absence of any value for this parameter, the token contents shall conform to version "2.1.1" of the OSP specification.

Encoding considerations: OSP tokens are normally carried as binary data by the call signalling protocol. Call signalling protocols which cannot reliably transfer binary data may use alternate encodings such as base-64, in which case standard MIME content-encoding parameters may indicate the particular encoding.

Security considerations: OSP tokens are intended to provide access control to resources of other administrative domains, and, as such, are inherently designed to address security concerns. For that reason, OSP tokens are digitally signed and, optionally, encrypted, as defined in the OSP specification.

Interoperability considerations: The means and/or algorithms by which a receiving system determines whether or not an OSP token is valid are a local matter. However, at a minimum, receiving systems should verify the digital signature of the token, and they should ensure that any call details included in the token contents (e.g. called number, calling number, etc.) are appropriate for the contemplated call.

Published specification: "Telecommunications and Internet Protocol Harmonization Over Networks (TIPHON); Open Settlement Protocol (OSP) for Inter-domain pricing, authorization, and usage exchange". Technical Specification 101 321. European Telecommunications Standards Institute. Version 2.1.1

Applications which use this media type: IP telephony call signalling protocols that use MIME types to convey additional information during call setup.

Additional information:

Magic number(s): none

File extension(s): none

Macintosh File Type Code(s): none

Person & email address to contact for further information: Stephen Thomas, stephen.thomas@transnexus.com

Intended usage: COMMON

Author/Change controller: European Telecommunications Standards Institute

NOTE:	A corresponding MIME type of osp-token may be registered with IANA in the future.

Editor’s Note: Was this done ???

# D.4	Sample Token

30 82 01 A1	SEQUENCE (len = 417)

06 09 ...		OBJECT IDENTIFIER = 1.2.840.113594.1.7.2 (id-signedData)

A0 82 01 92		EXPLICIT TAG [0] (len = 402)

30 82 01 8E			SEQUENCE (len = 398)

02 01 03				INTEGER = 3 (version)

31 0E					SET (len = 14)

30 0C						SEQUENCE (len = 12)

06 08 ...						OBJECT IDENTIFER = 1.2.840.113594.2.5 (md5)

05 00							NULL

30 82 00 B3				SEQUENCE (len = 179)

06 06 ...					OBJECT IDENTIFIER = 0.4.0.1321.2.3 (osp-token-contents-bxml)

A0 82 00 AB					EXPLICIT TAG [0] (len = 171)

04 82 00 A7						OCTET STRING (len = 167)

02 01 03 00 ...						token contents

31 82 00 C0				SET (len = 192)

30 82 00 BC					SEQUENCE (len = 188)

02 01 03						INTEGER = 3 (version)

A0 16							EXPLICIT TAG [0]

04 14								OCTET STRING (len = 20)

...										SHA-1 hash of signer’s public key information

30 0C							SEQUENCE (len = 12)

06 08 ...							OBJECT IDENTIFIER = 1.2.840.113594.2.5 (md5)

05 00								NULL

30 0D							SEQUENCE (len = 13)

06 09 ...							OBJECT IDENTIFIER = 1.2.840.113594.1.1.1 (rsa)

05 00								NULL

04 82 00 80						OCTET STRING (len = 128)

...									signature

Annex E (informative):
Example Messages

This annex provides complete example messages for several information exchanges. The messages included are full and complete, but subject to the following modifications for readability:

the content-type field of the HTTP header is shown as several lines. In actual implementations, that field will be constrained to a single line in conformance with HTTP specifications;

extra whitespace is included within the example XML documents. In actual implementations, this whitespace shall be removed before the digital signature is generated (according to the constraints of clause 7), and is not likely to be included in the actual transferred data;

the digital signatures within the example messages do not represent the actual digital signature for the example content. Actual signatures would likely result in unprintable characters.

# E.1	Pricing Exchange

The following two messages convey pricing information between two parties. The first message provides three separate price indications. Taken together, the three indications represent prices for basic Internet telephony service (thus the empty <Service> element) of ½ DM/minute to Berlin (country code 49, national destination code 30) 1 DM/minute elsewhere in Germany (country code 49), and 2 DM/minute elsewhere in the world. In all three cases no constraints are placed on the calling party (thus the empty <SourceInfo> elements).

POST scripts/settlements HTTP/1.0

content-type: multipart/signed;

protocol="application/pkcs7-signature";

micalg=sha1;

boundary=bar

content-length: 1968

--bar

Content-Type: text/plain

Content-Length: 1647

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<PricingIndication componentId="b">

<Timestamp>

1998-04-20T19:03:00Z

</Timestamp>

<SourceInfo type="e164prefix"/>

<DestinationInfo type="e164prefix"/>

<Currency>

DEM

</Currency>

<Amount>

2

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

<Service/>

<ValidAfter/>

<ValidUntil/>

</PricingIndication>

<PricingIndication componentId="c">

<Timestamp>

1998-04-20T19:03:02Z

</Timestamp>

<SourceInfo type="e164prefix"/>

<DestinationInfo type="e164prefix">

49

</DestinationInfo>

<Currency>

DEM

</Currency>

<Amount>

1

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

<Service/>

<ValidAfter/>

<ValidUntil/>

</PricingIndication>

<PricingIndication componentId="d">

<Timestamp>

1998-04-20T19:03:05Z

</Timestamp>

<SourceInfo type="e164prefix"/>

<DestinationInfo type="e164prefix">

4930

</DestinationInfo>

<Currency>

DEM

</Currency>

<Amount>

0.5

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

<Service/>

<ValidAfter/>

<ValidUntil/>

</PricingIndication>

</Message>

--bar

Content-Type: application/pkcs7-signature

Content-Length: 191

GhyHhHUujhJhjH77n8HHGTrfvbnj756tbB9HG4VQpfyF467GhIGfHfYT64VQpfyF467GhIGfHfYT6jH77n8HH

GghyHhHUujhh756tbB9HGTrfvbnjn8HHGTrfvhJhjH776tbB9HG4VQbnj7567GhIGfHfYT6ghyHhHUujpfyF4

7GhIGfHfYT64VQbnj756

--bar--

The following example shows a confirmation in reply to the above message. In the confirmation, all pricing information is accepted.

HTTP/1.0 200 OK

content-type: multipart/signed;

protocol="application/pkcs7-signature";

micalg=sha1;

boundary=bar

content-length: 1401

--bar

Content-Type: text/plain

Content-Length: 1080

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<PricingConfirmation componentId="b">

<Timestamp>

1998-04-20T19:04:00Z

</Timestamp>

<Status>

<Code>

201

</Code>

<Description>

new pricing created

</Description>

</Status>

</PricingConfirmation>

<PricingConfirmation componentId="c">

<Timestamp>

1998-04-20T19:04:22Z

</Timestamp>

<Status>

<Code>

210

</Code>

<Description>

revised pricing accepted

</Description>

</Status>

</PricingConfirmation>

<PricingConfirmation componentId="d">

<Timestamp>

1998-04-20T19:04:45Z

</Timestamp>

<Status>

<Code>

210

</Code>

<Description>

revised pricing accepted

</Description>

</Status>

</PricingConfirmation>

</Message>

--bar

Content-Type: application/pkcs7-signature

Content-Length: 191

GhyHhHUujhJhjH77n8HHGTrfvbnj756tbB9HG4VQpfyF467GhIGfHfYT64VQpfyF467GhIGfHfYT6j

H77n8HHGghyHhHUujhJh756tbB9HGTrfvbnjn8HHGTrfvhJhjH776tbB9HG4VQbnj7567GhIGfHfYT

6ghyHhHUujpfyF47GhIGfHfYT64VQbnj756

--bar—

# E.2	Authorization Exchange

The authorization exchange shows one party requesting and receiving authorization to access another party’s devices. In particular, the requestor wishes to complete a phone call in which the calling party is at +81 45 881 1202 and the called party is at +47 66 84 13 60.

POST scripts/settlements HTTP/1.0

content-type: text/plain

Content-Length: 600

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<AuthorizationRequest componentId="b">

<Timestamp>

1998-04-24T17:03:00Z

</Timestamp>

<CallId encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

</CallId>

<SourceInfo type="e164">

81458811202

</SourceInfo>

<DestinationInfo type="e164">

4766841360

</DestinationInfo>

<Service/>

<MaximumDestinations>

5

</MaximumDestinations>

</AuthorizationRequest>

</Message>

The reply to this request returns two separate authorizations, each representing a different peer system that is capable of completing the call. For each destination, the response authorizes up to 24 hours of service for the call.

HTTP/1.0 200 OK

content-type: text/plain

Content-Length: 1799

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<AuthorizationResponse componentId="b">

<Timestamp>

1998-04-24T17:03:01Z

</Timestamp>

<Status>

<Code>

200

</Code>

<Description>

success

</Description>

</Status>

<TransactionId>

67890987

</TransactionId>

<Destination>

<DestinationSignalAddress>

[172.16.1.2]:112

</DestinationSignalAddress>

<Token encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

HGTrfvbnjn8HHGTrfvhJhjH776tbB9HG4VQbnj7567GhIGfH

6ghyHhHUujpfyF47GhIGfHfYT64VQbnj

</Token>

<ValidAfter>

1998-04-24T17:01:01Z

</ValidAfter>

<ValidUntil>

1998-04-24T17:11:01Z

</ValidUntil>

<UsageDetail>

<Service/>

<Amount>

24

</Amount>

<Increment>

3600

</Increment>

<Unit>

s

</Unit>

</UsageDetail>

<CallId encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

</CallId>

</Destination>

<Destination>

<DestinationSignalAddress>

[10.0.1.2]:112

</DestinationSignalAddress>

<Token encoding="base64">

F467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756tYT64VQpfy

8HHGTrfvhJhjH776tbB9HG4VQbnj756HGTrfvbnjn7GhIGfH

ujpfyF47GhIGfHfYT64VQbnj6ghyHhHU

</Token>

<ValidAfter>

1998-04-24T17:01:02Z

</ValidAfter>

<ValidUntil>

1998-04-24T17:11:02Z

</ValidUntil>

<UsageDetail>

<Service/>

<Amount>

24

</Amount>

<Increment>

3600

</Increment>

<Unit>

s

</Unit>

</UsageDetail>

<CallId encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

</CallId>

</Destination>

</AuthorizationResponse>

</Message>

# E.3	Usage Exchange

The final examples demonstrate the exchange of usage information. The first message reports a call duration of 10 minutes.

POST scripts/settlements HTTP/1.0

content-type: text/plain

Content-Length: 926

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<UsageIndication componentId="b">

<Timestamp>

1998-04-24T22:03:00Z

</Timestamp>

<Role>

source

</Role>

<TransactionId>

67890987

</TransactionId>

<CallId encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

</CallId>

<SourceInfo type="e164">

81458811202

</SourceInfo>

<SourceAlternate type="subscriber">

8912342718473772

</SourceAlternate>

<DestinationInfo type="e164">

4766841360

</DestinationInfo>

<DestinationAlternate type="transport">

[10.0.1.2]:112

</DestinationAlternate>

<UsageDetail>

<Service/>

<Amount>

10

</Amount>

<Increment>

60

</Increment>

<Unit>

s

</Unit>

<StartTime critical="false">

1999-05-02T19:03:00Z

</StartTime>

<EndTime critical="false">

1999-05-02T19:13:00Z

</EndTime>

<TerminationCause critical="false">

<TCCode>

1016

</TCCode>

<Description>

normal call clearing

</Description>

</TerminationCause>

</UsageDetail>

</UsageIndication>

</Message>

To complete the exchange, the server responds with a UsageConfirmation.

HTTP/1.0 200 OK

content-type: text/plain

Content-Length: 404

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<UsageConfirmation componentId="b">

<Timestamp>

1998-04-24T22:44:00Z

</Timestamp>

<Status>

<Code>

201

</Code>

<Description>

new usage information created

</Description>

</Status>

</UsageConfirmation>

</Message>

# E.4	Subscriber Authentication Exchange

The following two messages show the authentication of an individual subscriber. The first message requests authentication for ISO/IEC 7812-1 [30] (calling card or charge card) number 5100123456789012.

POST scripts/settlements HTTP/1.0

content-type: text/plain

Content-Length: 336

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<SubscriberAuthenticationRequest componentId="b">

<Timestamp>

1998-04-24T17:03:00Z

</Timestamp>

<SourceInfo type="iso7812">

5100123456789012

</SourceInfo>

</SubscriberAuthenticationRequest>

</Message>

The following response indicates a successful authentication.

HTTP/1.0 200 OK

content-type: text/plain

Content-Length: 427

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<SubscriberAuthenticationResponse componentId="b">

<Timestamp>

1998-04-24T17:03:01Z

</Timestamp>

<Status>

<Code>

200

</Code>

<Description>

success

</Description>

</Status>

</SubscriberAuthenticationResponse>

</Message>

# E.5	Capabilities Exchange

This section illustrates the exchange of capabilities between client and server. In the first message, the client identifies its manufacturer’s serial number, indicates that it can support OSP version 2.1.1, that it expects to send AuthorizationRequest and UsageIndication messages, and that it has a total of 64 Kbit/s of capacity available.

POST scripts/settlements HTTP/1.0

content-type: text/plain

Content-Length: 779

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<CapabilitiesIndication componentId="b" critical="false">

<DeviceInfo type="serialnumber" critical="false">

12345678

</DeviceInfo>

<OSPVersion critical="false">

2.1.1

</OSPVersion>

<OSPCapability critical="false">

AuthorizationRequest

</OSPCapability>

<OSPCapability critical="false">

UsageIndication

</OSPCapability>

<Resources critical="false">

<DataRate critical="false">

<Bandwidth critical="false">

64000

</Bandwidth>

</DataRate>

</Resources>

</CapabilitiesIndication>

</Message>

The server replies with a confirmation that it can support OSP version 2.1.1, and it provides URLs for the client to use for OSP services. Note that the server includes an HTTP "Expires" header, indicating that the client should reconfirm its capabilities prior to 1 December 2000.

HTTP/1.0 200 OK

Expires: Thu, 01 Dec 2000 16:00:00 GMT

content-type: text/plain

Content-Length: 1542

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<CapabilitiesConfirmation componentId="b" critical="false">

<Timestamp>

1998-04-24T22:44:00Z

</Timestamp>

<Status>

<Code>

200

</Code>

</Status>

<OSPVersion critical="false">

2.1.1

</OSPVersion>

<OSPService critical="false">

<OSPCapability critical="false">

AuthorizationRequest

</OSPCapability>

<OSPServiceURL critical="false">

http://server1.domain.com/osp/authreq

</OSPServiceURL>

<OSPServiceURL critical="false">

http://server2.domain.com/osp/authreq

</OSPServiceURL>

<OSPSignatureRequired critical="false">

false

</OSPSignatureRequired>

</OSPService>

<OSPService critical="false">

<OSPCapability critical="false">

UsageIndication

</OSPCapability>

<OSPServiceURL critical="false">

http://server1.domain.com/osp/usageind

</OSPServiceURL>

<OSPServiceURL critical="false">

http://server2.domain.com/osp/usageind

</OSPServiceURL>

<OSPSignatureRequired critical="false">

false

</OSPSignatureRequired>

</OSPService>

</CapabilitiesConfirmation>

</Message>

Annex F (informative):
Billing Format Conversion

A wide variety of billing formats are currently in use within telecommunications and data networking systems. One of the advantages of using Extensible Markup Language within the present document is that it allows extremely easy conversion of usage information to and from other formats. As a demonstration of that flexibility, this annex includes example software to covert from <UsageIndication> components to a proprietary call detail record format of one particular, commercially-available system—the VocalTec Internet Telephony Gateway version 3.1. The software is written in Java, and relies on an XML parser available from Microsoft at http://www.microsoft.com/xml. Both the gateway and XML parser are selected solely to demonstrate the ease of the format conversion; this annex does not imply the suitability or fitness of either product for any purpose.

import com.ms.xml.parser.*;

import com.ms.xml.om.Document;

import com.ms.xml.om.Element;

import com.ms.xml.util.XMLOutputStream;

import com.ms.xml.util.Name;

import com.ms.xml.om.ElementEnumeration;

import com.ms.xml.om.ElementImpl;

import java.util.Enumeration;

import java.io.*;

import java.io.PrintStream;

import java.net.*;

import java.util.*;

import com.ms.xml.util.Name;

import com.ms.xml.om.ElementFactory;

class CDR

{

public static void main(String args[])

{

// "fix" the version to 2 digits to workaround a bug in the XML parser

Properties props = new Properties ();

props = System.getProperties ();

props.put ("java.version", "1.1");

System.setProperties (props);

parseArgs (args);

if (fileName == null)

printUsage (System.out);

else

{

URL url = createURL (fileName);

Document d = null;

try

{

d = new Document ();

d.setCaseInsensitive (true);

d.setLoadExternal (true);

d.load (url);

}

catch (ParseException e)

{

System.out.println (e);

}

if (d != null)

{

loadValues (d);

// type

cdr = pad ("Voice", 10);

// date

cdr += pad (((timestamp == null) ? "  --" :

timestamp.substring (5, 10) + "-" + timestamp.substring (2, 4)), 10);

// time

cdr += pad (((timestamp == null) ? "  --" :

timestamp.substring (11, 19)), 10);

// duration

cdr += pad (((unit != null && unit.equals ("s") && amount != null &&

increment != null) ? "" + Integer.parseInt (amount) *

Integer.parseInt (increment) : "  --"), 10);

// username

cdr += pad ("User Name", 30);

// dialed number

cdr += pad (destinationinfo, 30);

// line

cdr += pad ("1", 10);

// remote name

cdr += pad ("Remote Name", 30);

// disconnect reason

cdr += pad ("Disconnect Reason", 40);

// remote IP

cdr += pad ("000.000.000.000", 20);

// remote code

cdr += pad ("0000", 15);

// call type

cdr += pad ((role != null && role.equals ("destination") ? "IPVOD" : "OPVOD"), 15);

// user ID

cdr += pad ("000000", 10);

// fax pages

cdr += pad ("", 13);

// remote fax ID

cdr += pad ("", 25);

// local fax ref

cdr += pad ("", 16);

// remote fax ref

cdr += pad ("", 16);

// fax transfer mode

cdr += pad ("", 18);

// E.164 number

cdr += pad (destinationinfo, 24);

System.out.println (cdr);

}

}

System.exit(0);

}

static void printUsage (PrintStream o)

{

o.println ("Usage:  java CDR filename");

o.println ();

}

static void parseArgs (String args[])

{

if (args.length> 0)

fileName = args [0];

}

static String pad (String s, int num)

{

String str;

int len;

if (s == null)

{

str = "  --";

len = 4;

}

else

{

str = s;

len = s.length ();

if (len> num)

{

str = "  --";

len = 4;

}

}

for (int i = len; i <num; i++)

str += " ";

return str;

}

static URL createURL (String fileName)

// Microsoft method

{

URL url = null;

try

{

url = new URL (fileName);

}

catch (MalformedURLException ex)

{

File f = new File (fileName);

try

{

String path = f.getAbsolutePath();

String fs = System.getProperty("file.separator");

if (fs.length() == 1)

{

char sep = fs.charAt (0);

if (sep != '/')

path = path.replace (sep, '/');

if (path.charAt (0) != '/')

path = '/' + path;

}

path = "file://" + path;

url = new URL (path);

}

catch (MalformedURLException e)

{

System.out.println ("Cannot create url for: " + fileName);

System.exit(0);

}

}

return url;

}

static void loadValues (Element e)

{

String attName = "--NULL--";

String tagName;

if (e.getTagName () != null)

{

attName = e.getText ();

tagName = e.getTagName ().toString ();

if (tagName.equals ("TIMESTAMP"))

timestamp = attName;

if (tagName.equals ("DESTINATIONINFO"))

destinationinfo = attName;

if (tagName.equals ("AMOUNT"))

amount = attName;

if (tagName.equals ("INCREMENT"))

increment = attName;

if (tagName.equals ("UNIT"))

unit = attName;

if (tagName.equals ("ROLE"))

role = attName;

}

for (ElementEnumeration en = new ElementEnumeration (e); en.hasMoreElements(); )

{

Element child = (Element) en.nextElement();

loadValues (child);

}

}

static String fileName, timestamp, destinationinfo, amount, increment, unit, role, cdr;

}

Annex G (informative):
XML Overview

As an aid to those readers unfamiliar with Extensible Markup Language, this annex offers a brief overview of the syntax of XML documents. Key components of XML structure are document definitions, element declarations, and attribute declarations.

NOTE:	This annex is neither authoritative nor complete. A formal definition of XML may be found in [83].

# G.1	Document Definition

XML documents are defined using the following format:

<!DOCTYPE DocumentName [DocumentStructureDefinition]>

DocumentName is the name of the XML document, and DocumentStructureDefnition is a series of element, attribute, and entity declarations. Those declarations are described below. An example document definition is:

<!DOCTYPE Z [

<!ELEMENT Y (X, W)>

<!ATTLIST Y ('a' | 'b' | 'c') 'a' #REQUIRED>

<!ELEMENT X CDATA>

<!ELEMENT W CDATA>

]>

# G.2	Element Declaration

XML elements are declared using the following format:

<!ELEMENT ElementName (ElementContents)>

ElementName, as expected, is the name of the element being declared, while ElementContents specifies the permissible contents of the element. Elements may include child elements, in which case the ElementContents part defines their order and number within the parent element, and element may include character data.

Child elements within the ElementContents part of a declaration may be designated as a sequence of child elements or a choice of child elements. The comma (,) separates individual child elements in a sequence, while the vertical bar (|) separates child elements in a choice. For example, <!ELEMENT Y (X, W)> indicates that element Y consists of a child element X followed by a child element W. Similarly, <!ELEMENT Z (X | W)> indicates that element Z consists of either a child element X or a child element W.

The element declaration indicates the number of child elements permitted by a character following the child element's name. No character indicates that exactly one child element, a question mark (?) indicates zero or one child element, an asterisk (*) indicates zero or more child elements, and a plus (+) indicates one or more child elements.

For example:

<!ELEMENT X (A, B?, C*, D+)>

defines element X as consisting of:

Table G.1

| child element | element declaration | meaning |
| --- | --- | --- |
| A | (none) | exactly one child element |
| B | ? | zero or one child element |
| C | * | zero or more child elements |
| D | + | one or more child elements |

# G.3	Attribute Declaration

XML attributes are associated with elements; they are declared using the format:

<!ATTLIST ElementName AttributeName1 DeclaredValue1 DefaultValue1

AttributeName2 DeclaredValue2 DefaultValue2

...

AttributeNameN DeclaredValueN DefaultValueN>

ElementName identifies the XML element with which the attributes may be used. AttributeName1, AttributeName2, … are the names of the attributes defined for the element. Each attribute has either a list of permissible values or a data type, which the above construction labels DeclaredValue. The final part of each attribute's declaration is DefaultValue, which indicates the default value for the attribute, as well as constraints on its presence.

Explicit, permissible values for attributes are separated by the vertical bar (|). So that, for example the Boolean attribute for element X may be declared as:

<!ATTLIST X Boolean (true | false)>

NOTE 1:	No default value is specified in the above declaration. If a default value is specified, then the DefaultValue section may appear as one of:

#REQUIRED: some explicit value for the attribute shall be included each time the attribute is used;

#IMPLIED: if no explicit value for the attribute is used, then the application may infer a default value;

'value'	if no explicit value for the attribute is used, then the attribute should be assumed to have the value 'value';

#FIXED 'value': the attribute shall have, and can only have, the value 'value'.

EXAMPLE:	If the Boolean attribute should be assumed to be true unless otherwise, explicitly indicated, the following declaration would apply.

<!ATTLIST X Boolean (true | false) 'true'>

NOTE 2:	No quotation marks are used in the DeclaredValue part, while the DefaultValue part does enclose the value in quotations.

Annex H (informative):
Binary XML Content Format for OSP

In some implementations, minimizing the size of protocol messages is a critical requirement. The Binary XML Content Format [29], developed by the Wireless Application Protocol (WAP) Forum) provides a standard method for compressing XML content, and it can be directly applied to OSP messages. This annex provides the information necessary to use Binary XML Content Format in an interoperable manner by defining global extension tokens for OSP. It also includes an example message to illustrate the application of the format.

# H.1	Global Extension Tokens

The following tables define the global extension tokens for OSP.

Table H.1: Global extension tokens for OSP tags

| CODE PAGE | TOKEN | TAG NAME |
| --- | --- | --- |
| 0 | 05 | AlmostOutOfResources |
| 0 | 06 | Amount |
| 0 | 07 | AuthorityURL |
| 0 | 08 | AuthorizationConfirmation |
| 0 | 09 | AuthorizationIndication |
| 0 | 0A | AuthorizationRequest |
| 0 | 0B | AuthorizationResponse |
| 0 | 0C | Bandwidth |
| 0 | 0D | CallId |
| 0 | 0E | Certificate |
| 0 | 0F | CertificateChain |
| 0 | 10 | Code |
| 0 | 11 | Currency |
| 0 | 12 | DataRate |
| 0 | 13 | Description |
| 0 | 14 | Destination |
| 0 | 15 | DestinationAlternate |
| 0 | 16 | DestinationInfo |
| 0 | 17 | DestinationSignalAddress |
| 0 | 18 | DeviceId |
| 0 | 19 | DeviceInfo |
| 0 | 1A | EndTime |
| 0 | 1B | Fraction |
| 0 | 1C | Increment |
| 0 | 1D | LossReceived |
| 0 | 1E | LossSent |
| 0 | 1F | MaximumDestinations |
| 0 | 20 | Mean |
| 0 | 21 | Message |
| 0 | 22 | Minimum |
| 0 | 23 | NumberOfChannels |
| 0 | 24 | OneWayDelay |
| 0 | 25 | Packets |
| 0 | 26 | ReauthorizationRequest |
| 0 | 27 | ReauthorizationResponse |
| 0 | 28 | Resources |
| 0 | 29 | Role |
| 0 | 2A | RoundTripDelay |
| 0 | 2B | Samples |
| 0 | 2C | Service |
| 0 | 2D | SourceAlternate |
| 0 | 2E | SourceInfo |
| 0 | 2F | SourceSignalAddress |
| 0 | 30 | StartTime |
| 0 | 31 | Statistics |
| 0 | 32 | Status |
| 0 | 33 | TCCode |
| 0 | 34 | TerminationCause |
| 0 | 35 | Timestamp |
| 0 | 36 | Token |
| 0 | 37 | TokenInfo |
| 0 | 38 | TransactionId |
| 0 | 39 | Unit |
| 0 | 3A | UsageConfirmation |
| 0 | 3B | UsageDetail |
| 0 | 3C | UsageIndication |
| 0 | 3D | ValidAfter |
| 0 | 3E | ValidUntil |
| 0 | 3F | Variance |
| 1 | 05 | CapabilitiesConfirmation |
| 1 | 06 | CapabilitiesIndication |
| 1 | 07 | OSPCapability |
| 1 | 08 | OSPService |
| 1 | 09 | OSPServiceURL |
| 1 | 0A | OSPSignatureRequired |
| 1 | 0B | OSPVersion |
| 1 | 0C | PricingConfirmation |
| 1 | 0D | PricingIndication |
| 1 | 0E | SubscriberAuthenticationInfo |
| 1 | 0F | SubscriberAuthenticationRequest |
| 1 | 10 | SubscriberAuthenticationResponse |

Table H.2: Global extension tokens for OSP attributes

| TOKEN | ATTRIBUTE |
| --- | --- |
| 05 | componentId |
| 06 | critical false |
| 07 | critical true |
| 08 | encoding base64 |
| 09 | encoding cdata |
| 0A | messageId |
| 0B | random |
| 0C | type abbreviated |
| 0D | type customerId |
| 0E | type deviceId |
| 0F | type e164 |
| 10 | type e164prefix |
| 11 | type email |
| 12 | type epin |
| 13 | type h323 |
| 14 | type international |
| 15 | type iso7812 |
| 16 | type national |
| 17 | type network |
| 18 | type pin |
| 19 | type serialnumber |
| 1A | type subscriber |
| 1B | type transport |
| 1C | type url |

# H.2	Example Application

This section illustrates the process of converting an OSP message to Binary XML Content Format. It uses the example AuthorizationRequest from annex E.

## H.2.1	Standard XML Format (505 bytes)

<?xml version='1.0'?>

<Message messageId="a" random="1234">

<AuthorizationRequest componentId="b">

<Timestamp>

1998-04-24T17:03:00Z

</Timestamp>

<CallId encoding="base64">

YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t

</CallId>

<SourceInfo type="e164">

81458811202

</SourceInfo>

<DestinationInfo type="e164">

4766841360

</DestinationInfo>

<Service/>

<MaximumDestinations>

5

</MaximumDestinations>

</AuthorizationRequest>

</Message>

## H.2.2	Binary XML Content Format (160 bytes)

02 01 03 00

E1 0A 03 'a'  00 0B 03 '1234' 00

CA 05 03 'b' 00

75 03 '1998-04-24T17:03:00Z' 00 01

CD 08 01 03

'YT64VQpfyF467GhIGfHfYT6jH77n8HHGghyHhHUujhJh756t'00 01

EE 0F 01 03 '81458811202'00 01

D6 0F 01 03 '4766841360'00 01

2C

5F 03 '5'00 01

01

01

-

|  |  |  | 1 |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1 |  |

o

## I.3.5	Secure Socket Layer Security Capabilities

Table I.

| Table I.21 |  |  |  |  |
| --- | --- | --- | --- | --- |

C302:	IF I.3/1 
		THEN o
		ELSE n/a

: SSL Cipher Suite support Capabilities

## I.3.6	Hypertext Transfer Protocol Capabilities

: TCP Port capabilities

C701:	IF I.2/1 
		THEN m
		ELSE n/a

C702:	IF NOT I.2/1
		THEN m
		ELSE n/a

Table I.

Table I.

Table I.

Table I.

Table I.

| Table I.23 |  |  |  |  |
| --- | --- | --- | --- | --- |

| Table I.23 |  |  |  |  |
| --- | --- | --- | --- | --- |

| Table I.23 |  |  |  |  |
| --- | --- | --- | --- | --- |

| Table I.23 |  |  |  |  |
| --- | --- | --- | --- | --- |

| Table I.24 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 27 |  |  |  |  |  |  |

|  |  |  | 16 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16 |  |  |  |
|  |  |  | 16 |  |  |  |

: Identifier attribute Capabilities

: PDU en/decoding

Support

|  |  | 9 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |

|  |  | 9 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |
|  |  | 9 |  |  |  |  |

Table I.

Annex J (informative):
OSP Applications and Implementations

This annex describes the application of ETSI TS 101 321, the Open Settlement Protocol, in various network implementations. Further call scenarios are described in [84].

# J.1	Call Control Protocols

The Open Settlement Protocol (OSP) is not limited to any particular call control protocol. Rather, it is neutral, and is designed for deployment with devices conforming to H.323 [13] and Session Initiation Protocol (SIP), as well as various proprietary signalling protocols.

NOTE: 	Some well-known IP telephony protocols, including the Simple Gateway Control Protocol (SGCP), IP Device Control (IPDC), Media Gateway Control Protocol (MGCP), and MEGACO/H.248, do not integrate directly with OSP. Instead, systems relying on those protocols integrate with OSP at the level of protocols between gateway controllers (e.g. call agents). Today, those protocols are primarily based on H.323 [13] or SIP.

This section illustrates how OSP may be used with various call signalling protocols. Rather than considering each protocol separately, however, it considers three different architectures—peer-to-peer, distributed tightly-coupled, and distributed loosely-coupled—that can be applied to various signalling protocols. The H.323 [13] and SIP protocols are used as examples to describe the interaction with OSP in detail. The principals described in these sections can also be used for other protocols.

## J.1.1	Peer-to-Peer Architecture

In a peer-to-peer architecture, gateways contact each other directly, without any control or co-ordination from other devices. This architecture corresponds to simple H.323 [13] deployments without gatekeepers as well as basic SIP-based services. In such an environment, the endpoints of a call implement the open settlement protocol to find and authorize each other, and to directly report usage information to a settlement provider. The following subsections illustrate the use of OSP and in both H.323 [13] and SIP architectures.

### J.1.1.1	H.323    Gateways

When operating with H.323 [13] gateways, OSP provides services at both the start and end of each call. During initial call setup, gateways can use OSP to obtain call routing information and authorization tokens. Once the call has ended, gateways use OSP to report usage details.

#### J.1.1.1.1	Call Routing and Authorization

The following figure shows how the open settlement protocol may be used by H.323 [13] gateways to find and authorize each other.

Figure J.1

The figure highlights the following interactions between the devices.

Calling party indicates the desired, destination phone number to Gateway A (by, for example, responding to a DTMF-based IVR, sending an ISDN Q.931 Setup, or transmitting an SS7 Initial Address Message).

Gateway A sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include:

| <Timestamp> | Time of request |
| --- | --- |
| <CallId> | H.323 [13] Call Identifier to be used for the call |
| <SourceInfo type="e164"> | Calling party’s E.164 [12] number if available; otherwise a local E.164 [12] number controlled by Gateway A, e.g. 14048724887; this number shall be passed to the destination gateway(s) in Setup messages |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | Called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | Empty (for basic service) |
| <MaximumDestinations> | The maximum number of destinations, including alternatives, Gateway A will consider |

OSP Server replies with an <AuthorizationResponse> message. The message indicates two candidate destinations: Gateway B and Gateway C, in that order. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway B, for example gatewayB.itsp.fr |
| <Token> | authorization token to be passed to Gateway B |
| <ValidAfter> | time after which token for Gateway B is valid |
| <ValidUntil> | time until which token for Gateway B is valid |
| <UsageDetail> | how much service is authorized with Gateway B |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to Gateway B |
| <Destination> | second destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway C, for example gatewayC.isp.fr |
| <Token> | authorization token to be passed to Gateway C |
| <ValidAfter> | time after which token for Gateway C is valid |
| <ValidUntil> | time until which token for Gateway C is valid |
| <UsageDetail> | how much service is authorized with Gateway C |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to Gateway C |

Gateway A sends an H.225.0 [9] Setup message to Gateway B; however, the Setup is refused with a Release Complete.

NOTE:	The H.323 [13] Call Identifier in the Setup message has the same value as in the original <AuthorizationRequest>. The Setup shall also include the authorization token(s) provided in the OSP <AuthorizationResponse>. For maximum interoperability, any tokens should use the following object identifier, constructed according to ETSI guidelines.

itu-t(0), identified-organization(4), etsi(0), ts-101-321(1321), token(1), xml-format(2)

Gateway A then sends a second H.225.0 [9] Setup message, this time to Gateway C, and this time the Setup is accepted with a Setup Acknowledge. This Setup message also uses the H.323 [13] Call Identifier from the <AuthorizationRequest>, and the Setup message shall also include the authorization token(s) provided in the OSP <AuthorizationResponse>. For maximum interoperability, any tokens should use the following object identifier, constructed according to ETSI guidelines.

itu-t(0), identified-organization(4), etsi(0), ts-101-321(1321), token(1), xml-format(2)

Gateway C accepts the Setup and completes the call to the called party; the phone conversation can now take place.

#### J.1.1.1.2	Usage Reports

Once the call has ended, both gateways report usage details to an OSP server. As the following figure indicates, those reports are conveyed in OSP <UsageIndication> messages.

Figure J.2

The steps shown in the figure are straightforward.

Gateways A and C clear the call by exchanging an H.225.0 [9] Release Complete message.

Gateway A sends a <UsageIndication> message to the OSP server. If Gateway A is not using any protocol extensions, the message will contain the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as returned in the authorization response, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Gateway C also sends a <UsageIndication> to the OSP server. That message would include the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gateway C, destination |
| <TransactionId> | transaction ID assigned by OSP server and passed to Gateway C in authorization token |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the Setup message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example [172.16.1.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by Gateway C |
| <Packets> | number of packets lost from Gateway C to Gateway A |
| <Fraction> | fraction (from 0 to 255) of packets lost from C to A |
| <LossReceived> | loss information for packets sent by Gateway A |
| <Packets> | number of packets lost from Gateway A to Gateway C |
| <Fraction> | fraction (from 0 to 255) of packets lost from A to C |
| <OneWayDelay> | one way delay measured from Gateway A to C |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between Gateway A and C measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

### J.1.1.2	Session Initiation Protocol Gateways

In a simple peer-to-peer environment, Session Initiation Protocol (SIP) gateways operate in a manner nearly identical to H.323 [13] gateways. As before, it is convenient to consider interaction using OSP before and after the multimedia call.

#### J.1.1.2.1	Call Routing and Authorization

The following figure shows how the open settlement protocol may be used by SIP gateways to find and authorize each other.

NOTE 1:	This function allows a SIP gateway to find a peer gateway for a particular, PSTN-terminated, telephone user. As such, it is not the same as the standard SIP user location function.

Figure J.3

The figure highlights the following interactions between the devices.

Calling party indicates the desired, destination phone number to Gateway A (by, for example, responding to a DTMF-based IVR, sending an ISDN Q.931 Setup, or transmitting an SS7 Initial Address Message).

Gateway A sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include:

| <Timestamp> | time of request |
| --- | --- |
| <CallId> | SIP Call-ID to be used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number if available; otherwise a local E.164 [12] number controlled by Gateway A, e.g. 14048724887; this number shall be passed to the destination gateway(s) in the subsequent INVITE method |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | empty (for basic service) |
| <MaximumDestinations> | the maximum number of destinations, including alternatives, Gateway A will consider |

OSP Server replies with an <AuthorizationResponse> message. The message indicates two candidate destinations: Gateway B and Gateway C, in that order. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway B, for example gatewayB.itsp.fr |
| <Token> | authorization token to be passed to Gateway B |
| <ValidAfter> | time after which token for Gateway B is valid |
| <ValidUntil> | time until which token for Gateway B is valid |
| <UsageDetail> | how much service is authorized with Gateway B |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | SIP Call-ID to be used for the call to Gateway B |
| <Destination> | second destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway C, for example gatewayC.isp.fr |
| <Token> | authorization token to be passed to Gateway C |
| <ValidAfter> | time after which token for Gateway C is valid |
| <ValidUntil> | time until which token for Gateway C is valid |
| <UsageDetail> | how much service is authorized with Gateway C |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | SIP Call-ID to be used for the call to Gateway C |

Gateway A sends an INVITE message to Gateway B; however, Gateway B cannot accept the request. It responds with a "503 Service Unavailable," which Gateway A acknowledges with an ACK.

NOTE 2:	The SIP Call-ID in the INVITE message has the same value as in the original <AuthorizationRequest>. The INVITE shall also include authorization token(s) provided in the OSP <AuthorizationResponse>. Each token should be conveyed in the SIP header, as initially defined in: draft-johnston-sip-osp-token-01.txt.

Editor’s Note: 21TD089

Gateway A then sends a second INVITE message, this time to Gateway C, and this time the INVITE is accepted with "200 Success," to which Gateway A replies with an ACK. This INVITE message also uses the SIP Call-ID from the <AuthorizationRequest>, and the INVITE shall also include the authorization token(s) provided in the OSP <AuthorizationResponse>. Each token should be conveyed in the SIP header, as initially defined in: draft-johnston-sip-osp-token-01.txt.

Gateway C accepts the INVITE and completes the call to the called party; the phone conversation can now take place.

#### J.1.1.2.2	Usage Reports

Once the call has ended, both gateways report usage details to an OSP server. As the following figure indicates, those reports are conveyed in OSP <UsageIndication> messages.

Figure J.4

The steps shown in the figure are straightforward.

Gateways A and C clear the call with a SIP BYE message.

Gateway A sends a <UsageIndication> message to the OSP server. If Gateway A is not using any protocol extensions, the message will contain the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as returned in the authorization response, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Gateway C also sends a <UsageIndication> to the OSP server.

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gateway C, destination |
| <TransactionId> | transaction ID assigned by OSP server and passed to Gateway C in authorization token |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the INVITE message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example [172.16.1.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by Gateway C |
| <Packets> | number of packets lost from Gateway C to Gateway A |
| <Fraction> | fraction (from 0 to 255) of packets lost from C to A |
| <LossReceived> | loss information for packets sent by Gateway A |
| <Packets> | number of packets lost from Gateway A to Gateway C |
| <Fraction> | fraction (from 0 to 255) of packets lost from A to C |
| <OneWayDelay> | one way delay measured from Gateway A to C |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between Gateway A and C measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

## J.1.2	Tightly Controlled Distributed Architecture

The Open Settlement Protocol supports operation in a tightly controlled, distributed architecture. That architecture includes some H.323 [13]-based deployments with active gatekeepers and SIP proxy servers (but not SIP redirect servers). The following subsections illustrate the use of OSP in those environments.

### J.1.2.1	H.323    Gatekeeper Routed Calls

In H.323 [13] deployments with gatekeepers, the gatekeeper may play a very active roll in call signalling. In such an environment, gatekeepers not only assume responsibility for call routing and authorization on behalf of their endpoints. They also act as the signalling endpoint for calls into their zone. As the following figure shows, gatekeepers may support multimedia terminals as well as gateways. As that figure and the following figure also highlight, only gatekeepers are required to support the Open Settlement Protocol in this environment.

Figure J.5

NOTE:	All destination gateways (or endpoints) controlled by the gatekeepers shall be equivalent, as far as the OSP server is concerned. Indeed, the OSP server is not even aware of the existence of any H.323 [13] terminals or gateways. Such an architecture, therefore, places implicit restrictions on the services offered by the operators. An operator, for example, will be unable to price termination services differently for different gateways in the same zone.

#### J.1.2.1.1	Call Routing and Authorization

H.323 [13] Terminal begins a call by sending an H.225.0 [9] Setup message its gatekeeper, Gatekeeper A.

NOTE:	This scenario assumes the use of pre-granted admission requests.

The Setup indicates that the called party is identified by an E.164 [12] phone number such as +33 4 92 94 42 99.

Gatekeeper A sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include:

| <Timestamp> | time of request |
| --- | --- |
| <CallId> | H.323 [13] Call Identifier to be used for the call |
| <SourceInfo type="e164"> | a representation of the H.323 [13] Terminal using an E.164 [12] number; in the absence of other information, this number may be derived using the IP address to E.164 [12] number mapping of ETSI TIPHON; this number shall be passed to the destination gateway(s) in Setup messages |
| <SourceAlternate type="transport"> | DNS name or IP address of Gatekeeper A, for example gatekeeperA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | empty (for basic service) |
| <MaximumDestinations> | the maximum number of destinations, including alternatives, Gatekeeper A will consider |

OSP Server replies with an <AuthorizationResponse> message. The message identifies Gatekeeper B. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gatekeeper B, for example gatekeeperB.itsp.fr |
| <Token> | authorization token to be passed to Gatekeeper B |
| <ValidAfter> | time after which token for Gatekeeper B is valid |
| <ValidUntil> | time until which token for Gatekeeper B is valid |
| <UsageDetail> | how much service is authorized with Gatekeeper B |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to Gatekeeper B |

Gatekeeper A extends the call with its own Setup message to Gatekeeper B.

Gatekeeper B accepts the Setup and extends the call to the destination gateway with another Setup/Setup Acknowledge exchange.

The Gateway adds the final leg of the call by dialling the called party; the phone conversation can now take place.

#### J.1.2.1.2	Usage Reports

At the conclusion of the phone call, both gatekeepers shall report usage information to the OSP server. The following figure identifies the principle steps of a typical call completion.

Figure J.6

The steps shown in the figure are straightforward.

The gatekeepers release the call with an exchange of H.225.0 [9] Release and Release Complete messages.

Gatekeeper A then sends a <UsageIndication> message to the OSP server. In this example Gatekeeper A may not be able to support protocol extensions such as statistics. Its message, therefore contains just the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gatekeeper A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as returned in the authorization response, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gatekeeper A, for example gatekeeperA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gatekeeper B, for example, gatekeeperB.itsp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Gatekeeper B also sends a <UsageIndication> to the OSP server. That message would include the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gatekeeper B, destination |
| <TransactionId> | transaction ID assigned by OSP server and passed to Gatekeeper B in authorization token |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the Setup message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of H.323 [13] Terminal, for example [172.16.100.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gatekeeper B, for example, gatekeeperB.itsp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | Increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by Gatekeeper B |
| <Packets> | number of packets lost from Gatekeeper B to H.323 [13] Terminal |
| <Fraction> | fraction (from 0 to 255) of packets lost from B to H.323 [13] Terminal |
| <LossReceived> | loss information for packets sent by H.323 [13] Terminal |
| <Packets> | number of packets lost from H.323 [13] Terminal to Gatekeeper B |
| <Fraction> | fraction (from 0 to 255) of packets lost from Terminal to B |
| <OneWayDelay> | one way delay measured from Terminal to B |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between Terminal and B measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

### J.1.2.2	Session Initiation Protocol Proxy Servers

Session Initiation Protocol (SIP) proxy servers may function in a manner similar to H.323 [13] gatekeepers. In such an environment, the proxy servers may support the Open Settlement Protocol, while the systems on whose behalf they act need not.

NOTE:	All destination gateways (or endpoints) served by the SIP proxies shall be equivalent, as far as the OSP server is concerned. Indeed, the OSP server is not even aware of the existence of any endpoints or gateways. Such an architecture, therefore, places implicit restrictions on the services offered by the operators. An operator, for example, will be unable to price termination services differently for different gateways served by the same proxy.

#### J.1.2.2.1	Call Routing and Authorization

The following figure shows how OSP plays a role in environments that use SIP Proxy Servers. In the figure, The SIP Proxy Server, acting on behalf of the SIP client, completes a call to a SIP Gateway, and ultimately to the PSTN.

Figure J.7

The SIP Client begins a call by sending an INVITE request to its proxy server. The INVITE indicates that the called party is identified by an E.164 [12] phone number such as +33 4 92 94 42 99.

The SIP Proxy Server sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include the following:

| <Timestamp> | time of request |
| --- | --- |
| <CallId> | SIP Call-ID to be used for the call |
| <SourceInfo type="e164"> | a representation of the SIP client using an E.164 [12] number; in the absence of other information, this number may be derived using the IP address to E.164 [12] number mapping of ETSI TIPHON. |
| <SourceAlternate type="transport"> | DNS name or IP address of the Proxy Server, for example proxy.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | empty (for basic service) |
| <MaximumDestinations> | the maximum number of destinations, including alternatives, the Proxy Server will consider |

OSP Server replies with an <AuthorizationResponse> message. The message identifies the SIP gateway. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of destination gateway, e.g. gateway.itsp.fr |
| <Token> | authorization token to be passed to Gateway |
| <ValidAfter> | time after which token for Gateway is valid |
| <ValidUntil> | time until which token for Gateway is valid |
| <UsageDetail> | how much service is authorized with Gateway |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | SIP Call-ID to be used for the call to the Gateway |

The Proxy Server, acting on behalf of its client, sends an SIP INVITE to the Gateway. In this example, the Gateway accepts the connection with an 200, and the proxy responds with an ACK. This INVITE message also uses the SIP Call-ID from the <AuthorizationRequest>, and the INVITE shall also include the authorization token(s) provided in the OSP <AuthorizationResponse>. Each token should be conveyed in the SIP header, as initially defined in: draft-johnston-sip-osp-token-01.txt.

The Proxy Server, on establishing the connection with the Gateway, completes the connection with its client as well with a SIP 200/ACK exchange.

The Gateway, meanwhile, completes the call to the destination phone number.

#### J.1.2.2.2	Usage Reports

At the conclusion of the phone call, both the Proxy Server and Gateway report usage information to the OSP server. The following figure identifies the principle steps of a typical call completion.

Figure J.8

The steps shown in the figure are straightforward.

The Proxy Server and Gateway release the call by transferring a SIP BYE message.

The Proxy Server then sends a <UsageIndication> message to the OSP server. In this example, the Proxy Server does not support new protocol extensions such as statistics. Its message, therefore contains just the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Proxy Server, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as returned in the authorization response, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of the Proxy Server, for example proxy.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of the Gateway, for example, gateway.itsp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

The Gateway also sends a <UsageIndication> to the OSP server. That message would include the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for the Gateway, destination |
| <TransactionId> | transaction ID assigned by OSP server and passed to the gateway in authorization token |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the INVITE message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of the Proxy Server, for example [172.16.100.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of the Gateway, for example, gateway.itsp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | Increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by the Gateway |
| <Packets> | number of packets lost from the Gateway to the Proxy Server |
| <Fraction> | fraction (from 0 to 255) of packets lost from the Gateway to the Proxy Server |
| <LossReceived> | loss information for packets sent by the Proxy Server |
| <Packets> | number of packets lost from Proxy Server to the Gateway |
| <Fraction> | fraction (from 0 to 255) of packets lost from Proxy Server to Gateway |
| <OneWayDelay> | one way delay measured from Proxy Server to Gateway |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between Proxy Server and Gateway measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

## J.1.3	Loosely Controlled Distributed Architecture

The Open Settlement Protocol can also be used in environments based on loosely-coupled, distributed architectures. One such environment is an H.323 [13]-based architecture with direct call signalling. Another example is the use Session Initiation Protocol (SIP) redirect servers. This subsection describes the use of OSP in each of these architectures.

### J.1.3.1	H.323    Direct Routed Calls (with Gatekeepers)

When H.323 [13] gateways and gatekeepers both implement the Open Settlement Protocol, it is possible to use OSP in an architecture that relies on H.323 [13] Direct Call Signalling (as opposed to Gatekeeper Call Signalling). Such an approach is a hybrid of the peer-to-peer gateway architecture and the tightly controlled gatekeeper model discussed in the previous two subsections. In this approach, the gatekeepers acts as agents for call routing and authorization, but the gateways themselves are responsible for establishing and disconnecting calls directly with each other. In addition to gateways, H.323 [13] proxy devices may also be used to support this model on behalf of H.323 [13] terminals.

#### J.1.3.1.1	Call Routing and Authorization

The following figure shows a sample routing and authorization scenario for this model.

NOTE 1:	This figure shows a finer level of detail than previous examples in order to clarify several subtle points.

Figure J.9

Gateway A begins a call by sending an H.225.0 [9] Admission Request (ARQ) to its gatekeeper, Gatekeeper A. The ARQ indicates that the called party is identified by an E.164 [12] phone number such as +33 4 92 94 42 99.

Gatekeeper A sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include:

| <Timestamp> | time of request |
| --- | --- |
| <CallId> | H.323 [13] Call Identifier to be used for the call |
| <SourceInfo type="e164"> | the called party's E.164 [12] number, or, if that is not available, a local E.164 [12] number belonging to Gateway A; this number shall be passed to the destination gateway(s) in Setup messages |
| <SourceAlternate type="transport"> | DNS name or IP address of Gatekeeper A, for example gatekeeperA.carrier.com |
| <SourceAlternate type="h323"> | H.323 [13] identifier of Gateway A, e.g. 12345678 |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | empty (for basic service) |
| <MaximumDestinations> | the maximum number of destinations, including alternatives, Gatekeeper A will consider |

OSP Server replies with an <AuthorizationResponse> message. The message identifies Gateway B and Gateway C as candidate destinations. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway B, for example gatewayB.itsp.fr |
| <Token> | authorization token to be passed to Gateway B |
| <ValidAfter> | time after which token for Gateway B is valid |
| <ValidUntil> | time until which token for Gateway B is valid |
| <UsageDetail> | how much service is authorized with Gateway B |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to Gateway B |
| <Destination> | second destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of destination Gateway C, e.g. gatewayC.isp.fr |
| <Token> | authorization token to be passed to Gateway C |
| <ValidAfter> | time after which token for Gateway C is valid |
| <ValidUntil> | time until which token for Gateway C is valid |
| <UsageDetail> | how much service is authorized with Gateway C |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to Gateway C |

Gatekeeper A sends an H.225.0 [9] Admission Confirm (ACF) message to its gateway. That ACF identifies the destination as Gateway B and it shall include the authorization token for gateway B.

Gateway A sends an H.225.0 [9] Setup message to Gateway B. This Setup message uses the H.323 [13] Call Identifier from the <AuthorizationRequest>, and the message shall include the authorization token(s) provided in the OSP <AuthorizationResponse>.

Gateway B refuses the setup attempt with a Release Complete message, perhaps, for example, because no outbound PSTN ports are available.

Gateway A sends another Admission Request (ARQ) message to its gatekeeper to request an alternate destination for the phone call.

NOTE 2:	This ARQ shall use the same H.323 [13] Call Identifier as the original ARQ. For clarity, the example omits other details of the exchange between Gateway A and Gatekeeper A, such as, for example, a Disengage Request (DRQ) and Disengage Confirm (DCF).

Gatekeeper A replies with an Admission Confirm (ACF) message that identifies Gateway C as a potential destination.

NOTE 3:	The Gatekeeper need not query the OSP server to get that information. Rather, the information is available in the original <AuthorizationResponse> noted in step 3.

Gateway A tries a second setup attempt, this time by sending a Setup message to Gateway C.

Gateway C receives the Setup message and asks its gatekeeper for permission to accept the call. It does so by sending an Admission Request (ARQ) to Gatekeeper C.

NOTE 4:	This message shall include the authorization token(s) received in the Setup.

Gatekeeper C authorizes the call and returns an Admission Confirm (ACF) message to Gateway C.

Gateway C receives the ACF and accepts the call by returning a Setup Acknowledge message to Gateway A.

#### J.1.3.1.2	Usage Reports

At the conclusion of the phone call, both gateways shall report usage information to the OSP server. The following figure identifies the principle steps of a typical call completion.

Figure J.10

The steps shown in the figure are straightforward.

The gateways close the connection by exchanging H.323 [13] Release and Release Complete messages.

Gateway A then sends a <UsageIndication> message to the OSP server. In this example Gateway A’s message will include two complete <UsageIndication> components, one for the failed attempt and one for the successful call. The sub-elements for each will include the following:

| <UsageIndication> | usage information for the failed setup attempt |
| --- | --- |
| <Timestamp> | time of request |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number as returned in the authorization response, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway B, for example, gatewayB.itsp.fr |
| <FailureReason> | reason for failure of attempted setup, e.g. 422 |
| <UsageIndication> | usage information for the successful setup attempt |
| <Timestamp> | time of request |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatekeeperA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number as returned in the authorization response, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Gateway C also sends a <UsageIndication> to the OSP server. That message would include the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for Gateway C, destination |
| <TransactionId> | Transaction ID assigned by OSP server and passed to Gateway C in authorization token |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the Setup message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example [172.16.100.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.itsp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | Increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by Gateway C |
| <Packets> | number of packets lost from Gateway C to Gateway A |
| <Fraction> | fraction (from 0 to 255) of packets lost from C to A |
| <LossReceived> | loss information for packets sent by Gateway A |
| <Packets> | number of packets lost from Gateway A to Gateway C |
| <Fraction> | fraction (from 0 to 255) of packets lost from A to C |
| <OneWayDelay> | one way delay measured from A to C |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between A and C measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

### J.1.3.2	Session Initiation Protocol Redirect Servers

Session Initiation Protocol (SIP) redirect servers represent another example of the loosely coupled distributed architecture, particularly on the source side of a call. In this environment, the redirect server provides the call routing and authorization on behalf of the systems it serves, but those systems themselves report usage information.

#### J.1.3.2.1	Call Routing and Authorization

The following figure shows a sample routing and authorization scenario for SIP redirect servers.

Figure J.11

Gateway A begins a call by sending a SIP INVITE method to its redirect server. The INVITE indicates that the called party is identified by an E.164 [12] phone number such as +33 4 92 94 42 99.

The Redirect Server sends an OSP <AuthorizationRequest> message to the OSP Server. The significant elements within the <AuthorizationRequest> include:

| <Timestamp> | time of request |
| --- | --- |
| <CallId> | SIP Call-ID to be used for the call |
| <SourceInfo type="e164"> | calling party's E.164 [12] number if available; otherwise a local E.164 [12] number controlled by Gateway A (e.g. 14048724887); this number shall be passed to the destination gateway(s) in future INVITE messages |
| <SourceAlternate type="transport"> | DNS name or IP address of Redirect Server, for example redirect.carrier.com |
| <SourceAlternate type="h323"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com; the type "h323", in this case, implies a device-specific ID rather than a particular protocol |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | empty (for basic service) |
| <MaximumDestinations> | the maximum number of destinations, including alternatives, Redirect Server will consider |

OSP Server replies with an <AuthorizationResponse> message. The message identifies Gateway B and Gateway C as candidate destinations. In particular, the <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway B, for example gatewayB.itsp.fr |
| <Token> | authorization token to be passed to Gateway B |
| <ValidAfter> | time after which token for Gateway B is valid |
| <ValidUntil> | time until which token for Gateway B is valid |
| <UsageDetail> | how much service is authorized with Gateway B |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | SIP Call-ID to be used for the call to Gateway B |
| <Destination> | second destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of Gateway C, for example gatewayC.isp.fr |
| <Token> | authorization token to be passed to Gateway C |
| <ValidAfter> | time after which token for Gateway C is valid |
| <ValidUntil> | time until which token for Gateway C is valid |
| <UsageDetail> | how much service is authorized with Gateway C |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | SIP Call-ID to be used for the call to Gateway C |

The Redirect Server returns a "300 Multiple Choices" redirection status code to Gateway A. This response relays the data from the <AuthorizationResponse> to the gateway, explicitly identifying Gateways B and C as candidate destinations. This response shall also include the authorization tokens returned by the OSP Server as application/osp-token MIME types in the response body.

Gateway A sends an INVITE message to Gateway B. This INVITE message uses the SIP Call-ID from the <AuthorizationRequest>, and the message shall include all authorization tokens associated with that destination in the server's OSP <AuthorizationResponse>.

Gateway B refuses the setup attempt with a 503 message, perhaps, for example, because no outbound PSTN ports are available.

Gateway A tries a second setup attempt, this time by sending an INVITE message to Gateway C.

Gateway C receives the INVITE message and accepts the call by responding with a 200 message, to which Gateway A replies with an ACK.

#### J.1.3.2.2	Usage Reports

Once the call has ended, both gateways report usage details to an OSP server. As the following figure indicates, those reports are conveyed in OSP <UsageIndication> messages.

Figure J.12

The steps shown in the figure are straightforward.

Gateways A and C clear the call with a SIP BYE message.

Gateway A sends a <UsageIndication> message to the OSP server. In this example Gateway A’s message will include two complete <UsageIndication> components, one for the failed attempt and one for the successful call. The sub-elements for each will include the following:

| <UsageIndication> | usage information for the failed setup attempt |
| --- | --- |
| <Timestamp> | time of request |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number as returned in the authorization response, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway B, for example, gatewayB.itsp.fr |
| <FailureReason> | reason for failure of attempted setup, e.g. 422 |
| <UsageIndication> | usage information for the successful setup attempt |
| <Timestamp> | time of request |
| <Role> | for Gateway A, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example gatewayA.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number as returned in the authorization response, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | Unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Gateway C also sends a <UsageIndication> to the OSP server. That message would include the following elements:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | For Gateway C, destination |
| <TransactionId> | transaction ID assigned by OSP server and passed to Gateway C in authorization token |
| <CallId> | SIP Call-ID used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as presented in the INVITE message, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of Gateway A, for example [172.16.1.1] |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of Gateway C, for example, gatewayC.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Statistics> | statistical information for call |
| <LossSent> | loss information for packets sent by Gateway C |
| <Packets> | number of packets lost from Gateway C to Gateway A |
| <Fraction> | fraction (from 0 to 255) of packets lost from C to A |
| <LossReceived> | loss information for packets sent by Gateway A |
| <Packets> | number of packets lost from Gateway A to Gateway C |
| <Fraction> | fraction (from 0 to 255) of packets lost from A to C |
| <OneWayDelay> | one way delay measured from Gateway A to C |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |
| <RoundTripDelay> | round trip delay between Gateway A and C measured during call |
| <Minimum> | minimum measured value for delay, in seconds |
| <Mean> | sample mean of delay measurements, in seconds |
| <Variance> | sample variance of delay measurements, in squared seconds |
| <Samples> | number of sample measurements |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

# J.2	Prepaid Calling Card and Roaming User Support

As the following figure shows, the most general environment requires support from four different domains: the source and destination domains of the IP telephony gateways, the settlement service provider, and the end user billing domain. User billing may distinct from the source domain in the case, for example, or a roaming user.

The figure also shows the general operational procedure for authorization divided into seven discrete steps.

Figure J.13

The user accesses a gateway in the source domain. The gateway, perhaps using an IVR application, collects the user’s calling card number and personal identification number, in addition to the called number.

The source gateway forwards this information to the settlement provider in an OSP <AuthorizationRequest>.

The settlement provider, in addition to authenticating the source gateway, also authenticates the end user. That authentication procedure is outside the scope of OSP, but may, as the example shows, rely on another standard protocol such as RADIUS (RFC 2138).

The end user billing application authenticates and authorizes the user.

The settlement provider returns an <AuthorizationResponse> to the source gateway indicating acceptance of the end user and providing authorization tokens for the destination gateway.

The call proceeds normally with a Setup message from the source to the destination gateway.

The destination gateway completes the call to the called party.

These steps represent the beginning of a calling card transaction. Additional phases, including refreshing the user’s authorization and reporting usage information, can be found in the following section on implementation details.

## J.2.1	Call Routing and Authorization

The OSP <AuthorizationRequest> message shown in step 2 contains the following significant elements:

| <Timestamp> | Time of request |
| --- | --- |
| <CallId> | H.323 [13] Call Identifier to be used for the call |
| <SourceInfo type="e164"> | Calling party’s E.164 [12] number if available; otherwise a local E.164 [12] number controlled by the source gateway, e.g. 14048724887; this number shall be passed to the destination gateway(s) in Setup messages |
| <SourceAlternate type="transport"> | DNS name or IP address of source gateway, for example sourcegateway.carrier.com |
| <SourceAlternate type="subscriber"> | The user’s calling card and PIN; following conventions established by the Voice over IP Forum, these should be combined into a single character string, with the two components separated by the pound sign (#). For example, the calling card number 12345678, combined with the PIN 4444, should be represented as "12345678#4444". |
| <DestinationInfo type="e164"> | Called party’s E.164 [12] number, e.g. 33492944299 |
| <Service/> | Empty (for basic service) |
| <MaximumDestinations> | The maximum number of destinations, including alternatives, the source gateway will consider |

The OSP server replies with an <AuthorizationResponse> message as the figure indicates in step 5. The message indicates candidate destination gateways, in order of priority. In this example (which only shows a single destination gateway, the OSP <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | first destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of destination gateway, for example destgateway.itsp.fr |
| <Token> | authorization token to be passed to destination gateway |
| <ValidAfter> | time after which token for destination gateway is valid |
| <ValidUntil> | time until which token for destination gateway is valid |
| <UsageDetail> | how much service is authorized with destination gateway |
| <Service/> | empty (for basic service) |
| <Amount> | amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <CallId> | H.323 [13] Call Identifier to be used for the call to destination gateway |

## J.2.2	Reauthorization

In many calling card services, it may be necessary to refresh the authorization of a call that is already in progress. For example, some debit card applications will only authorize a limited amount of service at any given time; this can minimize the risk of fraudulent, simultaneous use of the debit card by multiple users. In such scenarios, and when the users wish to continue their conversation past the limited amount of time initially authorized, it will be necessary for the supporting devices to request additional authorization for the call. The following figure shows the message flow for this process. The six steps in the figure begin when the originating gateway recognizes that the currently authorized service limit is approaching.

Figure J.14

Source gateway recognizes that the duration currently authorized for the call is nearing its limit.

Source gateway sends an OSP <ReauthorizationRequest> to the OSP server.

The OSP server uses some other means (such as RADIUS, in the example) to authorize additional service for the user.

The OSP server confirms that additional service is authorized.

The OSP server returns a <ReauthorizationResponse> to the source gateway, granting the additional authorized service. The response tells the source gateway the new authorization limits explicitly, and it includes an updated authorization token.

The source gateway passes the new authorization token to the destination gateway. The exact method of transfer depends on the gateway implementations and the particular call signalling protocol; as an example, the source gateway may include the token in an H.323 [13] Facility message.

The OSP <ReauthorizationRequest> message shown in step 9 contains the following significant elements:

| <Timestamp> | Time of request |
| --- | --- |
| <Role> | for the source gateway, source |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | Calling party’s E.164 [12] number if available; otherwise a local E.164 [12] number controlled by the source gateway, e.g. 14048724887; this number shall be passed to the destination gateway(s) in Setup messages |
| <SourceAlternate type="transport"> | DNS name or IP address of source gateway, for example sourcegateway.carrier.com |
| <SourceAlternate type="subscriber"> | The user’s calling card and PIN; following conventions established by the Voice over IP Forum, these should be combined into a single character string, with the two components separated by the pound sign (#). For example, the calling card number 12345678, combined with the PIN 4444, should be represented as "12345678#4444". |
| <DestinationInfo type="e164"> | Called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of destination gateway, for example, destgateway.itsp.fr |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <UsageDetail> | usage information for the call so far |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used so far, e.g. 300 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |
| <Token> | authorization token to be passed to destination gateway |

The OSP server returns that information within an <ReauthorizationResponse> message, as the figure indicates in step 12. The message refreshes the authorization information for the call. In this example, the OSP <AuthorizationResponse> contains the following elements:

| <Timestamp> | time of response |
| --- | --- |
| <Status> | result of response, e.g. <Code>200</Code> |
| <TransactionId> | transaction identifier assigned by settlement provider |
| <Destination> | destination gateway to try for call |
| <DestinationSignalAddress type="transport"> | DNS name or IP address of destination gateway, for example destgateway.itsp.fr |
| <Token> | updated authorization token to be passed to destination gateway |
| <ValidAfter> | time after which token for destination gateway is valid |
| <ValidUntil> | time until which token for destination gateway is valid |
| <UsageDetail> | how much (cumulative) service is authorized with destination gateway |
| <Service/> | empty (for basic service) |
| <Amount> | (cumulative) amount of authorized service, e.g. 3600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

Once the call has ended, both gateways report usage details to an OSP server. As the following figure indicates, those reports are conveyed in OSP <UsageIndication> messages.

Figure J.15

The steps shown in the figure are straightforward.

The gateways clear the call by exchanging an H.225.0 [9] Release Complete message.

The source gateway sends a <UsageIndication> message to the OSP server, reporting its usage details for the call.

The OSP server acknowledges receipt with a <UsageConfirmation> message.

The destination gateway also reports its usage details with a <UsageIndication> message.

The server acknowledges this message as well with a <UsageConfirmation>.

The <UsageIndication> messages from both gateways will be substantially the same. As an example, here are the significant fields of the source gateway’s message:

| <Timestamp> | time of request |
| --- | --- |
| <Role> | for source gateway, source |
| <TransactionId> | transaction ID assigned by OSP server in authorization response |
| <CallId> | H.323 [13] Call Identifier used for the call |
| <SourceInfo type="e164"> | calling party’s E.164 [12] number as returned in the authorization response, e.g. 14048724887 |
| <SourceAlternate type="transport"> | DNS name or IP address of source gateway, for example sourcegateway.carrier.com |
| <DestinationInfo type="e164"> | called party’s E.164 [12] number, e.g. 33492944299 |
| <DestinationAlternate type="transport"> | DNS name or IP address of destination gateway, for example, destgateway.isp.fr |
| <UsageDetail> | usage information for the call |
| <Service/> | empty (for basic service) |
| <Amount> | amount of service used, e.g. 600 |
| <Increment> | increment of service measurement, e.g. 1 |
| <Unit> | unit of service measurement, e.g. s for seconds |

The OSP server responds with a <UsageConfirmation> message. If it has accepted the usage report, that message will contain a successful <Status> element (e.g. <Code>200</Code>).

Annex K (informative):	History of OSP codes

This annex provides a comparison of selected HTTP [85], SIP [86] and OSP codes. The logic for each recommended OSP code change or addition is documented in this annex.

## K.1	Code 2XX

HTTP /1.1 RFC 2616 Successful 2XX This class of status code indicates that the client's request was successfully received, understood, and accepted.

SIP RFC 3261 (June 2002) Successful 2XX The request was successful

OSP 101 321 VK.1.1 (2000-08) 2xx = operation successful

OSP 101 321 V3 recommendation No change.

## K.2	Code 200

HTTP /1.1 RFC 2616 200 OK The request has succeeded. The information returned with the response is dependent on the method used in the request.

SIP RFC 3261 (June 2002) 200 OK The request has succeeded. The information returned with the response depends on the method used in the request.

OSP 101 321 V2.1.1 (2000-08) 200 = success (no other information)

OSP 101 321 V3 No change.

## K.3	Code 4XX

HTTP /1.1 RFC 2616 Client Error 4xx The 4xx class of status code is intended for cases in which the client seems to have erred. Except when responding to a HEAD request, the server SHOULD include an entity containing an explanation of the error situation, and whether it is a temporary or permanent condition. These status codes are applicable to any request method. User agents SHOULD display any included entity to the user. If the client is sending data, a server implementation using TCP SHOULD be careful to ensure that the client acknowledges receipt of the packet(s) containing the response, before the server closes the input connection. If the client continues sending data to the server after the close, the server's TCP stack will send a reset packet to the client, which may erase the client's unacknowledged input buffers before they can be read and interpreted by the HTTP application.

SIP RFC 3261 (June 2002) Request Failure 4xx 4xx responses are definite failure responses from a particular server. The client SHOULD NOT retry the same request without modification (for example, adding appropriate authorization). However, the same request to a different server might be successful.

OSP 101 321 V2.1.1 (2000-08) 4xx = client error

OSP 101 321 V3 recommendation Replace 4xx = client error with: “4xx = request failure” Reason for recommendation. 4xx codes should be designated in general as a Request Failure, not a client failure. For example, if an OSP client sends an AuthorizationRequest and the route that is blocked, the AuthorizationResponse is not a successful operation for the client, nor a failure of the client. It is a failed request.

## K.4	Code 400

HTTP /1.1 RFC 2616 400 Bad Request The request could not be understood by the server due to malformed syntax. The client SHOULD NOT repeat the request without modifications.

SIP RFC 3261 (June 2002) 400 Bad Request The request could not be understood due to malformed syntax. The Reason-Phrase SHOULD identify the syntax problem in more detail, for example, missing Call-ID header field”.

OSP 101 321 V2.1.1 (2000-08) 400 = bad request (generic problem interpreting message)

OSP 101 321 V3 recommendation No change

## K.5	Code 401

HTTP /1.1 RFC 2616 401 Unauthorized The request requires user authentication. The response MUST include a WWW-Authenticate header field (section 14.47) containing a challenge applicable to the requested resource. The client MAY repeat the request with a suitable Authorization header field (section 14.8). If the request already included Authorization credentials, then the 401 response indicates that authorization has been refused for those credentials. If the 401 response contains the same challenge as the prior response, and the user agent has already attempted authentication at least once, then the user SHOULD be presented the entity that was given in the response, since that entity might include relevant diagnostic information. HTTP access authentication is explained in "HTTP Authentication: Basic and Digest Access Authentication" [43].

SIP RFC 3261 (June 2002) 401 Unauthorized The request requires user authentication. This response is issued by UASs and registrars, while 407 (Proxy Authentication Required) is used by proxy server.

OSP 101 321 V2.1.1 (2000-08) 401 = unauthorized

OSP 101 321 V3 recommendation Expand explanation from: 401 = unauthorized to 401 = unauthorized, user authentication failed

## K.6	Code 402

HTTP /1.1 RFC 2616 402 Payment Required This code is reserved for future use.

SIP RFC 3261 (June 2002) 402 Payment Required This code is reserved for future use.

OSP 101 321 V2.1.1 (2000-08) 402 = authentication unsuccessful

OSP 101 321 V3 recommendation 402 = payment required Reason for recommended change: Make OSP status code the same as HTTP and SIP response codes. Also this code is more specific and indicates the client request failed because payment is required for authorization.

## K.7	Code 403

HTTP /1.1 RFC 2616 403 Forbidden The server understood the request, but is refusing to fulfill it. Authorization will not help and the request SHOULD NOT be repeated. If the request method was not HEAD and the server wishes to make public why the request has not been fulfilled, it SHOULD describe the reason for the refusal in the entity. If the server does not wish to make this information available to the client, the status code 404 (Not Found) can be used instead.

SIP RFC 3261 (June 2002) 403 Forbidden The server understood the request, but is refusing to fulfill it. Authorization will not help, and the request should not be repeated.

OSP 101 321 V2.1.1 (2000-08) 403 = call authorization unsuccessful

OSP 101 321 V3 recommendation Change “403 = call authorization unsuccessful” to 403 = forbidden (route blocked) Reason for recommended change: Make OSP status code the same as HTTP and SIP response codes. Also this code definition is more specific. Unsuccessful call authorization may be due to no route or a blocked route.

## K.8	Code 404

HTTP /1.1 RFC 2616 404 Not Found The server has not found anything matching the Request-URI. No indication is given of whether the condition is temporary or permanent. The 410 (Gone) status code SHOULD be used if the server knows, through some internally configurable mechanism, that an old resource is permanently unavailable and has no forwarding address. This status code is commonly used when the server does not wish to reveal exactly why the request has been refused, or when no other response is applicable.

SIP RFC 3261 (June 2002) 404 Not Found The server has definitive information that the user does not exist at the domain specified in the Request-URI. This status is also returned if the domain in the Request-URI does not match any of the domains handled by the recipient of the request.

OSP 101 321 V2.1.1 (2000-08) 404 = route authorization unsuccessful

OSP 101 321 V3 recommendation Change “404 = route authorization unsuccessful” to 404 = not found (no route found to destination) Reason for recommended change: Make OSP status code the same as HTTP and SIP response codes. Also this code is more specific since unsuccessful route authorization may be due to no route as well as a blocked route. A more specific “no route found” case is identified with the proposed OSP code 447 which indicates that destination is known, but that it is not available.

## K.9	Code 405

HTTP /1.1 RFC 2616 405 Method Not Allowed The method specified in the Request-Line is not allowed for the resource identified by the Request-URI. The response MUST include an Allow header containing a list of valid methods for the requested resource.

SIP RFC 3261 (June 2002) 405 Method Not Allowed The method specified in the Request-Line is understood, but not allowed for the address identified by the Request-URI.

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 405 = source may not originate call to destination Reason for recommended change: Make OSP status code the same as HTTP and SIP response codes. Also this code provides specific information of why the request failed.

## K.10	Code 428

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 428 = SourceInfo invalid or missing Reason for recommendation: Invalid or missing SourceInfo in a client message will cause the client request to fail, therefore this is logically a 4xx OSP code. The second two digits, 28, are taken from the TC Code 0028 which indicates “invalid number format (incomplete address)”.

## K.11	Code 441

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 441 = temporary failure Reason for recommendation: This 4xx code indicates there is some temporary failure, not related to the server, that cause the request to fail. (A client request failure due to server error would be indicated with a 5xx code.) The second two digits, 41, are taken from the TC Code 0041 which indicates “temporary failure”.

## K.12	Code 447

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 447 = resource unavailable (route to destination may not terminate) Reason for recommendation: This code provides more specific information to the network operator why the call failed than the more general 404 “route not found” code. This code indicates that the route is known, but not available. The second two digits, 47, are taken from the TC Code 0047 which indicates “resource unavailable, unspecified”.

## K.13	Code 449

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 449 = quality of service unavailable Reason for recommendation: Addresses client request failures which are not successful due to unavailable quality of service. The second two digits, 49, are taken from the TC Code 0049 which indicates “quality of service unavailable”.

## K.14	Code 450

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 450 = requested facility is not subscribed Reason for recommendation: This OSP code is required when the client requests authorization to specific destination clients. (This use case is sometimes called “suggested routing” since the client is suggesting routes to the server.) If the destination clients suggested by the source client are unknown to the OSP Server, the server cannot authorize the request. This code indicates the request failed because the requested facility is not subscribed with the OSP Server. The second two digits, 50, are taken from the TC Code 0050 which indicates “requested facility is not subscribed”.

## K.15	Code 463

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 463 = Service or option (as indicated in the Service element) not available. Reason for recommendation: The code indicates that the service requested is not available. The second two digits, 63, are taken from the TC Code 0063 which indicates “service or option not available, unspecified”.

## K.16	Code 481

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) 481 Call/Transaction Does Not Exist This status indicates that the UAS received a request that does not match any existing dialog or transaction.

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 481 = call reference ID is invalid or missing Reason for recommendation: The code indicates that the request failed due to invalid or missing call reference. The second two digits, 81, are taken from the TC Code 0081 which indicates “invalid call reference value”.

## K.17	Code 488

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 488 = incompatible destination Reason for recommendation: The code indicates that the request failed because the source and destination are incompatible. This code will be useful for operators who face inter-working issues between H.323 and SIP networks. The second two digits, 88, are taken from the TC Code 0081 which indicates “incompatible destination”.

## K.18	Code 495

HTTP /1.1 RFC 2616 None

SIP RFC 3261 (June 2002) None

OSP 101 321 V2.1.1 (2000-08) None

OSP 101 321 V3 recommendation 495 = invalid message Reason for recommendation: The code indicates that the request was received and understood, but is not valid. This case is different from cases 400 and 403. The second two digits, 95, are taken from the TC Code 0095 which indicates “invalid message”.

# Bibliography

The following material, though not specifically referenced in the body of the present document (or not publicly available), gives supporting information.

-	RFC 2138 (1997): Remote Authentication Dial In User Service (RADIUS). C. Rigney, A. Rubens, W. Simpson, S. Willens.

# History

| Document history |  |  |
| --- | --- | --- |
| V1.4.2 | December 1998 | Publication |
| V2.1.1 | August 2000 | Publication |
| V2.2.2 | November 2000 | TIPHON21: Edits per 21TD070, 21TD088, and 21TD089 |
| V3.0.1 | April 2002 | TIPHON27: Edits in drafting session |