SIP Working Group                                         H. Schulzrinne

Internet-Draft                                               Columbia U.

Expires: August 21st, 2005                                       J. Polk

Cisco

February 21st, 2005

Communications Resource Priority for the Session Initiation Protocol

(SIP)

draft-ietf-sip-resource-priority-06

Status of this Memo

By submitting this Internet-Draft, I certify that any applicable

patent or other IPR claims of which I am aware have been disclosed,

and any of which I become aware will be disclosed, in accordance with

RFC 3668.

Internet-Drafts are working documents of the Internet Engineering

Task Force (IETF), its areas, and its working groups. Note that other

groups may also distribute working documents as Internet-Drafts.

Internet-Drafts are draft documents valid for a maximum of six months

and may be updated, replaced, or obsoleted by other documents at any

time. It is inappropriate to use Internet-Drafts as reference

material or to cite them other than as "work in progress."

The list of current Internet-Drafts can be accessed at http://

www.ietf.org/ietf/1id-abstracts.txt.

The list of Internet-Draft Shadow Directories can be accessed at

http://www.ietf.org/shadow.html.

This Internet-Draft will expire on August 21st, 2005.

Copyright Notice

Copyright (C) The Internet Society (2005). All Rights Reserved.

Abstract

This document defines two new Session Initiation Protocol (SIP) header fields for communicating

resource priority, namely "Resource-Priority" and "Accept-Resource-

Priority".  The "Resource-Priority" header field can influence the

behavior of SIP user agents, such as telephone gateways and IP

telephones, and SIP proxies.  It does

not directly influence the forwarding behavior of IP routers.

Table of Contents

1.  Introduction . . . . . . . . . . . . . . . . . . . . . . . . .  3

2.  Terminology  . . . . . . . . . . . . . . . . . . . . . . . . .  5

3.  The Resource-Priority and Accept-Resource-Priority SIP

Header Fields  . . . . . . . . . . . . . . . . . . . . . . . .  5

3.1   The 'Resource-Priority' Header Field . . . . . . . . . . .  5

3.2   The 'Accept-Resource-Priority' Header Field  . . . . . . .  6

3.3   Usage of the 'Resource-Priority' and

'Accept-Resource-Priority' Header Fields . . . . . . . . .  7

3.4   The 'resource-priority' Option Tag . . . . . . . . . . . .  8

4.  Behavior of SIP Elements that Receive Prioritized Requests . .  8

4.1   General Rules  . . . . . . . . . . . . . . . . . . . . . .  8

4.1.1 Policy Guidelines Involving Preemption Policy  . . . . .  9

4.2   Rejection Messages . . . . . . . . . . . . . . . . . . . .  9

4.3   Error Conditions . . . . . . . . . . . . . . . . . . . . . 10

4.3.1   Known Namespace and Priority Value . . . . . . . . . . 10

4.3.2   Handling Unknown Namespaces and Priority Values  . . . 11

4.4   User Agent Client Behavior . . . . . . . . . . . . . . . . 14

4.5   User Agent Server Behavior . . . . . . . . . . . . . . . . 14

4.5.1 User Agent Servers and Preemption Policy . . . . . . . . 15

4.6   Proxy Behavior . . . . . . . . . . . . . . . . . . . . . . 15

5.  Third-Party Authentication . . . . . . . . . . . . . . . . . . 16

6.  Backwards Compatibility  . . . . . . . . . . . . . . . . . . . 16

7.  Namespace Description  . . . . . . . . . . . . . . . . . . . . 17

7.1 Multiple Namespaces in a Message . . . . . . . . . . . . . . 19

7.1.1 The "DSN" Namespace  . . . .   . . . . . . . . . . . . . 16

7.1.2 The "DRSN" Namespace . . . .   . . . . . . . . . . . . . 16

7.1.3 The "Q735" Namespace . . . .   . . . . . . . . . . . . . 16

7.1.4 The "ETS" Namespace  . . . .   . . . . . . . . . . . . . 16

7.1.5 The "WPS" Namespace  . . . .   . . . . . . . . . . . . . 16

7.2 . . . . . . . . . . . . . . 19

7.3   . . . . . . . . . . . . . . 19

8.  Examples . . . . . . . . . . . . . . . . . . . . . . . . . . . 21

8.1   Simple Call  . . . . . . . . . . . . . . . . . . . . . . . 21

8.2   Receiver Does Not Understand Namespace . . . . . . . . . . 22

9.  Security Considerations  . . . . . . . . . . . . . . . . . . . 24

9.1   Authentication and Authorization . . . . . . . . . . . . . 25

9.2   Confidentiality and Integrity  . . . . . . . . . . . . . . 26

9.3   Anonymity  . . . . . . . . . . . . . . . . . . . . . . . . 27

9.4   Denial-of-Service Attacks  . . . . . . . . . . . . . . . . 27

10. IANA Considerations  . . . . . . . . . . . . . . . . . . . . . 27

10.1  IANA Registration of 'Resource-Priority' and

'Accept-Resource-Priority' Header Fields . . . . . . . . . 27

10.2  IANA Registration for Option Tag resource-priority . . . . 28

10.3  IANA Registration for Response Code 417  . . . . . . . . . 28

10.4  IANA Namespace Registrations . . . . . . . . . . . . . . . 28

10.5  IANA Priority-Value Registrations  . . . . . . . . . . . . 29

11. Acknowledgments  . . . . . . . . . . . . . . . . . . . . . . . 29

12. References . . . . . . . . . . . . . . . . . . . . . . . . . . 30

12.1  Normative References . . . . . . . . . . . . . . . . . . . 30

12.2  Informative References . . . . . . . . . . . . . . . . . . 30

Authors' Addresses . . . . . . . . . . . . . . . . . . . . . . 32

Intellectual Property and Copyright Statements . . . . . . . . 32

1.  Introduction

During emergencies, communications resources including telephone

circuits, IP bandwidth and gateways between the circuit-switched and

IP networks may become congested.  Congestion can occur due to heavy

usage, loss of resources caused by the natural or man-made disaster

and attacks on the network during man-made emergencies.  This

congestion may make it difficult for persons charged with emergency

assistance, recovery or law enforcement to coordinate their efforts.

As IP networks become part of converged or hybrid networks along with

public and private circuit-switched (telephone) networks, it becomes

necessary to ensure that these networks can assist during such

emergencies.

Also, users may want to interrupt their lower-priority communications

activities and dedicate their end system resources to the

high-priority communications attempt if a high-priority

communications request arrives at their end system.

There are many IP-based services that can assist during emergencies.

This memo only covers real-time communications applications involving

the Session Initiation Protocol (SIP) [RFC3261], including

voice-over-IP, multimedia conferencing, instant messaging and

presence.

SIP applications may involve at least five different resources that

may become scarce and congested during emergencies.  These resources

include gateway resources, circuit-switched network resources, IP

network resources, receiving end system resources and SIP proxy

resources.  IP network resources are beyond the scope of SIP

signaling and are therefore not considered here.

In order to improve emergency response, it may become necessary to

prioritize access to SIP-signaled resources during periods of

emergency-induced resource scarcity.  We call this "resource

prioritization". The mechanism itself may well be in place at all times, but only

materially affect call handling during times of resource scarcity.

Currently, SIP does not include a mechanism that allows a request

originator to indicate to a SIP element that it wishes the request to

invoke such resource prioritization. To address this need, this

document adds a SIP protocol element that labels certain SIP

requests.

This document defines (Section 3) a new SIP [RFC3261] header field

for communications resource priority, called 'Resource-Priority' This

header field MAY be used by SIP user agents, including General

Switched Telephone Network (GSTN) gateways and terminals, and SIP

proxy servers to influence their treatment of SIP requests, including

the priority afforded to GSTN calls.  For GSTN gateways, the behavior

translates into analogous schemes in the GSTN, for example the ITU

Recommendation Q.735.3 [Q.735.3] prioritization mechanism, in both

the GSTN-to-IP and IP-to-GSTN directions.  ITU Recommendation I.255.3

[I.255.3] is another example.

The 'Resource-Priority' header field may be used in several

situations. A SIP request with such an indication can be treated

differently in these situations:

1.  The request can be given elevated priority for access to GSTN

gateway resources such as trunk circuits.

2.  The request can interrupt lower-priority requests at a user

terminal, such as an IP phone.

3.  The request can carry information from one multi-level priority

domain in the telephone network, e.g., using the facilities of

Q.735.3 [Q.735.3], to another, without the SIP proxies themselves

inspecting or modifying the header field.

4.  In SIP proxies and back-to-back user agents, requests of higher

priorities may delay, reject or error existing signaling requests

of lower priorities.

This header field is related to, but differs in semantics from, the

'Priority' header field (RFC 3261 [RFC3261], Section 20.26).  The

'Priority' header field describes the importance that the SIP request

should have to the receiving human or its agent.  For example, that

header may be factored into decisions about call routing to mobile

devices and assistants and call acceptance when the call destination

is busy.  The 'Priority' header field does not affect the usage of

GSTN gateway or proxy resources, for example.  In addition, any User

Agent Client (UAC) can assert any 'Priority' value, while access to

resource priority values are subject to authorization.

While the 'Resource-Priority' header does not directly influence the

forwarding behavior of IP routers or the use of communications

resources such as packet forwarding priority, procedures for using

this header to cause such influence may be defined in other

documents.

Existing implementations of RFC 3261 that do not participate in the

resource priority mechanism follow the normal rules of RFC 3261,

Section 8.2.2:  "If a UAS does not understand a header field in a

request (that is, the header field is not defined in this

specification or in any supported extension), the server MUST ignore

that header field and continue processing the message." Thus, the use

of this mechanism is wholly invisible to existing implementations

unless the request includes the Require header field with the

resource-priority option tag.

The mechanism described here can be used for emergency preparedness

in emergency telecommunications systems, but is only a small part of

an emergency preparedness network and is not restricted to such use.

The mechanism aims to satisfy the requirements in [RFC3487].  It is

structured so that it works in all SIP and Real-Time Transport

Protocol (RTP) [RFC3550] transparent networks defined in [RFC3487].

In such networks, all network elements and SIP proxies let valid SIP

requests pass through unchanged.  This is important since it is

likely that this mechanism will often be deployed in networks where

the edge networks are unaware of the resource priority mechanism and

provide no special privileges to such requests.  The request then

reaches a GSTN gateway or set of SIP elements that are aware of the

mechanism.

For conciseness, we refer to SIP proxies and user agents (UAs) that

act on the 'Resource-Priority' header field as RP actors.

The remainder of this document is structured as follows. After defining terminology in Section 2, we define the syntax for the two new SIP header fields in Section 3 and then describe protocol behavior in Section 4. The two principal mechanisms for differentiated treatment of SIP requests, namely preemption and queueing, are described in Section 4.2. Rejection messages are covered in Section 4.3, error conditions in Section 4.4. Section 4.4 through Section 4.6 detail the behavior of specific SIP elements. Third-party authentication is briefly summarized in Section 5.  Section 6 describes how this feature affects existing

systems that do not support it. Sections 7 and 8 delve into namespace issue, namely their general properties, how to deal with multiple namespaces on the same SIP element and in the same SIP message. Section 9 enumerates five namespaces that are registered through this document. Protocol examples are given in Section 10. Security issues are considered in Section 11, but this document does not define new security mechanisms.

2.  Terminology

In this document, the key words "MUST", "MUST NOT", "REQUIRED",

"SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY",

and "OPTIONAL" are to be interpreted as described in BCP 14, RFC 2119

[RFC2119] and indicate requirement levels for compliant

implementations.

3.  The Resource-Priority and Accept-Resource-Priority SIP Header Fields

This document defines the 'Resource-Priority' and

'Accept-Resource-Priority' SIP header field syntax. Behavior is described in Section 4.

3.1  The 'Resource-Priority' Header Field

In a request, the 'Resource-Priority' header field indicates that a SIP request desires prioritized access to resources, as described in the introduction.  In

responses, the 'Resource-Priority' header fields indicates the actual

resource priority that was granted to the request.  While it is

likely that those responses contain the same 'Resource-Priority' header

field value as the requests, local policy MAY call for the UAS to

insert no header field or a different value.

There is no protocol requirement that all requests within a SIP

dialog or session use the 'Resource-Priority' header field.  Local administrative policy MAY mandate the inclusion of the ‘Resource-Priority’ header field in all requests. Implementations of this specification MUST allow inclusion to be either by explicit user request or automatic.

The syntax of the 'Resource-Priority' header field is described

below.  The "token-nodot" production is copied from [RFC3265].

Resource-Priority  = "Resource-Priority" HCOLON

resource-value *(COMMA resource-value)

resource-value     = namespace "." r-priority

namespace          = token-nodot

r-priority         = token-nodot

token-nodot        = 1*( alphanum / "-"  / "!" / "%" / "*"

/ "_" / "+" / "`" / "'" / "~" )

An example 'Resource-Priority' header field is shown below:

Resource-Priority: dsn.flash, wps.3

The 'resource-value' parameter in the 'Resource-Priority' header

indicates the resource priority desired by the request originator.

Each resource value is formatted as 'namespace' '.' 'priority value'.

The value is drawn from the namespace identified by the 'namespace'

token.  Namespaces and priorities are case-insensitive ASCII tokens that do not contain periods. Thus, ‘dsn.flash’ and ‘DSN.Flash’, for example, are equivalent.

Each

namespace has at least one priority value.  Namespaces and priority

values within each namespace MUST be registered with IANA (Section

10).  This document registers five namespaces in Section XX.

Since a request may traverse multiple administrative domains with

multiple different namespaces, it is necessary to be able to

enumerate several different namespaces within the same message.

However, a particular namespace MUST NOT appear more than once

in the same SIP message. These may be expressed equivalently as either comma-separated lists within a single header field, as multiple header fields or some combination. The ordering of r-values within the header field has no significance. Thus, for example, the following three header snippets are equivalent:

Resource-Priority: dsn.flash, wps.3

Resource-Priority: wps.3, dsn.flash

Resource-Priority: wps.3

Resource-Priority: dsn.flash

3.2  The 'Accept-Resource-Priority' Header Field

The 'Accept-Resource-Priority' response header field enumerates the

resource values (r-values) a SIP user agent server is willing to accept.  The

syntax of the 'Accept-Resource-Priority' header field is as follows:

Accept-Resource-Priority = "Accept-Resource-Priority" HCOLON

[r-value *(COMMA r-value)]

An example is given below:

Accept-Resource-Priority: dsn.flash-override,

dsn.flash, dsn.immediate, dsn.priority, dsn.routine

Inclusion of this header fields is OPTIONAL and some administrative domains MAY choose to disable its inclusion in responses. However, this behavior is NOT RECOMMENDED, as this header field aids in troubleshooting.

3.3  Usage of the 'Resource-Priority' and 'Accept-Resource-Priority'

Header Fields

The following table extends the values in Table 2 of RFC3261

[RFC3261].  (The PRACK method, labeled as PRA, is defined in

[RFC3262], the SUBSCRIBE (labeled SUB) and NOTIFY (labeled NOT)

methods in [RFC3265], the UPDATE (UPD) method in [RFC3311], the

MESSAGE (MSG) method in [RFC3428], the REFER (REF) method in

[RFC3515], the INFO (INF) method in [RFC2976], and the PUBLISH (PUB)

method is in [RFC3903].)

Header field             where proxy INV ACK CAN BYE REG OPT PRA

----------------------------------------------------------------

Resource-Priority        R     amdr   o   o   o   o   o   o   o

Resource-Priority        r     amdr   o   o   o   o   o   o   o

Resource-Priority        200   amdr   o   -   o   o   o   o   o

Accept-Resource-Priority 200   amdr   o   -   o   o   o   o   o

Accept-Resource-Priority 417   amdr   o   -   o   o   o   o   o

Accept-Resource-Priority 420   amdr   o   -   o   o   o   o   o

Header field             where proxy SUB NOT UPD MSG REF INF PUB

----------------------------------------------------------------

Resource-Priority        R     amdr   o   o   o   o   o   o   o

Resource-Priority        r     amdr   o   o   o   o   o   o   o

Resource-Priority        200   amdr   o   o   o   o   o   o   o

Accept-Resource-Priority 200   amdr   o   o   o   o   o   o   o

Accept-Resource-Priority 417   amdr   o   o   o   o   o   o   o

Accept-Resource-Priority 420   amdr   o   o   o   o   o   o   o

Other request methods may define their own handling rules; unless

otherwise specified, recipients MAY ignore these header fields.

The 'Accept-Resource-Priority' header field SHOULD be returned in a 420 (Bad Extension) response if an element implements the

resource priority mechanism, but but does not implement the particular namespace or priority value contained in the request.

3.4  The 'resource-priority' Option Tag

This document also defines the "resource-priority" option tag. The

behavior is described in Section 4.2. and the IANA registration is

in Section 10.2.

4.  Behavior of SIP Elements that Receive Prioritized Requests

All SIP user agents and proxy servers that support this

specification share certain common behavior, which we describe

below in Section 4.1. The behavior when encountering a ‘resource-priority’ option tag in a ‘Require’ header field is describe in Section 4.2. Section 4.3 describes the treatment of OPTIONS requests. The two fundamental resource contention resolution mechanisms, preemption and queueing, are described in Section 4.4. Rejection Behavior specific to user agent clients is covered in

section 4.4, for user agent servers in Section 4.5, while Section

4.6 deals with proxy behavior.

4.1  General Rules

The Resource-Priority header field is potentially applicable to all

SIP request messages. At a minimum, implementations of the following

request types MUST support the Resource-Priority header to be in

compliance with this specification:

INVITE [RFC3261]

ACK [RFC3261]

PRACK [RFC3262]

UPDATE [RFC3311]

REFER [RFC3515]

and SHOULD support the Resource-Priority header in the following

request types (if the SIP element does) to be in compliance with

this specification:

MESSAGE [RFC3428]

SUBSCRIBE [RFC3265]

NOTIFY [RFC3265]

Note that this does not imply that all implementations have to support every request method listed.

If a SIP element receives the Resource-Priority header field in a request

message other than what is listed above, the header MAY be ignored,

according to the rules of [RFC3261].

4.2 Usage of Require Header Fields

Following standard SIP behavior, if a SIP request contains the Require header field with the ‘resource-priority’ option tag, a SIP element MUST respond with a 420 (Bad Extension) if it does not support the SIP extensions described in this document. It then lists ‘resource-priority’ in the ‘Unsupported’ header field included in the response.

The use of the 'resource-priority' option tag in 'Proxy-Require'

header field is NOT RECOMMENDED.

4.3 OPTIONS

An OPTIONS request can be used to determine if an element supports

the resource priority mechanism.  RP actors MUST implement a capability to return a

'Accept-Resource-Priority' header field in the OPTIONS response

enumerating all valid resource values.  An RP actor MAY be configured to not return such values or only return them to authorized requestors.

According to RFC 3261, proxies reached with a Max-Forwards

value of zero answer the OPTIONS request, allowing a UAC to discover

the capabilities of both proxy and user agent servers.

4.4 Alternatives for Preferential Treatment of Requests

SIP elements may use the resource priority mechanism to modify a variety of behavior such as routing requests, authentication requirements or logging. However, the primary behavior of interest is likely to be some form of preferential treatment of the request itself or of the session created by the request. We use the terms session and call interchangeably in this document, both implying a continuous data stream between two or more parties. Sessions are established by SIP dialogs.

Below, we define two common policies, namely preemption and priority queueing. Preemption applies only to sessions created by SIP requests, while both sessions and request handling can be subject to priority queueing. Both policies can sometimes be combined in the same element, although none of the namespaces described in this document do this. Policies can be defined for each namespace or, in some cases, can be specific to an administrative domain.

Naturally, only SIP elements that understand this mechanism and the namespace and resource value perform these policies. Section 4.5 discusses what happens if an RP actor does not understand namespaces or priority values contained in a request.

4.4.1 Preemption Policy

An RP actor following a preemption policy may disrupt an existing session to make room for a higher-priority incoming session. Since sessions may require different amounts of bandwidth or number of circuits, a single higher priority session may displace more than one lower-priority session. As noted above, the processing of SIP requests itself is not preempted. Thus, since proxies do not manage sessions, they do not perform preemption.

[I-D.ietf-sipping-reason-header-for-preemption] provides more details

and examples of this behavior.

UAS behavior for preemption is discussed in Section XX.

4.4.2 Priority Queueing Policy

In a priority queueing policy, requests that find no available resources are queued on a first-come, first-served basis to the queue assigned to the priority value. Each priority value may have its own queue or several priority values may share a single queue. If a resource becomes available, a request from the queue with the highest priority is served. Each queue can hold a finite number of pending requests. If the queue for a newly arriving request is full, the request is rejected immediately. In addition, a priority queueing policy MAY impose a residence time limit. If a request exceeds a specified waiting time, the RP actor then rejects the request and removes it from the queue.

In addition, an RP actor MAY impose a global queue size limit summed across all queues and drop waiting lower-priority requests. This does not imply preemption since the session has not been established yet.

Often, normal, non-prioritized requests will not be queued, i.e., the queue size for such requests is zero.

Details are described in Section 4.X below.

4.5 Error Conditions

A request containing a resource priority indication can fail for four reasons: the RP actor does not understand the name space or priority, the requestor is not authenticated, an authenticated requestor is not authorized to make such a request or there are insufficient resources for an authorized request. We treat these error cases in the order that they typically arise in the processing of requests with Resource-Priority headers. However, this order is not mandated. For example, an RP actor that knows that a particular resource value cannot be served or queued MAY, as a matter of local policy, forego authorization since it would only add processing load without changing the outcome.

4.5.1 Unknown Namespace or Priority Value

If an RP actor does not understand any of the resource values in the request, there are two possible outcomes:

(1) it can treat the request as if it contained no ‘Resource-Priority’ header field and treat it with default priority;

(2) it can reject the request with a 417 (Unknown Resource-Priority) response code.

The second behavior SHOULD only be invoked if the request was marked with a ‘Require: resource-priority’ header field.

Note that making case (1) the default is necessary since otherwise there would be no way to successfully complete any calls in the case where a proxy on the way to the UAS shares no common namespaces with the UAC, but the UAC and UAS do have such a namespace in common.

In general, as noted, a SIP request can contain more than one Resource-Priority header. This is necessary if a request needs to traverse different administrative domains, each with their own set of valid resource values. For example, the ETS namespace might be enabled for United States government networks that also support the DSN and/or DRSN namespaces for most individuals in those domains.

If one resource value is understood, the resource values that are not understood MUST NOT be modified, deleted or cause a rejection of the message.

A 417 (Unknown Resource-Priority) response MAY, according to local policy, include an ‘Accept-Resource-Priority’ header field enumerating the acceptable resource values.

4.5.2 Authentication Failure

If the request is not authenticated, a 401 (Unauthorized) or 407 (Proxy Authentication Required) response is returned to allow the requestor to insert appropriate credentials.

4.5.2 Authorization Failure

If the RP actor receives an authenticated request with a

namespace and priority value it recognizes, but the originator is

not authorized for that level of service, the element MUST return

a 403 (Forbidden) response.

4.5.3 Insufficient Resources

If an RP actor receives an authorized request, has insufficient resources and the request neither preempts another request nor is queued, the response is the same as stated in Section 13.3.1.3 of [RFC3261]. A request can fail either because the RP actor has insufficient resources to handle the SIP request or the requested session.

If the request fails since the RP actor cannot handle the signaling load, 503 (Service Unavailable) is the appropriate response.

For the latter case, a 488 (Not Acceptable Here) response indicates that the RP actor is rejecting the request for reasons of media path availability, such as insufficient gateway resources. In that case, [RFC3261] advises that a 488 response SHOULD include a ‘Warning’ header field with a reason for the rejection, with warning code 370 (Insufficient Bandwidth) typical.

If the originator did not include a Resource-Priority header field, these error responses will provide the necessary indication to do so. (In some administrative domains, requests with normal priority will not include the Resource-Priority header field.) In addition, if authorized, the requestor may also attempt to use a higher resource priority value.

Note that 486 (Busy here) or 600 (Busy Everywhere) responses are not appropriate in this case, as they refer to the unavailability of the human callee, not scarce signaling or bandwidth resources. (Thus, these responses are sometimes associated with insufficiently high priority in the ‘Priority’ header field, not the ‘Resource-Priority’ header field.)

4.6  User Agent Client Behavior

SIP UACs supporting this specification MUST be able to generate the

'Resource-Priority' header field for requests that require elevated

resource access priority.  As stated previously, the UAC SHOULD be

able to generate more than one resource value in a single SIP request.

Upon a 417 (Unknown Resource-Priority) response, the

UAC MAY attempt a subsequent request with the same or different resource value. If available, it SHOULD choose authorized resource values from the set of values returned in the 'Accept-Resource-Priority' header field.

4.6.1 User Agent Client Behavior with a Preemption Policy

A UAC in an administrative domain that employs preemption

MUST be prepared to receive a BYE request with a Reason header

explaining why the session was terminated, as discussed in [I-D.ietf-sipping-reason-header-for-preemption].

4.6.2 User Agent Client Behavior with a Queueing Policy

By standard SIP protocol rules, a UAC MUST be epared to receive a 182 (Queued) response from an RP actor that

is currently at capacity, but has put the original request into a

queue. A UAC MAY indicate this queued status to the user by some audio or visual indication to prevent the user from interpreting the call as having failed.

4.5  User Agent Server Behavior

The precise effect of the 'Resource-Priority' indication depends on

the type of UAS, the namespace and local policy.

4.5.1 User Agent Servers and Preemption Policy

A UAS compliant with this specification MUST terminate a session

established with a valid namespace and lower priority value in favor

of a new session set-up with a valid namespace and higher relative

priority-value, unless local policy has some form of call-waiting

capability enabled. If a session is terminated, the BYE method is

used with a Reason header indicating why/where the preemption took

place.

[I-D.ietf-sipping-reason-header-for-preemption] provides additional

information in the case of purposeful or administrative termination

of a session by including the Reason header in the BYE message that

states why the BYE was sent (in this case, a preemption event).

That document offers several reasons for where the termination

occurred  ('at the UA', 'in the network', 'at a IP/PSTN gateway'),

and includes call flow examples of each reason.

4.5.2 User Agent Servers and Queue-based Policy

A UAS compliant with this specification SHOULD generate a 182

(Queued) response if that element's resources are busy, until it is

able to handle the request and provide a final response.

Local policy will determine additional behaviors of a queue-based

SIP element.  However, the frequency of provisional messages

indicated in [RFC3261] still need to apply to keep each session set-

up active until successful or rejected.

4.6  Proxy Behavior

SIP proxies MAY ignore the 'Resource-Priority' header

field.  SIP proxies MAY reject any unauthenticated request bearing

that header field.

When the 'Require' header is included in a message, it ensures that in parallel forking, only branches that support the

resource-priority mechanism succeed.

If a Proxy is expecting a message to have a 'Resource-Priority'

header and the header is encrypted via S/MIME encapsulation in a SIP

message fragment [RFC3420], the proxy MUST return an error response. this message.

Therefore, the use of this security mechanism with proxies that require the resource priority is NOT RECOMMENDED.

If no S/MIME is used, SIP proxies MAY downgrade or upgrade the

'Resource-Priority' of a request or insert a new 'Resource-Priority'

header if allowed by local policy.

This behavior is similar to that for any header field, as a UA can

decide to reject a request for the presence, absence or value of any

information in the request.

If a stateful proxy has authorized a particular resource priority

level and if it offers differentiated treatment to responses

containing resource priority levels, the proxy SHOULD ignore any

higher value contained in responses, to avoid that colluding user

agents artificially raise the priority level.

A SIP proxy MAY use the 'Resource-Priority' indication in its

routing decisions, e.g., to retarget to a SIP node or SIP URI that

is reserved for a particular resource priority.

There are no special considerations for proxies

when forking requests containing a resource priority indication.

Otherwise, the proxy behavior is the same as for user agent servers

Section 4.X.

5.  Third-Party Authentication

In some cases, the RP actor may not be able to authenticate the

requestor or determine whether an authenticated user is authorized to

make such a request.  In these circumstances, the SIP entity may

avail itself of general SIP mechanisms that are not specific to this

application.  The authenticated identity management mechanism

[RFC3893] allows a third party to verify the identity of the

requestor and certify this towards an RP actor.  In networks with

mutual trust, the SIP asserted identity mechanism  [RFC3325] can help

the RP actor determine the identity of the requestor.

6.  Backwards Compatibility

The resource priority mechanism described in this document is fully

backwards compatible with SIP systems following [RFC3261].  Systems

that do not understand the mechanism can only deliver standard, not

elevated, service priority. User agent servers and proxies can ignore

any 'Resource-Priority' header field just like any other unknown

header field and then treat the request like any other request.

Naturally, the request may still succeed.

Introducing 'Require' or 'Proxy-Require' would not help backwards

compatibility, as systems that do not support these mechanism are no

better off rejecting the request due to feature failure.  Since the

intent of resource priority indications is to increase the

probability of call completion, adding failure modes appears

counterproductive.

7.  Namespace Description

This specification defines five unique namespaces below: dsn, dsrn, q735, ets and wps, constituting their registration with IANA. Each INAA registration contains:

1) The namespace label, a unique namespace label within the IANA

registry for the Resource-Priority header;

2) The number of priority levels, i.e., the number of priority

levels the namespace has defined;

3) The intended operation, identifying whether the namespace is to be used with priority queueing or preemption;

4) Any new warning header codes unique to the namespace

being defined

5) Any new response codes specific to this namespace;

6) The IETF reference document specifying the parameters and

specifications of the namespace, the listing of the priority-

values in relative order, and any new error behaviors.

8. Multiple Namespaces

There are two cases where multiple namespaces might occur. Namely, a single RP actor may understand more than one namespace and a SIP request may contain more than one resource value from different namespaces. As noted earlier, the order of resource values in a SIP request is immaterial.

If a SIP element understands multiple namespaces, it must create a total ordering across all resource values from these namespaces, maintaining the relative ordering within each namespace. It it RECOMMENDED that the same ordering is used across an administrative domain.

If a ‘Resource-Priority’ header contains multiple namespaces, the RP actor MUST select one resource value, typically the one with the highest ranking as determined in the paragraph above.

For example, a SIP element supports two namespaces (foo and bar).

Foo's priority-values are 3 (highest), then 2 and then 1 (lowest ),

and bar's priority-values are C (highest), then B and then A

(lowest).

The following are 3 lists of acceptable priority orders the SIP

element MAY process are:

Foo.3        Foo.3       Bar.C    (Highest Priority)

Foo.2        Bar.C       Foo.3

Foo.1   or   Foo.2   or  Foo.2

Bar.C        Bar.B       Foo.1

Bar.B        Foo.1       Bar.B

Bar.A        Bar.A       Bar.A    (Lowest Priority)

Bar.C   (Highest Priority)

Foo.3  Bar.B <= both are equivalently processed FIFO)

or     Foo.2  Bar.A <= both are equivalently processed FIFO)

Foo.1   (Lowest Priority)

Bar.C     (Highest Priority)

Foo.3

or     Foo.2

Foo.1     (Lowest Priority)

Where Bar.A and Bar.B are ignored within this example administrative domain

Based on the stated priority order of each namespace above, the

following (non-inclusive list of) combinations are NOT acceptable

and MUST NOT be configurable:

Example 1    Example 2   Example 3

---------    ---------   ---------

Foo.3        Foo.3       Bar.C

Foo.2        Bar.A       Foo.1

Foo.1   or   Foo.2   or  Foo.3

Bar.C        Bar.B       Foo.2

Bar.A        Foo.1       Bar.A

Bar.B        Bar.C       Bar.B

Example 4

---------

Bar.C

Foo.1  Bar.B

or     Foo.3  Bar.A

Foo.2

In Example 1 above, Bar.A is ordered higher than Bar.B - which is

not consistent within the order of the namespace of this section.

In Example 2 above, Bar.A is ordered higher than Bar.B and Bar.C -

which is not consistent within the order of the namespace of this

section.

In Example 3 above, Foo.1 is ordered higher than Foo.2 and Foo.3 -

which is not consistent within the order of the namespace of this

section.

In Example 4 above, Foo.1 is ordered higher than Foo.3 and Foo.2 is

ignored - which is not consistent within the order of the namespace

of this section.

7.3 Future Namespace Considerations

Organizations considering the use of the Resource-Priority header

should investigate if an existing combination of namespace and

priority-values meets the needs, reducing the number of newly defined namespaces. For example, emergency first responders around the world are

discussing utilizing this mechanism for preferential treatment in

future networks.  There should not be a unique namespace for different jurisdictions. This will greatly

increase interoperability and reduce development time, and probably

reduce future confusion if there is ever a need to map one namespace

to another in an interworking function.

In the specification of any future namespace, several facets need to

be included:

1) New resource values proposed in the future

MUST be defined in a Standards Track RFC and MUST include an

Entry to following table offered to IANA for registration:

Intended          New     New Error

Namespace  Levels     Operation      Rej. code    code    Reference

---------  ------  ----------------  ---------    ----    ---------

<new      <# of    <preemption     <new error  <new Rej   <which

namespace>  Levels>    or queue>        code>      code>    IETF doc>

- as well as list set of priority values in relative

priority order (with no wildcards) for IANA Registration.

2) New priority-values MUST NOT be added to any previously (IANA)

registered list associated with a particular namespace, as doing so would cause interoperability problems.

3) If there is a new "intended behavior" (other than preemption-

based or queue-based), the parameters for that behavior must be

provided and how said behavior affects different types of RP actors.

4) Any new response codes used by this namespace need to be

explained.

5) Any new rejection codes or changes to existing rejections codes

as a result of the creation of the namespace need to be offered

and explained.

10 Namespace Descriptions

10.1 The "DSN" Namespace

The DSN namespace comes from the name of a US Government network

called "The Defense Switched Network".

The DSN namespace has five priority-values

listed here in the order from lowest priority to highest priority:

(lowest)   dsn.routine

dsn.priority

dsn.immediate

dsn.flash

(highest)  dsn.flash-override

The dsn namespace operates on the preemption policy.

The dsn namespace introduces no new error behaviors.

7.1.2 The "DRSN" Namespace

The DRSN namespace derives its name from the US Government network

called "The Defense RED Switched Network".

The DRSN namespace features six priority values

listed here in the order from lowest to highest priority:

(lowest)   drsn.routine

drsn.priority

drsn.immediate

drsn.flash

drsn.flash-override

(highest)  drsn.flash-override-override

The drsn namespace operates according to the preemption policy.

One unique facet regarding the DRSN namespace relative to the DSN

and Q735 namespaces is the behavior of SIP elements with regard to

the 6th priority-value:

drsn.flash-override-override

This is the highest relative priority-value within the DRSN

namespace, but it is only to be used at this higher level in message

processing in SIP Proxies, and dialog establishment in SIP UAs

(including gateways).  In any UAS, once the session is established

at the drsn.flash-override-override priority level, it defends that

dialog with a priority level of drsn.flash-override only; thus

allowing a new drsn.flash-override-override to preempt any existing

session.  This behavior MUST be followed to be compliant with this

specification. [I DON’t UNDERSTAND THIS.]

The drsn namespace introduces no new error behavior.

7.3 The "Q735" Namespace

The Q735 namespace is defined here as Q.735.3 was defined, namely, as

operationally equivalent to the DSN specification for MLPP.  Q.735.3

was created to be a commercial version of the same specification.

This namespace is case insensitive and does not use the ".3"

identifier. [WHY IS .3 LISTED BELOW THEN?]

The Q735 namespace has five priority values

listed here in the order of lowest to highest priority:

(lowest)   q735.4

q735.3

q735.2

q735.1

(highest)  q735.0

The q735 namespace operates according to the preemption policy.

The q735 namespace introduces no new error behaviors.

7.4 The "ETS" Namespace

The ETS namespace derives its name indirectly from the name of the

US Government Telecommunications Service called "Government

Emergency Telecommunications Service" (or GETS), though the

organization responsible for the GETS service chose the acronym

"ETS" for its GETS over IP service, which stands for "Emergency

Telecommunications Service".

The ETS namespace has five priority-values

listed here in the order from lowest to highest priority:

(lowest)   ets.4

ets.3

ets.2

ets.1

(highest)  ets.0

The ets namespace operates according to the priority queueing policy. ETS-based administrative domains will not queue normal, non-prioritized requests.

The ets namespace does not define new error behaviors.

7.5 The "WPS" Namespace

The WPS namespace derives its name from the "Wireless Priority

Service" defined in GSM and other wireless technologies.

This namespace has five priority values, listed here in the order from lowest to highest priority:

(lowest)   wps.4

wps.3

wps.2

wps.1

(highest)  wps.0

The wps namespace operates according to the priority queueuing policy and will not queue normal, non-prioritized messages.

The wps namespace defines no new error behaviors.

8.  Examples

The SDP message body and the BYE and ACK exchanges are the same as in

RFC 3665 [RFC3665] and omitted for brevity.

8.1  Simple Call

User A                  User B

|                        |

|       INVITE F1        |

|----------------------->|

|    180 Ringing F2      |

|<-----------------------|

|                        |

|       200 OK F3        |

|<-----------------------|

|         ACK F4         |

|----------------------->|

|   Both Way RTP Media   |

|<======================>|

|                        |

In this scenario, User A completes a call to User B directly. The

call from A to B is marked with a resource priority indication.

F1 INVITE User A -> User B

INVITE sip:UserB@biloxi.example.com SIP/2.0

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

Max-Forwards: 70

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 INVITE

Resource-Priority: dsn.flash

Contact: <sip:UserA@client.atlanta.example.com;transport=tcp>

Content-Type: application/sdp

Content-Length: ...

...

F2 180 Ringing User B -> User A

SIP/2.0 180 Ringing

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

;received=192.0.2.101

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>;tag=8321234356

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 INVITE

Resource-Priority: dsn.flash

Contact: <sip:UserB@client.biloxi.example.com;transport=tcp>

Content-Length: 0

F3 200 OK User B -> User A

SIP/2.0 200 OK

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

;received=192.0.2.101

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>;tag=8321234356

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 INVITE

Resource-Priority: dsn.flash

Contact: <sip:UserB@client.biloxi.example.com;transport=tcp>

Content-Type: application/sdp

Content-Length: ...

...

8.2  Receiver Does Not Understand Namespace

In this example, the receiving UA does not understand the "dsn"

namespace and thus returns a 417 (Unknown Resource-Priority) status

code.  We omit the message details for messages F5 through F7 since

they are essentially the same as in the first example.

User A                  User B

|                        |

|       INVITE F1        |

|----------------------->|

| 417 R-P failed F2      |

|<-----------------------|

|         ACK F3         |

|----------------------->|

|                        |

|       INVITE F4        |

|----------------------->|

|    180 Ringing F5      |

|<-----------------------|

|       200 OK F6        |

|<-----------------------|

|         ACK F7         |

|----------------------->|

|                        |

|   Both Way RTP Media   |

|<======================>|

F1 INVITE User A -> User B

INVITE sip:UserB@biloxi.example.com SIP/2.0

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

Max-Forwards: 70

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 INVITE

Resource-Priority: dsn.flash

Contact: <sip:UserA@client.atlanta.example.com;transport=tcp>

Content-Type: application/sdp

Content-Length: ...

...

F2 417 Resource-Priority failed  User B -> User A

SIP/2.0 417 Resource-Priority failed

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

;received=192.0.2.101

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>;tag=8321234356

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 INVITE

Accept-Resource-Priority: q735.0, q735.1, q735.2, q735.3, q735.4

Contact: <sip:UserB@client.biloxi.example.com;transport=tcp>

Content-Type: application/sdp

Content-Length: 0

F3 ACK User A -> User B

ACK sip:UserB@biloxi.example.com SIP/2.0

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bd5

Max-Forwards: 70

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>;tag=8321234356

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 1 ACK

Content-Length: 0

F4 INVITE User A -> User B

INVITE sip:UserB@biloxi.example.com SIP/2.0

Via: SIP/2.0/TCP client.atlanta.example.com:5060;branch=z9hG4bK74bf9

Max-Forwards: 70

From: BigGuy <sip:UserA@atlanta.example.com>;tag=9fxced76sl

To: LittleGuy <sip:UserB@biloxi.example.com>

Call-ID: 3848276298220188511@atlanta.example.com

CSeq: 2 INVITE

Resource-Priority: q735.3

Contact: <sip:UserA@client.atlanta.example.com;transport=tcp>

Content-Type: application/sdp

Content-Length: ...

...

9.  Security Considerations

Any resource priority mechanism can be abused to obtain resources and

thus deny service to other users.  An adversary may be able to take

over a particular gateway, cause additional congestion during PSTN

emergencies or deny service to legitimate users.  In the Internet,

or any IP domain, this mechanism can cause certain messages or

sessions (calls) to be given a higher relative priority of

processing within a SIP element (move to the head of the line

scenario), to the point of prematurely terminating an existing

session in favor of a newer one.  In some domains, this will be the

expected behavior for authenticated and authorized users (see

section 7). Unauthorized users MUST NOT be given this opportunity to

abuse network/element resources.

While the indication itself does not have to provide separate

authentication, any SIP request containing this header has higher

authentication requirements than regular requests.

A poor implementation of authentication and authorization will

likely cause illegitimate higher priority messages to process

without being successfully challenged for the privilege to do so.

While this will not likely have a great affect on the performance of

SIP Servers, this could have some (to a great) impact on the

premature termination of existing sessions.  Those domains wishing

to utilize a namespace with an operation of preemption of lower

relative priority sessions should use caution to ensure only the

proper usage of this header is granted.

Care should be taken on three counts:

1) To the domain that enables usage of the Resource-Priority header

such that adequate control exists to prevent unwanted

preferential message treatment from internal users.

Without an authentication and authorization mechanism to validate

each user request, unwanted usage (and potentially user hacked

settings) can have undesired affects on any internal network.

2) To the domain that enables usage of the Resource-Priority header

such that inadequate control exists to prevent unwanted

preferential message treatment from SIP messages from outside the

domain coming into the domain (and outside the area of direct AAA

control).

3) In the choosing of a namespace itself, to make sure the desired

behavior of SIP elements have equivalent behaviors defined in this

document to ensure interoperability and no surprises.

An example of this is choosing a namespace throughout a domain and

configuring it for preferential treatment with no preemption, even

though a neighbor domain uses it as it is IANA defined (with

preemption as one expected behavior), resulting in poor performance

of fist domain's calls into the second administrative domain's network.

Below, we describe authentication and authorization aspects,

confidentiality and privacy requirements, protection against denial

of service attacks and anonymity requirements.  Naturally, the

general discussion in RFC 3261 [RFC3261] applies.

All user agents and proxy servers which support this extension MUST

implement SIP over TLS [RFC3546] and the sips: URI scheme as

described in Section 26.2 of RFC 3261, and Digest Authentication

[RFC2617] as described in Section 22 of RFC 3261. In addition, user

agents which support this extension SHOULD also implement S/MIME

[RFC2633] as described in Section 23 of RFC 3261 to allow for signing

and verification of signatures over requests which use this

extension.

9.1  Authentication and Authorization

Prioritized access to network and end system resources imposes

particularly stringent requirements on authentication and

authorization mechanisms since access to prioritized resources may

impact overall system stability and performance, not just result in

theft of, say, a single phone call.

Under certain emergency conditions, the network infrastructure,

including its authentication and authorization mechanism, may be

under attack.

Given the urgency during emergency events, normal statistical fraud

detection may be less effective, thus placing a premium on reliable

authentication.

Common requirements for authentication mechanisms apply, such as

resistance to replay, cut-and-paste and bid-down attacks.

Authentication MAY be SIP-based or use other mechanisms.  Use of

Digest authentication and/or S/MIME is RECOMMENDED for UAS

authentication.  Digest authentication requires that the parties

share a common secret, thus limiting its use across administrative

domains.  SIP systems employing resource priority SHOULD implement S/

MIME at least for integrity, as described in Section 23 of [RFC3261].

However, in some environments, receipt of asserted identity [RFC3325]

from a trusted entity may be sufficient authorization.  Section 5

describes third-party authentication.

Trait-based authorization [I-D.ietf-sipping-trait-authz] "entails an

assertion by a authorization service of attributes associated with an

identity" and may be appropriate for this application.  With

trait-based authorization, a network element can directly determine,

by inspecting the certificate, that a request is authorized to obtain

a particular type of service, without having to consult a mapping

mechanism that converts user identities to authorizations.

Authorization may be based on factors beyond the identity of the

caller, such as the requested destination.  Namespaces MAY also

impose particular authentication or authorization consideration that

are stricter than the baseline described here.

9.2  Confidentiality and Integrity

Calls which use elevated resource priority levels provided by the

'Resource-Priority' header field are likely to be sensitive and often

need to be protected from intercept and alteration.  In particular,

requirements for protecting the confidentiality of communications

relationships may be higher than for normal commercial service.  For

SIP, the 'To', 'From', 'Organization' and 'Subject' header fields are

examples of particularly sensitive information.  Systems MUST

implement encryption at the transport level using TLS and MAY

implement other transport-layer or network-layer security mechanisms.

UACs SHOULD use the "sips" URI to request a secure transport

association to the destination.

The 'Resource-Priority' header field can be carried in the SIP

message header or can be encapsulated in a message fragment carried

in the SIP message body [RFC3420].  To be considered valid

authentication for the purposes of this specification, S/MIME signed

SIP messages or fragments MUST contain, at a minimum, the Date, To,

From, Call-ID, and Resource-Priority header fields.  Encapsulation in

S/MIME body parts allows the user to protect this header field

against inspection or modification by proxies.  However, in many

cases, proxies will need to authenticate and authorize the request,

so that encapsulation is undesirable.

Removal of a Resource-Priority header field or downgrading its

priority value affords no additional opportunities to an adversary

since that man-in-the-middle could simply drop or otherwise

invalidate the SIP request and thus prevent call completion.

Only SIP elements within the same administrative trust domain

employing a secure channel between their SIP elements will trust a

Resource-Priority header field that is not appropriately signed.

Others will need to authenticate the request independently.  Thus,

insertion of a Resource-Priority header field or upgrading the

priority value has no further security implications except causing a

request to fail (see discussion in the previous paragraph).

9.3  Anonymity

Some users may wish to remain anonymous to the request destination.

Anonymity for requests with resource priority is no different than

for any other authenticated SIP request.  For the reasons noted

earlier, users have to authenticate themselves towards the SIP

elements carrying the request where they desire resource priority

treatment.  The authentication may be based on capabilities and noms,

not necessarily their civil name.  Clearly, they may remain anonymous

towards the request destination, using the network-asserted identity

and general privacy mechanism described in [RFC3323].

9.4  Denial-of-Service Attacks

As noted, systems described here are likely to be subject to

deliberate denial-of-service (DoS) attacks during certain types of

emergencies.  DoS attacks may be launched on the network itself as

well as its authentication and authorization mechanism.  As noted,

systems should minimize the amount of state, computation and network

resources that an unauthorized user can command.  The system must not

amplify attacks by causing the transmission of more than one packet

to a network address whose reachability has not been verified.

10. IANA Considerations

This section defines two new SIP headers (10.1), one SIP OPTION tag

(10.2), one new 4XX error code (10.3), and a new registry within the

sip-parameters section of IANA for Resource-Priority namespaces and

priority-values (10.4).

Additional namespaces and priority values MUST be registered with

IANA.  Within each namespace, the registration MUST indicate the

relative priority levels, expressed as an ordered list.  New

priority-values MUST NOT be added to existing namespace registries.

The registration MUST describe, in the registration itself, how SIP

elements should treat requests from that namespace in 'operation',

e.g., whether preemption or only preferential queuing are allowed.

The SIP Change Process [RFC 3427] establishes a policy for the

registration of new SIP extension headers. Resource priority

namespaces and priority values have similar interoperability

requirements to those of SIP extension headers. Consequently,

registration of new resource priority namespaces and priority values

requires documentation in an RFC using the extension header approval

process specified in RFC 3427.

10.1 IANA Registration of 'Resource-Priority' and

'Accept-Resource-Priority' Header Fields

[NOTE TO RFC EDITOR:  Replace RFC XXXX with RFC number of this

document.]

The following is the registration for the 'Resource-Priority' header

field:

RFC number: XXXX

Header name: 'Resource-Priority'

Compact form: none

The following is the registration for the 'Accept-Resource-Priority'

header field:

RFC number: XXXX

Header name: Accept-Resource-Priority

Compact form: none

10.2 IANA Registration for Option Tag resource-priority

RFC number: XXXX

Name of option tag: 'resource-priority'

Descriptive text: Indicates or requests support for the resource

priority mechanism.

10.3 IANA Registration for Response Code 417

RFC number: XXXX

Response code: 417

Default reason phrase: Unknown Resource-Priority

10.4 IANA Namespace and Priority-Value Registrations

A new registry ("Resource-Priority Namespaces") in the

sip-parameters section of IANA is to be created taking a form

similar to this table below:

Intended          New     New Error

Namespace  Levels     Operation      Rej. code    code    Reference

---------  ------  ----------------  ---------    ----    ---------

dsn       5     preemption           no        no     [This RFC]

drsn      6     preemption        no        no     [This RFC]

q735      5     preemption        no        no     [This RFC]

ets       5     priority-queue    no        no     [This RFC]

wps       5     priority-queue    no        no    [This RFC]

Legend

------

Namespace      = the unique string of the namespace

Levels         = the number of priority-values within the namespace

Operation      = Intended (or expected) operational behavior of SIP

elements encountering this namespace

New Rej. Code  = New Rejection Codes introduced for this namespace

New Error Code = New Error Codes introduced for this namespace

Reference      = IETF Document reference for this namespace

10.5 IANA Priority-Value Registrations

A new registry ("Resource-Priority Priority-values") in the

sip-parameters section of IANA is to be created taking a form

similar to this table below (Reference [XXXX] is this RFC):

Namespace: drsn

Reference: [XXXX]

Priority-Values (least to greatest): "routine", "priority",

"immediate", "flash", "flash-override", "flash-override-override"

Namespace: dsn

Reference: [XXXX]

Priority-Values (least to greatest): "routine", "priority",

"immediate", "flash", "flash-override",

Namespace: q735

Reference: [XXXX]

Priority values (least to greatest): "4", "3", "2", "1", "0"

Namespace: ets

Reference: [XXXX]

Priority values (least to greatest): "4", "3", "2", "1", "0"

Namespace: wps

Reference: [XXXX]

Priority values (least to greatest): "4", "3", "2", "1", "0"

11.  Acknowledgments

Ben Campbell, Paul Kyzivat, Rohan Mahy, Allison Mankin, Mike Pierce, Samir

Srivastava and Dean Willis provided helpful comments.

Martin Dolly, An Nguyen and Niranjan Sandesara assisted with the

ets and wps namespaces.

Janet Gunn helped improve the queue-based policy

text.

12.  References

12.1  Normative References

[I.255.3]  International Telecommunications Union, "Integrated

Services Digital Network (ISDN) - General Structure and

Service Capabilities - Multi-Level Precedence and

Preemption", Recommendation I.255.3, July 1990.

[Q.735.3]  International Telecommunications Union, "Stage 3

description for community of interest supplementary

services using Signaling System No. 7: Multi-level

precedence and preemption", Recommendation Q.735.3, March

1993.

[RFC0791]  Postel, J., "Internet Protocol", STD 5, RFC 791, September

1981.

[RFC2119]  Bradner, S., "Key words for use in RFCs to Indicate

Requirement Levels", BCP 14, RFC 2119, March 1997.

[RFC3261]  Rosenberg, J., Schulzrinne, H., Camarillo, G., Johnston,

A., Peterson, J., Sparks, R., Handley, M. and E. Schooler,

"SIP: Session Initiation Protocol", RFC 3261, June 2002.

[RFC3262]  Rosenberg, J. and H. Schulzrinne, "Reliability of

Provisional Responses in Session Initiation Protocol

(SIP)", RFC 3262, June 2002.

[RFC3265]  Roach, A., "Session Initiation Protocol (SIP)-Specific

Event Notification", RFC 3265, June 2002.

[RFC3311]  Rosenberg, J., "The Session Initiation Protocol (SIP)

UPDATE Method", RFC 3311, October 2002.

[RFC3420]  Sparks, R., "Internet Media Type message/sipfrag", RFC

3420, November 2002.

[RFC3428]  Campbell, B., Rosenberg, J., Schulzrinne, H., Huitema, C.

and D. Gurle, "Session Initiation Protocol (SIP) Extension

for Instant Messaging", RFC 3428, December 2002.

[I-D.ietf-sipping-reason-header-for-preemption]

Polk, J., "Reason Header for Preemption within the Session

Initiation Protocol  (SIP)",

draft-ietf-sipping-reason-header-for-preemption-02 (work

in progress), August 2004

12.2  Informative References

[I-D.ietf-ieprep-framework]

Carlberg, K., Brown, I. and C. Beard, "Framework for

Supporting ETS in IP Telephony",

draft-ietf-ieprep-framework-09 (work in progress), April

2004.

[RFC3893]  Peterson, J., "SIP Authenticated Identity Body (AIB)

Format", RFC 3893, May 2004.

[RFC3903]  Niemi, A., "An Event State Publication Extension to the

Session Initiation Protocol  (SIP)", RFC3903, May 2004.

[I-D.ietf-sipping-trait-authz]

Peterson, J., "Trait-based Authorization Requirements for

the Session Initiation Protocol  (SIP)",

draft-ietf-sipping-trait-authz-00 (work in progress),

February 2004.

[RFC2617]  Franks, J., Hallam-Baker, P., Hostetler, J., Lawrence, S.,

Leach, P., Luotonen, A. and L. Stewart, "HTTP

Authentication: Basic and Digest Access Authentication",

RFC 2617, June 1999.

[RFC2633]  Ramsdell, B., "S/MIME Version 3 Message Specification",

RFC 2633, June 1999.

[RFC2976]  Donovan, S., "The SIP INFO Method", RFC 2976, October

2000.

[RFC3323]  Peterson, J., "A Privacy Mechanism for the Session

Initiation Protocol (SIP)", RFC 3323, November 2002.

[RFC3324]  Watson, M., "Short Term Requirements for Network Asserted

Identity", RFC 3324, November 2002.

[RFC3325]  Jennings, C., Peterson, J. and M. Watson, "Private

Extensions to the Session Initiation Protocol (SIP) for

Asserted Identity within Trusted Networks", RFC 3325,

November 2002.

[RFC3487]  Schulzrinne, H., "Requirements for Resource Priority

Mechanisms for the Session Initiation Protocol (SIP)", RFC

3487, February 2003.

[RFC3515]  Sparks, R., "The Session Initiation Protocol (SIP) Refer

Method", RFC 3515, April 2003.

[RFC3546]  Blake-Wilson, S., Nystrom, M., Hopwood, D., Mikkelsen, J.

and T. Wright, "Transport Layer Security (TLS)

Extensions", RFC 3546, June 2003.

[RFC3550]  Schulzrinne, H., Casner, S., Frederick, R. and V.

Jacobson, "RTP: A Transport Protocol for Real-Time

Applications", STD 64, RFC 3550, July 2003.

[RFC3665]  Johnston, A., Donovan, S., Sparks, R., Cunningham, C. and

K. Summers, "Session Initiation Protocol (SIP) Basic Call

Flow Examples", BCP 75, RFC 3665, December 2003.

Authors' Addresses

Henning Schulzrinne

Columbia University

Department of Computer Science

450 Computer Science Building

New York, NY  10027

US

Phone: +1 212 939 7042

EMail: hgs@cs.columbia.edu

URI:   http://www.cs.columbia.edu

James Polk

Cisco Systems

2200 East President George Bush Turnpike

Richardson, TX  75082

US

EMail: jmpolk@cisco.com

Intellectual Property Statement

The IETF takes no position regarding the validity or scope of any

Intellectual Property Rights or other rights that might be claimed to

pertain to the implementation or use of the technology described in

this document or the extent to which any license under such rights

might or might not be available; nor does it represent that it has

made any independent effort to identify any such rights. Information

on the IETF's procedures with respect to rights in IETF Documents can

be found in BCP 78 and BCP 79.

Copies of IPR disclosures made to the IETF Secretariat and any

assurances of licenses to be made available, or the result of an

attempt made to obtain a general license or permission for the use of

such proprietary rights by implementers or users of this

specification can be obtained from the IETF on-line IPR repository at

http://www.ietf.org/ipr.

The IETF invites any interested party to bring to its attention any

copyrights, patents or patent applications, or other proprietary

rights that may cover technology that may be required to implement

this standard. Please address the information to the IETF at

ietf-ipr@ietf.org.

Disclaimer of Validity

This document and the information contained herein are provided on an

"AS IS" basis and THE CONTRIBUTOR, THE ORGANIZATION HE/SHE REPRESENTS

OR IS SPONSORED BY (IF ANY), THE INTERNET SOCIETY AND THE INTERNET

ENGINEERING TASK FORCE DISCLAIM ALL WARRANTIES, EXPRESS OR IMPLIED,

INCLUDING BUT NOT LIMITED TO ANY WARRANTY THAT THE USE OF THE

INFORMATION HEREIN WILL NOT INFRINGE ANY RIGHTS OR ANY IMPLIED

WARRANTIES OF MERCHANTABILITY OR FITNESS FOR A PARTICULAR PURPOSE.

Copyright Statement

Copyright (C) The Internet Society (2005). This document is subject

to the rights, licenses and restrictions contained in BCP 78, and

except as set forth therein, the authors retain all their rights.

Acknowledgment

Funding for the RFC Editor function is currently provided by the

Internet Society.