LIF DELIVERABLE COPYRIGHT NOTICE AND DISCLAIMER

© COPYRIGHT LOCATION INTEROPERABILITY FORUM LTD. 2001

Implementation of all or part of this LIF Deliverable may require licenses under or in respect of intellectual property rights including, without limitation, patent rights of a third party (who may or may not be a LIF Member).  Neither Location Interoperability Forum Ltd. nor Location Interoperability Forum nor any of its members is responsible, and shall not be held responsible, in any manner for identifying or failing to identify any or all such third party intellectual property rights.

- Mobile Location Protocol Specification

Abstract

The purpose of this specification is to define a simple and secure access method that allows Internet applications to query location information from a wireless network, irrespective of its underlying air interface technologies and positioning methods.

This specification covers the core of a Mobile Location Protocol that can be used by a location-based application to request MS location information from a location server (GMLC/MPC or other entity in the wireless network).

This specification has been prepared by LIF to provide a simple and secure API (Application Programmer’s Interface) to the location server, but that also could be used for other kinds of location servers and entities in the wireless network.

The API is based on existing and well-known Internet technologies as HTTP, SSL/TLS and XML, in order to facilitate the development of location-based applications.

Contents

1	Revision History	4

2	Introduction	5

2.1	Abbreviations	5

2.2	Notational Conventions and Generic Grammar	5

3	General	7

3.1	Overview	7

3.2	MLP structure	7

3.3	Protocol bearer	8

3.4	Location Services	9

3.5	Coordinate systems (Informative)	11

3.6	Supported coordinate systems and datum	14

3.7	Conversions between coordinate systems and datum (Informative)	14

3.8	Shapes representing a geographical position	15

3.9	MLP extension mechanism	18

4	Mobile Location Service Definitions	19

4.1	General	19

4.2	Common Element Definition	19

4.3	Request Header Components	23

4.4	Standard Location Immediate Service	25

4.5	Emergency Location Immediate Service	30

4.6	Standard Location Reporting Service	32

4.7	Emergency Location Reporting Service	33

4.8	Triggered Location Reporting Service	35

5	Elements and attributes in DTD	41

5.1	add_info	41

5.2	alt	41

5.3	alt_acc	41

5.4	angle	42

5.5	cc	42

5.6	cellid	42

5.7	coord_sys	42

5.8	direction	43

5.9	datum	43

5.10	easting	43

5.11	eme_event	43

5.12	esrd	44

5.13	esrk	45

5.14	format	45

5.15	geo_info	46

5.16	hor_acc	47

5.17	id	47

5.18	in_rad	47

5.19	interval	47

5.20	lac	48

5.21	lat	48

5.22	lev_conf	48

5.23	ll_acc	49

5.24	loc_type	49

5.25	long	49

5.26	max_loc_age	50

5.27	mcc	50

5.28	mnc	50

5.29	ms_action	50

5.30	msid	51

5.31	ndc	52

5.32	nmr	52

5.33	northing	52

5.34	out_rad	53

5.35	prio	53

5.36	pwd	54

5.37	rad	54

5.38	req_id	54

5.39	resp_req	54

5.40	resp_timer	55

5.41	result	55

5.42	semi_major	56

5.43	semi_minor	56

5.44	serviceid	56

5.45	servicetype	57

5.46	session	57

5.47	sessionid	58

5.48	speed	58

5.49	start_angle	58

5.50	start_msid	58

5.51	start_time	59

5.52	stop_angle	59

5.53	stop_msid	60

5.54	stop_time	60

5.55	subclient	61

5.56	ta	61

5.57	time	61

5.58	time_remaining	62

5.59	trl_trigger	63

5.60	url	63

5.61	vlrno	63

5.62	vmscno	63

5.63	x	64

5.64	y	64

5.65	zone	64

5.66	zone_des	65

5.67	Service attributes	65

6	Result codes and error codes	66

6.1	Result codes	66

7	References (Normative and Informative)	67

8	Appendix A (informative)	68

9	Appendix B (informative)	69

# Revision History

| 1.0 | 23-Jan-2001 | Sanjiv Bhatt, Motorola | Motorola, Nokia, Ericsson contribution to LIF |
| --- | --- | --- | --- |
| 1.1 | 26-Jan-2001 | Sanjiv Bhatt, Motorola | Updated after review in MLP adhoc committee in LIF #2 meeting |
| 1.1.1 | 5-Nov-2001 | Sanjiv Bhatt, Motorola | Updated after SIG#6 meeting |
| 1.1.2 | 17-Nov-2001 | Sanjiv Bhatt, Motorola | Updated after SIG#7 meeting |
| 2.0.0 | 20-Nov-2001 | Sanjiv Bhatt, Motorola | Final version (public release) |

# Introduction

The Mobile Location Protocol (MLP) is an application-level protocol for getting the position of mobile stations (mobile phones, wireless personal digital assistants, etc.) independent of underlying network technology. The MLP serves as the interface between a Location Server and a Location Services (LCS) Client. This specification defines the core set of operations that a Location Server should be able to perform.

## Abbreviations

ANSI	American National Standards Institute

DTD	Document Type Definition

GMLC	Gateway Mobile Location Center

GMT	Greenwich Mean Time

HTTP	Hypertext Transfer Protocol

HTTPS	HTTP Secure

LCS	Location Services
MLC	Mobile Location Center

MLP	Mobile Location Protocol

MPC	Mobile Positioning Center 
MS	Mobile Station
MSID	Mobile Station Identifier
SSL	Secure Socket Layer

TLS	Transport Layer Security
URI	Uniform Resource Identifier
URL	Uniform Resource Locator

UTM 	Universal Transverse Mercator

WGS	World Geodetic System

XML	Extensible Markup Language

## Notational Conventions and Generic Grammar

The following rules are used throughout this specification to describe basic parsing constructs. ANSI X3.4-1986 defines the US-ASCII coded character set, see ref. [7]

CR 	= <US-ASCII CR, carriage return (13)>
LF	= <US-ASCII LF, linefeed (10)>
SP             	= <US-ASCII SP, space (32)>

A set of characters enclosed in brackets ([]) is a one-character expression that matches any of the characters in that set. E.g., "[lcs]" matches either an "l", "c", or "s". A range of characters is indicated with a dash. E.g., "[a-z]" matches any lower-case letter.

The one-character expression can be followed by an interval operator, for example [a-zA-Z]{min,max} in which case the one-character expression is repeated at least min and at most max times. E.g., "[a-zA-Z]{2,4}" matches for example the strings "at", "Good", and "biG".

DTD Syntax Notation

The table below describes the special characters and separators used in the DTDs defining the different services.

| Character | Meaning |
| --- | --- |
| + | One or more occurrence |
| * | Zero or more occurrences |
| ? | Optional |
| () | A group of expressions to be matched together |
| ¦ | OR...as in, "this or that" |
| , | Strictly ordered. Like an AND |

# General

## Overview

The Mobile Location Protocol (MLP) is an application-level protocol for querying the position of mobile stations independent of underlying network technology. The MLP serves as the interface between a Location Server and a location-based application.

Wireless

Network

Location-based application

request (MLP)

response (MLP)

Location Server

Possible realizations of a Location Server are the GMLC, which is the location server defined in GSM and UMTS, and the MPC, which is defined in ANSI standards. Since the location server should be seen as a logical entity, other implementations are possible.

In the most scenarios (except where explicitly mentioned) an LCS client initiates the dialogue by sending a query to the location server and the server responds to the query.

## MLP structure

In our heterogeneous world, different devices may support different means of communication. A ubiquitous protocol for location services should support different transport mechanisms.

In MLP, the transport protocol is separated from the XML content. The following diagram shows a layered view of MLP.

On the lowest level, the transport protocol defines how XML content is transported. Possible MLP transport protocols include HTTP, WSP, SOAP and others. The HTTP protocol is the only currently defined MLP transport, but others will be defined in the future.

The Element Layer shown in the diagram defines all common elements used by the services in the service layer.

The Service Layer defines the actual services offered by the MLP framework. Basic MLP Services are based on location services defined by 3GPP, and are defined by this specification. The Advanced MLP Services and Other MLP Services are additional services LIF will define in other specifications.

Note: The boxes representing services in the Service Layer may contain more than one message. E.g. SLIS (Standard Location Immediate Service) consists of SLIR (Standard Location Immediate Request) and SLIA (Standard Location Immediate Answer) messages.

The Service Layer is divided into two sub-layers. The topmost defines the services mentioned in the previous paragraph. The lower sub-layer holds common elements used by that group of services.

## Protocol bearer

MLP can be implemented using various transport mechanism as stated in above section. The following describes how to use MLP over the HTTP transport mechanism.

MLP is implemented on top of "HTTP/1.1". HTTP is a request/response protocol involving a server and a client. In the context of MLP, the client is referred to as the LCS Client and the server is the Location Server (GMLC/MPC). For more information about HTTP, refer to http://www.w3.org and ref [3].

The Location Server should provide two socket ports for operation, one for encryption with SSL/TLS and one without. The reason for having one insecure port is that encryption can consume resources, and if the client is in a secure domain there might not be a need for encryption. Applications residing in an insecure domain, i.e. on the Internet, may use the secure port to ensure the security and privacy of the location information.

For further information about SSL/TLS see ref [4].

Two port numbers should be selected and proposed as standard ports for location servers implementing MLP. The ports should be registered by IANA (Internet Assigned Numbers Authority, see ref [6]). Two port numbers are proposed below.

- 700 for secure SSL/TLS transfers

- 701 for insecure transfers

A Location Server can choose to introduce any other socket based or HTTP transparent technology for secure transfers. Any such technology should be provided over a different port than the two mentioned above.

## Location Services

An LCS Client requests a Location Service by issuing an HTTP POST request towards the Location Server. For more information about HTTP POST, see ref. [3]. The request line syntax is shown below.

Request-line:	 POST SP host SP HTTP/1.1 CRLF

The request must include the entity-header Content-length field as part of the request. The message body of the request should include the XML formatted request and should have the length specified by the LCS Client in the Content-length field.

If the request is a deferred request (triggered or periodic) the result is delivered to the client through an HTTP POST operation issued by the Location Server. This implies that the client must be able to receive HTTP POST requests and be able to give a valid response.

All Location Services are invoked by sending a request using HTTP POST to a certain URI. An example of an URI is shown below.

http://host:port/LocationQueryService/

The response to the invocation of a Location Service is returned using an HTTP response.

If the LCS client requests triggered or periodic reporting of location, the Location Server will return the answer by performing an HTTP POST operation towards the client. The client must specify the URI that the answer should be posted to. This is done in the service request or by having it in the LCS client profile that can be stored in the Location Server.

The answer will be included in the message body and the Content-length entity will be set to the length of the answer.

There are a number of different possible types of location services. Each implementation of location server can select which services it wants/needs to support. The services are described in the table below.

| Service | Description |
| --- | --- |
| Standard Location Immediate Service | This is a standard query service with support for a large set of parameters. This service is used when a single location response is required immediately (within a set time). This service consists of the following messages: - Standard Location Immediate Request - Standard Location Immediate Answer - Standard Location Immediate Report |
| Emergency Location Immediate Service | This is a service used especially for querying of the location of a mobile subscriber that has initiated an emergency call. The response to this service is required immediately (within a set time). This service consists of the following messages: - Emergency Location Immediate Request - Emergency Location Immediate Answer |
| Standard Location Reporting Service | This is a service that is used when a mobile subscriber wants an LCS Client to receive the MS location. The position is sent to the LCS Client from the location server. Which application and its address are specified by MS or defined in the location server. This service consists of the following message: - Standard Location Report |
| Emergency Location Reporting Service | This is a service that is used when the wireless network automatically initiates the positioning at an emergency call. The position and related data is then sent to the emergency application from the location server. Which application and its address are defined in the location server. This service consists of the following message: - Emergency Location Report |
| Triggered Location Reporting Service | This is a service used when the mobile subscriber’s location should be reported at a specific time interval or on the occurrence of a specific event. This service consists of the following messages: - Triggered Location Reporting Request - Triggered Location Reporting Answer - Triggered Location Report - Triggered Location Reporting Stop - Triggered Location Reporting Stop Answer |

## Coordinate systems (Informative)

### Cartesian coordinates

The simplest coordinate system is Cartesian coordinates, defined by values of (x,y,z). x is the distance from the x-axis, y is the distance from the y-axis, z the distance from the z-axis.

This coordinate system is primary used for conversions.

### Ellipsoid coordinates

Most geographic calculations are based on the surface of the earth. So we need a second coordinate system that describes each position relative to other points and lines on the earth’s surface. The irregular shape of the earth is typically approximated by an ellipsoid.

Each point with the Cartesian coordinates (x, y, z) can then be described as set of values (longitude, latitude, altitude) relative to the ellipsoid we choose to describe the earth. The longitude tells us how far east we have to move on the equator from the null-meridian, the latitude tells us how far north to move from the equator and the altitude tells us how far above the ellipsoid  to go to finally reach the location. Negative values direct us to go in the opposite direction.

#### To minimize the difference between the real shape of the earth and the mathematical ellipsoid in different regions, different geographic authorities use different ellipsoids. All such ellipsoids are defined by a set of 6 parameters which describe the size and the orientation of the ellipsoid in respect of the Cartesian coordinate system. This set is called "Datum". Typical examples are WGS-84 and Bessel-1841

### Planar coordinates

Ellipsoid coordinates describe a 3D object (ellipsoid) and are not located on a plane. To draw 2D maps and easily calculate distances, angles or areas the ellipsoid coordinates have to be converted to planar coordinates (x,y).

There are several ways to convert ellipsoid coordinates to planar coordinates:

The simplest is a 1:1 transformation: x=longitude, y=latitude. If the longitude/latitude pairs of the borders of the countries are converted this way to (x,y) and drawn on a paper, we get a typical earth map with x in the range of -180 till +180 and y between -90  till +90.

If the transformation is x=1/180*longitude and y=1/90*latitude the resulting map will be smaller and have both for x and y a range of -1 till 1.

Using trigonometric transformations, we can create other earth maps including various well known maps.

Each transformation system has advantages and disadvantages. Some transformations have a special behaviors suited for particular regions or purposes. If we transform three different points from an ellipsoid to a planar map, the planar points may have the following desirable characteristics:

- Have nearly the same distance to each other as on the ellipsoid. A typical example is the UTM coordinate system

- Have nearly the same angle to each other as on the ellipsoid

- Span the same area as on the ellipsoid.

No planar system supports all these characteristics.

Furthermore, one planar transformation cannot be used for all points of the world. Depending on the transformation, some ellipsoid points cannot be transformed or several ellipsoid points can be mapped to the same planar destination. To solve this problem a rotation is added to the mathematical transformation. A candidate for a modification parameter is for example the longitude divided by a constant factor, for example 6 (UTM) or 15. This is the so called 'zone'.

#### UTM coordinate system

The UTM coordinate system is a planar coordinate systems based on the Universal Transverse Mercator map projection.

It provides positional descriptions accurate to 1 meter in 2,500 across the entire earth's surface except the poles. At the poles, the Universal Polar Stereographic projection is used. The UTM system divides the earth's surface into a grid in which each cell, excluding overlap with its neighbors, is 6 degrees east to west and 8 degrees north to south (with the exception of the row from 72-84 degrees north latitude).

For any position in the UTM grid, coordinates can be determined in eastings and northings.

Eastings are in meters with respect to a central meridian drawn through the center of each grid zone (and given an arbitrary easting of 500,000 meters). In the Northern Hemisphere, northings are read in meters from the equator (0 meters). In the Southern Hemisphere, the equator is given the false northing of 10 million meters.

The UTM coordinate system defines two-dimensional, horizontal positions.

In the UTM coordinate system the world is divided into 6-degree longitudal strips extending from 80 degrees South latitude to 84 degrees North latitude. UTM zone characters designate 8-degree zones extending north and south from the equator.

There are special UTM zones between 0 degrees and 36 degrees longitude above 72 degrees latitude and a special zone 32 between 56 degrees and 64 degrees north latitude. Each zone has a central meridian. As an example, zone 14 has a central meridian of 99 degrees west longitude. The zone extends from 96 to 102 degrees west longitude.

Positions are measured in Easting from the central meridian and in Northing from the equator.

UTM Zone Numbers

UTM Zone Designators

## Supported coordinate systems and datum

All MLP implementations support at least the following coordinate systems:

- The ellipsoid coordinate system

- UTM

Although the ellipsoid coordinate system is sufficient for many calculations, MLP supports one additional planar coordinate system: UTM. Using UTM, clients can just ask for rectangular metric coordinates and don't have to worry about complicated coordinate transformations.

All implementations support at least the ellipsoid WGS-84.

Other locally used ellipsoid coordinate systems and datums (such as BESSEL-1841) may additionally be supported by particular implementations.

For each UTM position request, the UTM zone can be preselected. If the zone is not defined in the request the Location Server assigns a zone automatically.

When an application wants to compute the approximate distance between nearby points, it is easy if the planar coordinates are relative to the same zone. The zone with the point closest to the central meridian should be used as this would minimize distortion (unless the points are separated by more than ~6 degrees).

## Conversions between coordinate systems and datum (Informative)

The supported coordinate systems and datums can be converted to other coordinate systems and datums. For example, to change coordinates based on datum WGS-84 to another datum perform the following two steps:

- Convert the values (longitude, latitude, altitude) on the WGS-84 ellipsoid to Cartesian coordinates (x,y,z). Well-known mathematical equations do this transformation and typically input the coordinate parameters (longitude, latitude, altitude) and the 6 datum parameters defined by the datum WGS-84.

- Transform the Cartesian coordinates from step [1] to the desired datum. The Cartesian coordinates (x,y,z) are independent of all ellipsoids. So these coordinates can be transformed with the inverse mathematical equation from step [1] to coordinates of the new ellipsoid. A typical equation takes as parameters (x,y,z) and the 6 datum parameters defined by the new requested datum. The result is a new set of values for (longitude, latitude, altitude).

To get from there to another coordinate system the ellipsoid points are simply mathematically converted into the new coordinate system, such as Gauss-Krüger. A typical input for these transformations is (longitude, latitude, altitude) and the output Cartesian values (x,y,h). The planar coordinate units are usually metric and may be used to calculate distances, angles or areas depending on the characteristics of the transformation which is used.

The units of Cartesian values depend on the transformation formulas used. If the same type of transformation (i.e. JP19) should be used with different units (ie. 1 meter or 0.1 meter) this can be handled by defining for each combination a transformation (ie. JP19_m and JP19_dm). In this example, if a client selects JP19_dm the client gets JP19 coordinates with the unit 0.1.

## Shapes representing a geographical position

There are a number of shapes used to represent a geographic area that describes where a mobile subscriber is located.

Such shapes can also be used to define triggering criteria that might initiate a positioning when the mobile subscriber enters or leaves the described geographical area. (Such area-based triggering events are not yet defined in the MLP specification.)

### Ellipsoid point with uncertainty circle

An ellipsoid point with uncertainty circle is characterized by the coordinates of an ellipsoid point (the origin) and a radius, “r”. It describes the set of points on the ellipsoid, which are at a distance from the point of origin less than or equal to “r”. This shape can be used to indicate points on the Earth surface, or near the Earth surface.

The typical use of this shape is to indicate a point when its position is known only with a limited accuracy.

r

### Ellipsoid point with uncertainty ellipse

The shape of an "ellipsoid point with uncertainty ellipse" is characterized by the following:

- The coordinates of an ellipsoid point (the origin)

- The distances r1 and r2

- The angle of orientation A

It describes formally the set of points on the ellipsoid, which fall within or on the boundary of an ellipse. This ellipse has a semi-major axis of length r1 oriented at angle A (0 to 180) measured clockwise from north and a semi-minor axis of length r2. The distances being the geodesic distance over the ellipsoid, i.e., the minimum length of a path staying on the ellipsoid and joining the two points, as shown in figure below.

As for the ellipsoid point, this can be used to indicate points on the Earth’s surface, or near the Earth’s surface, of same latitude and longitude.

semi-major axis, r1

North

angle, A

semi-minor axis, r2

The typical use of this shape is to indicate a point when its position is known only with a limited accuracy, but the geometrical contributions to uncertainty can be quantified.

### Ellipsoid point with uncertainty arc

The shape of an "ellipsoid point with uncertainty arc" is characterized by the following:

- The coordinates of an ellipsoid point (the origin)

- The inner and outer radius, rin and rout

- The start and stop angles, Astart and Astop

An arc is defined by a point of origin with one start and one stop angle plus one inner radius and one outer radius. In this case the striped area describes the actual arc area. The smaller circle defines the inner radius and the outer circle defines the outer radius.

0/3600

rout

Astart

Astop

rin

### Polygon

A polygon is an arbitrary shape described by an ordered series of points. The minimum number of points allowed is 3, and the maximum number of points allowed is 15. The points shall be connected in the order that they are given. A connecting line is defined as the line over the ellipsoid joining the two points and of minimum distance (geodesic). The last point is connected to the first. The list of points must respect a number of conditions:

- A connecting line shall not cross another connecting line;

- Two successive points must not be diametrically opposed on the ellipsoid.

The described area is situated to the right of the lines with the downward direction being toward the Earth’s center and the forward direction being from a point to the next.

NOTE: This definition does not permit connecting lines greater than 
roughly 20 000 km. If such a need arises, the polygon can be described by adding an intermediate point.

Computation of geodesic lines is not simple. Approximations leading to a maximum distance between the computed line and the geodesic line of less than 3 meters are acceptable.

## MLP extension mechanism

The MLP specification has been designed with extensibility in mind. Examples of design principles employed to achieve this include:

- Separate DTDs for each message element allows new messages to be added.

- Separate DTDs for definitions that are common to all messages, e.g. client address and shapes, so they can be re-used.

- Parameter extension mechanism allowing the addition of additional parameters to existing messages. This mechanism works by specifying an entity parameter referring to an extension DTD. The extension DTD MUST contain another entity parameter ‘%extension.param’ containing the definition of the extension as a string together with the actual parameters being added.

Each extension parameters should have a vendor specific prefix in order to guarantee their uniqueness.  

In order to use the extension, the extension DTD has to be explicitly referenced in the XML document.

The Location Server may ignore any extension that is not recognized and process the message as if the extension is not available.

| <!—truckco_MLP_extension --> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY | % extension_param | “truko_extension”> |  |
| <!ELEMENT | Trucko_extension | (#PCDATA)> |  |

Example

<?xml version = "1.0" ?>

<!DOCTYPE slir SYSTEM "MLP_SLIR_200.DTD" [

<!ENTITY % extension SYSTEM "http://www.truckco.com/truckco_MLP_extension.dtd">

%extension;

]>

<slir ver="2.0.0">

...

<truckco_extension>

...

</truckco_extension>

</slir>

# Mobile Location Service Definitions

## General

All the different Location Services in the MLP are defined using XML DTDs.

Since the presence of common structured element among the different services, the DTD that defines a single location service, is composed only by the definition of the root element and the inclusion of the necessary common DTD. The MLP is distributed over a set of common DTDs, each acting to define its core element:

MLP_ID.DTD : Identify Element Definitions

MLP_FUNC.DTD : Function Element Definitions

MLP_LOC.DTD : Location Element Definitions

MLP_SHAPE.DTD : Shape Element Definitions

MLP_QoP.DTD : Quality of Position Element Definitions

MLP_GSM_NET.DTD : GSM Network Parameters Element Definitions

MLP_CTXT: Context Element Definitions

The following DTDs describe the different Location Services. All elements and their attributes are described in chapter 5.

## Common Element Definition

### Identity Element Definitions

| <!-- MLP_ID --> |  |  |
| --- | --- | --- |
| <!ELEMENT | msid | (#PCDATA)> |
| <!ATTLIST | msid |  |
|  | type (MSISDN ¦ IMSI ¦ IMEI ¦ MIN ¦ MDN ¦ EME_MSID ¦ IPV4 ¦ IPV6) | "MSISDN" |
|  | enc (ASC ¦ B64 ¦ CRP) | "ASC"> |
| <!ELEMENT | msid_range | (start_msid, stop_msid)> |
| <!ATTLIST | msid_range |  |
|  | type (MSISDN ¦ IMSI ¦ IMEI ¦ MIN ¦ MDN ¦ EME_MSID ¦ IPV4 ¦ IPV6) | "MSISDN" |
|  | enc (ASC ¦ B64 ¦ CRP) | "ASC"> |
| <!ELEMENT | msids | ((msid, session? ¦ msid_range)+)> |
| <!ELEMENT | esrd | (#PCDATA)> |
| <!ATTLIST | esrd |  |
|  | type (NA) | "NA"> |
| <!ELEMENT | esrk | (#PCDATA)> |
| <!ATTLIST | esrk |  |
|  | type (NA) | "NA"> |
| <!ELEMENT | session | (#PCDATA) |
|  | session |  |
|  | Type (APN ¦ dial) | #REQUIRED> |
| <!ELEMENT | start_msid | (#PCDATA)> |
| <!ELEMENT | stop_msid | (#PCDATA)> |

### Function Element Definitions

| <!-- MLP_FUNC --> |  |  |
| --- | --- | --- |
| <!ELEMENT | add_info | (#PCDATA)> |
| <!ELEMENT | eme_event | (eme_pos+)> |
| <!ATTLIST | eme_event |  |
|  | eme_trigger (EME_ORG ¦ EME_REL) | #REQUIRED> |
| <!ELEMENT | tlrr_event | (time ¦ area_change ¦ ms_action)> |
| <!ELEMENT | ms_action | EMPTY> |
| <!ATTLIST | ms_action |  |
|  | type (MS_AVAIL) | #REQUIRED> |
| <!ELEMENT | interval | (#PCDATA)> |
| <!ELEMENT | loc_type | EMPTY> |
| <!ATTLIST | loc_type |  |
|  | type (CURRENT ¦ LAST ¦ CURRENT_OR_LAST ¦ INITIAL) | "CURRENT"> |
| <!ELEMENT | poserr | (result, time)> |
| <!ELEMENT | prio | EMPTY> |
| <!ATTLIST | prio |  |
|  | type (NORMAL ¦ HIGH) | "NORMAL"> |
| <!ELEMENT | pushaddr | (url)> |
| <!ELEMENT | req_id | (#PCDATA)> |
| <!ELEMENT | result | (#PCDATA)> |
| <!ATTLIST | result |  |
|  | resid CDATA | #REQUIRED> |
| <!ELEMENT | start_time | (#PCDATA)> |
| <!ATTLIST | start_time |  |
|  | utc_off CDATA | "0000"> |
| <!ELEMENT | stop_time | (#PCDATA)> |
| <!ATTLIST | stop_time |  |
|  | utc_off CDATA | "0000"> |
| <!ELEMENT | time | (#PCDATA)> |
| <!ATTLIST | time |  |
|  | utc_off CDATA | "0000"> |
| <!ELEMENT | url | (#PCDATA)> |
| <!ELEMENT | Time_remaining | (#PCDATA)> |

### Location Element Definitions

| <!-- MLP_LOC --> |  |  |
| --- | --- | --- |
| <!ELEMENT | pos | (msid, (pd ¦ poserr), gsm_net_param?)> |
| <!ELEMENT | eme_pos | (msid, esrd?, esrk?, (pd ¦ poserr))> |
| <!ELEMENT | trl_pos | (msid, (pd ¦ poserr))> |
| <!ELEMENT | trl_pos |  |
| <!ATTLIST | trl_trigger (TIMER ¦ PERIODIC ¦ MS_AVAIL) |  |
| <!ELEMENT | pd | (time, shape, (alt, alt_acc?)?, speed?, direction?, lev_conf?)> |
| <!ELEMENT | alt | (#PCDATA)> |
| <!ELEMENT | alt_acc | (#PCDATA)> |
| <!ELEMENT | direction | (#PCDATA)> |
| <!ELEMENT | easting | (#PCDATA)> |
| <!ELEMENT | geo_info | (coord_sys, datum, format? Zone?)> |
| <!ELEMENT | coord_sys | (#PCDATA) |
| <!ELEMENT | datum | (#PCDATA) |
| <!ELEMENT | format | (#PCDATA)> |
| <!ELEMENT | lat | (#PCDATA)> |
| <!ELEMENT | ll_point | (lat, long)> |
| <!ELEMENT | long | (#PCDATA)> |
| <!ELEMENT | northing | (#PCDATA)> |
| <!ELEMENT | utm_point | (easting, northing, zone, zone_des)> |
| <!ELEMENT | speed | (#PCDATA)> |
| <!ELEMENT | x | (#PCDATA)> |
| <!ELEMENT | xy_point | (x, y)> |
| <!ELEMENT | y | (#PCDATA)> |
| <!ELEMENT | zone | (#PCDATA)> |
| <!ELEMENT | zone_des | (#PCDATA)> |
| <!ELEMENT | lev_conf | (#PCDATA)> |

### Shape Element Definitions

| <!-- MLP_SHAPE --> |  |  |
| --- | --- | --- |
| <!ELEMENT | shape | (point ¦ circle ¦ circ_arc ¦ ellipse ¦ polygon )> |
| <!ELEMENT | point | (ll_point ¦ utm_point ¦ xy_point)> |
| <!ELEMENT | angle | (#PCDATA)> |
| <!ELEMENT | circ_arc | (point, in_rad, out_rad, start_angle, stop_angle)> |
| <!ELEMENT | circle | (point, rad)> |
| <!ELEMENT | ellipse | (point, angle, semi_major, semi_minor)> |
| <!ELEMENT | in_rad | (#PCDATA)> |
| <!ELEMENT | out_rad | (#PCDATA)> |
| <!ELEMENT | polygon | (point+)> |
| <!ELEMENT | rad | (#PCDATA)> |
| <!ELEMENT | semi_major | (#PCDATA)> |
| <!ELEMENT | semi_minor | (#PCDATA)> |
| <!ELEMENT | start_angle | (#PCDATA)> |
| <!ELEMENT | stop_angle | (#PCDATA)> |

### Quality of Position Element Definitions

| <!-- MLP_QoP --> |  |  |
| --- | --- | --- |
| <!ELEMENT | eqop | (resp_req?, resp_timer?, (ll_acc ¦ hor_acc)?, alt_acc?, max_loc_age?)> |
| <!ELEMENT | qop | ((ll_acc ¦ hor_acc)?, alt_acc?)> |
| <!ELEMENT | ll_acc | (#PCDATA)> |
| <!ELEMENT | hor_acc | (#PCDATA)> |
| <!ELEMENT | resp_req | EMPTY> |
| <!ATTLIST | resp_req |  |
|  | type (NO_DELAY ¦ LOW_DELAY ¦ DELAY_TOL) | "DELAY_TOL"> |
| <!ELEMENT | resp_timer | (#PCDATA)> |

### Network Parameters Element Definitions

| <!-- MLP_GSM_NET --> |  |  |
| --- | --- | --- |
| <!ELEMENT | gsm_net_param | (cgi?, neid?, nmr?, ta?)> |
| <!ELEMENT | cgi | (mcc, mnc, lac, cellid)> |
| <!ELEMENT | neid | (vmscid ¦ vlrid)> |
| <!ELEMENT | vmscid | (cc, ndc, vmscno)> |
| <!ELEMENT | vlrid | (cc, ndc, vlrno)> |
| <!ELEMENT | nmr | (#PCDATA)> |
| <!ELEMENT | mcc | (#PCDATA)> |
| <!ELEMENT | ndc | (#PCDATA)> |
| <!ELEMENT | cc | (#PCDATA)> |
| <!ELEMENT | vmscno | (#PCDATA)> |
| <!ELEMENT | vlrno | (#PCDATA)> |
| <!ELEMENT | lac | (#PCDATA)> |
| <!ELEMENT | cellid | (#PCDATA)> |
| <!ELEMENT | ta | (#PCDATA)> |

### Context Element Definitions

| <!-- MLP_CTXT --> |  |  |
| --- | --- | --- |
| <!ELEMENT | client | (id, pwd?, serviceid? servicetype?)> |
| <!ELEMENT | sessionid | (#PCDATA)> |
| <!ELEMENT | id | (#PCDATA)> |
| <!ELEMENT | originator | (id, serviceid?)> |
| <!ELEMENT | pwd | (#PCDATA)> |
| <!ELEMENT | serviceid | (#PCDATA)> |
| <!ELEMENT | servicetype | EMPTY> |
| <!ATTLIST | servicetype |  |
|  | type (ACTIVE ¦ PASSIVE) | "PASSIVE"> |
| <!ELEMENT | subclient | (id, pwd?, serviceid?)> |
| <!ATTLIST | subsclient |  |
|  | last_client (YES ¦ NO) | "NO"> |

## Request Header Components

The service has two main parts, namely a context or header part and a body part.  The body part is described in the sections 4.4 - 4.8.  The context or header part consists of the information that identifies the client as defined in this section.

The SUBCLIENT elements, if present, identify the ASPs, resellers and portals in the chain of service providers between the network and the end-user.  The distinction between CLIENT and SUBCLIENT elements is that the CLIENT element identifies the provider of the service that the network has the initial relationship with, whereas the SUBCLIENT elements identify the chain of other service providers up to the end-user.  The final service provider in the chain is identified as such. On the other hand ORIGINATOR is indicating the initiator of the location request, so in this context besides an ASP it could also be an MS subscriber who is asking the position of another target MS. The identity of the ORIGINATOR may be an MSISDN or any other identifier identifying the initiator of the location request.

### Context DTD

| <!-- MLP_HDR --> |  |  |  |
| --- | --- | --- | --- |
| <!ELEMENT | hdr | ((client ¦ sessionid), subclient*, originator?)> |  |
| <!ATTLIST | hdr |  |  |
|  | ver CDATA | #FIXED "1.0"> |  |
| <!ENTITY | % mlp_ctxt.dtd | SYSTEM "MLP_CTXT_200.DTD"> |  |
| %mlp_ctxt.dtd; |  |  |  |

#### Example (ASP as Originator)

<?xml version=“1.0” ?>

<!DOCTYPE hdr SYSTEM "MLP_HDR_200.DTD">

<hdr ver="2.0.0">

<client>

<id>theasp</id>

<pwd>thepwd</pwd>

<serviceid>0005</serviceid>

<servicetype type="PASSIVE"/>

</client>

<originator>

<id>theoriginalasp</id>

<serviceid>0003</serviceid>

</originator>

<subclient last_client="YES">

<id>thelastasp</id>

<serviceid>0007</serviceid>

</subclient>

</hdr>

#### Example (MS as Originator)

<?xml version=“1.0” ?>

<!DOCTYPE hdr SYSTEM "MLP_HDR_200.DTD">

<hdr ver="2.0.0">

<client>

<id>theasp</id>

<pwd>thepwd</pwd>

<serviceid>0005</serviceid>

<servicetype type="ACTIVE"/>

</client>

<originator>

<id>461018765710</id>

</originator>

</hdr>

## Standard Location Immediate Service

This is a standard service for requesting the location of one or more Mobile Subscribers. The service is used when a single location response is required immediately (within a set time). An immediate request should be used when the LCS Client wants to receive the answer to the request over a persistent connection.

When a lot of positioning reports are requested, it may take an unacceptably long time to get the all responses from the network. If the Location Server supports it the LCS Client can how to receive the responses, either at a time using a persistent connection, or individually using one or more connections initiated by the Location Server.

The extended service supports a number of different formats for describing the location of the mobile subscriber. It has also support for requesting a certain Quality of Service, Type of location and priority.

The service consists of the following messages:

- Standard Location Immediate Request

- Standard Location Immediate Answer

- Standard Location Immediate Report

The following HTTP message flow encapsulates this service:

### Standard Location Immediate Request DTD

| <!--MLP_SLIR--> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY | % extension |  | ""> |
| <!ENTITY | % extension.param |  | ""> |
| <!ELEMENT | slir |  | (client, (msids ¦ (msid, gsm_net_param)+), eqop?, geo_info?, loc_type?, prio?, pushaddr? %extension.param;)> |
| <!ATTLIST | slir |  |  |
|  | ver CDATA |  | #FIXED "1.0" |
|  | res_types (PERSISTENT ¦ PUSH) |  | "PERSISTENT"> |
| <!ENTITY | % mlp_loc.dtd |  | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_id.dtd |  | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| <!ENTITY | % mlp_qop.dtd |  | SYSTEM "MLP_QoP_200.DTD"> |
| <!ENTITY | % mlp_gsm_net_param.dtd |  | SYSTEM "MLP_GSM_NET_PARAM_200.DTD"> |
| %mlp_loc.dtd; |  |  |  |
| %mlp_id.dtd; |  |  |  |
| %mlp_func.dtd; |  |  |  |
| %mlp_qop.dtd; |  |  |  |
| %mlp_gsm_net_param.dtd |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE slir SYSTEM "MLP_SLIR_200.DTD"[

<!ENTITY % extension SYSTEM "http://truckco.com/some_MLP_extension.dtd">

%extension;

]>

<slir ver="2.0.0" res_type="PERSISTENT">

<msids>

<msid type="IPV4">93.10.0.250</msid>

<msid_range>

<start_msid>461018765710</start_msid>

<stop_msid>461018765712</stop_msid>

</msid_range>

</msids>

<eqop>

<resp_req type="LOW_DELAY" />

<hor_acc>1000</hor_acc>

</eqop>

<geo_info>

<format>IDMS3</format>

</geo_info>

<loc_type type="CURRENT_OR_LAST" />

<prio type="HIGH" />

<truckco_send_sms>

You have been positioned by The Truck Company

</truckco_send_sms>

</slir>

### Standard Location Immediate Answer DTD

| <!-- MLP_SLIA --> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY |  | % extension | ""> |
| <!ENTITY |  | % extension.param | ""> |
| <!ELEMENT |  | slia | ((pos+ ¦ req_id), result, add_info? %extension.param;)> |
| <!ATTLIST |  | slia |  |
|  |  | ver CDATA | #FIXED "1.0"> |
|  |  | res_type (PERSISTENT ¦ PUSH) | "PERSISTENT"> |
| <!ENTITY |  | % mlp_shape.dtd | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY |  | % mlp_loc.dtd | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY |  | % mlp_id.dtd | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY |  | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |  |
| %mlp_loc.dtd; |  |  |  |
| %mlp_id.dtd; |  |  |  |
| %mlp_func.dtd; |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE slia SYSTEM "MLP_SLIA_200.DTD">

<slia ver="2.0.0" res_type= "PERSISTENT">

<pos>

<msid>461011334411</msid>

<pd>

<time utc_off="+0200">20000623134453</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301628.312</lat>

<long>451533.431</long>

</ll_point>

</point>

<rad>240</rad>

</circle>

</shape>

</pd>

</pos>

<pos>

<msid>461018765710</msid>

<pd>

<time utc_off="+0300">20000623134454</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301228.302</lat>

<long>865633.863</long>

</ll_point>

</point>

<rad>570</rad>

</circle>

</shape>

</pd>

</pos>

<pos>

<msid>461018765711</msid>

<pd>

<time utc_off="+0300">20000623110205</time>

<shape>

<circle>

<point>

<ll_point>

<lat>781234.322</lat>

<long>762162.823</long>

</ll_point>

</point>

<rad>15</rad>

</circle>

</shape>

</pd>

</pos>

<pos>

<msid>461018765712</msid>

<poserr>

<result resid="10">QOP NOT ATTAINABLE</result>

<time>20000623134454</time>

</poserr>

</pos>

<result resid="0">OK</result>

</slia>

### Standard Location Immediate Report DTD

| <!-- MLP_SLIREP --> |  |  |
| --- | --- | --- |
| <!ENTITY | % extension | ""> |
| <!ENTITY | % extension.param | ""> |
| <!ELEMENT | slirep | (req_id, pos+, utc_off, result, add_info? %extension.param;)> |
| <!ATTLIST | slirep |  |
|  | ver CDATA | #FIXED "1.0"> |
| <!ENTITY | % mlp_shape.dtd | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY | % mlp_loc.dtd | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_id.dtd | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |
| %mlp_loc.dtd; |  |  |
| %mlp_id.dtd; |  |  |
| %mlp_func.dtd; |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE slirep SYSTEM "MLP_SLIREP_200.DTD">

<slirep ver="2.0.0">

<req_id>25267</req_id>

<pos>

<msid type="IPV6">10:A1:45::23:B7:89</msid>

<pd>

<time utc_off="+0300">20000813010423</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301628.312</lat>

<long>451533.431</long>

</ll_point>

</point>

<rad>15</rad>

</circle>

</shape>

</pd>

</pos>

<result resid="0">OK</result>

</slirep>

## Emergency Location Immediate Service

The emergency location immediate service is used to retrieve the position of a mobile subscriber that is involved in an emergency call or have initiated an emergency service in some other way.

The service consists of the following messages:

- Emergency Location Immediate Request

- Emergency Location Immediate Answer

The following HTTP message flow encapsulates this service:

### Emergency Location Immediate Request DTD

| <!-- MLP_EME_LIR --> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY | % extension |  | ""> |
| <!ENTITY | % extension.param |  | ""> |
| <!ELEMENT | eme_lir |  | ((msids ¦ (msid, gsm_net_param)+),, qop?, geo_info?, loc_type? %extension.param;)> |
| <!ATTLIST | eme_lir |  |  |
|  | ver CDATA |  | #FIXED "1.0"> |
| <!ENTITY | % mlp_id.dtd |  | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_loc.dtd |  | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| <!ENTITY | % mlp_qop.dtd |  | SYSTEM "MLP_QoP_200.DTD"> |
| <!ENTITY | % mlp_gsm_net_param.dtd |  | SYSTEM "MLP_GSM_NET_PARAM_200.DTD"> |
| %mlp_id.dtd; |  |  |  |
| %mlp_loc.dtd; |  |  |  |
| %mlp_func.dtd; |  |  |  |
| %mlp_qop.dtd; |  |  |  |
| %mlp_gsm_net_param.dtd; |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE eme_lir SYSTEM "MLP_EME_LIR_200.DTD">

<eme_lir ver="2.0.0">

<msids>

<msid type="EME_MSID">520002-51-431172-6-06</msid>

<esrk>7839298236</esrk>

</msids>

<geo_info>

<format>IDMS3</format>

</geo_info>

<loc_type type="CURRENT_OR_LAST" />

</eme_lir>

### Emergency Location Immediate Answer DTD

| <!-- MLP_EME_LIA --> |  |  |
| --- | --- | --- |
| <!ENTITY | % extension | ""> |
| <!ENTITY | % extension.param | ""> |
| <!ELEMENT | eme_lia | (eme_pos+, result, add_info? %extension.param;)> |
| <!ATTLIST | eme_lia |  |
|  | ver CDATA | #FIXED "1.0"> |
| <!ENTITY | % mlp_shape.dtd | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY | % mlp_id.dtd | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_loc.dtd | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |
| %mlp_id.dtd; |  |  |
| %mlp_loc.dtd; |  |  |
| %mlp_func.dtd; |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE eme_lia SYSTEM "MLP_EME_LIA_200.DTD">

<eme_lia ver="2.0.0">

<eme_pos>

<msid type="EME_MSID">520002-51-431172-6-06</msid>

<esrk>7839298236</esrk>

<pd>

<time utc_off="+0300">20000623134453</time>

<shape>

<circle>

<point>

<ll_point>

<lat>N301628.312</lat>

<long>W451533.431</long>

</ll_point>

</point>

<rad>20</rad>

</circle>

</shape>

</pd>

</eme_pos>

<result resid="0">OK</result>

</eme_lia>

## Standard Location Reporting Service

When a mobile subscriber wants an LCS client to receive the MS location a standard location report is generated. The LCS Client that the location report should be sent to is specified by MS or defined within the Location Server.

The service consists of the following message:

- Standard Location Report

The following HTTP message flow encapsulates this service:

LCS Client

Location Server

HTTP POST(Standard Location Report)

HTTP Response

### Standard Location Report DTD

| <!-- MLP_SLREP --> |  |  |  |  |
| --- | --- | --- | --- | --- |
| <!ENTITY |  | % extension |  | ""> |
| <!ENTITY |  | % extension.param |  | ""> |
| <!ELEMENT |  | slrep |  | (result, pos+, add_info? %extension.param;)> |
| <!ATTLIST |  | slrep |  |  |
|  |  | ver CDATA |  | #FIXED "1.0"> |
| <!ENTITY |  | % mlp_shape.dtd |  | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY |  | % mlp_loc.dtd |  | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY |  | % mlp_id.dtd |  | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY |  | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |  |  |
| %mlp_loc.dtd; |  |  |  |  |
| %mlp_id.dtd; |  |  |  |  |
| %mlp_func.dtd; |  |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE slrep SYSTEM "MLP_SLREP_200.DTD">

<slrep ver="2.0.0">

<result resid="0">OK</result>

<pos>

<msid>461011678298</msid>

<pd>

<time>20000813010423</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301628.312</lat>

<long>451533.431</long>

</ll_point>

</point>

<rad>15</rad>

</circle>

</shape>

</pd>

</pos>

</slrep>

## Emergency Location Reporting Service

If the wireless network spontaneously initiates a positioning when a user initiates or releases an emergency call, an emergency location report is generated. The application(s) that the emergency location report should be sent to is defined within the location server. Data as required geographical format and address to application is also defined within the location server.

This is the only case where location server initiate the dialogue instead of LCS client.

The service consists of the following message:

- Emergency Location Report

The following HTTP message flow encapsulates this service:

### Emergency Location Report DTD

| <!-- MLP_EMEREP --> |  |  |  |  |
| --- | --- | --- | --- | --- |
| <!ENTITY |  | % extension |  | ""> |
| <!ENTITY |  | % extension.param |  | ""> |
| <!ELEMENT |  | emerep |  | (result, eme_event, add_info? %extension.param;)> |
| <!ATTLIST |  | emerep |  |  |
|  |  | ver CDATA |  | #FIXED "1.0"> |
| <!ENTITY |  | % mlp_shape.dtd |  | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY |  | % mlp_loc.dtd |  | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY |  | % mlp_id.dtd |  | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY |  | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |  |  |
| %mlp_loc.dtd; |  |  |  |  |
| %mlp_id.dtd; |  |  |  |  |
| %mlp_func.dtd; |  |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE emerep SYSTEM "MLP_EMEREP_200.DTD">

<emerep ver="2.0.0">

<id>LocApplID</id>

<pwd>LocApplPW</pwd>

<result resid="0">OK</result>

<eme_event eme_trigger="EME_ORG">

<eme_pos>

<msid>461011678298</msid>

<pd>

<time utc_off="+0300">20000623010003</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301628.312</lat>

<long>451533.431</long>

</ll_point>

</point>

<rad>15</rad>

</circle>

</shape>

</pd>

</eme_pos>

</eme_event>

</emerep>

## Triggered Location Reporting Service

The triggered location reporting service is used when an application wants the position of the a list of MS to be tracked. The triggers could be:

- The periodicity time defined by an interval

- An MS action, defined as the event "UE available" in 3GPP TS 23.271 rel. 4 [ref. 11].

The report will be triggered when one of the pre-defined MS’s action is occurred or the time interval elapses.

The service consists of the following messages:

- Triggered Location Reporting Request

- Triggered Location Reporting Answer

- Triggered Location Report

- Triggered Location Reporting Stop

- Triggered Location Reporting Stop Answer

Note:It is the intention that Triggered services will support entering or leaving an area in future releases.  An area may be defined as a specified geographical area, a city or locale, a country or a network. Other triggers that may be supported are specific events not yet defined, such a subscriber being  in proximity to a friend in a FriendFinder application. Other events are FFS within 3GPP and are targeted for rel.5.

The following HTTP message flow encapsulates this service:

### Triggered Location Reporting Request DTD

| <!-- MLP_TLRR --> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY | % extension |  | ""> |
| <!ENTITY | % extension.param |  | ""> |
| <!ELEMENT | tlrr |  | (msids, interval, start_time?, stop_time?, tlrr_event?, qop?, geo_info?, pushaddr?, loc_type?, prio? %extension.param;)> |
| <!ATTLIST | tlrr |  |  |
|  | ver CDATA |  | #FIXED "1.0"> |
| <!ENTITY | % mlp_id.dtd |  | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_loc.dtd |  | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| <!ENTITY | % mlp_qop.dtd |  | SYSTEM "MLP_QoP_200.DTD"> |
| %mlp_id.dtd; |  |  |  |
| %mlp_loc.dtd; |  |  |  |
| %mlp_func.dtd; |  |  |  |
| %mlp_qop.dtd; |  |  |  |

The following rules apply to the use of ‘start_time’ and stop_time’:

- If no START_TIME is specified reporting starts immediately.

- If no STOP_TIME is specified the reporting will occur until explicitly canceled with ‘Triggered Location Stop Request’ or a time out occurs (depending on system configuration).

- If START_TIME is ‘older’ than current time the Location Server MUST reject the request with an error indication ‘110’ to the client.

- If STOP_TIME is ‘older’ than current time the Location Server MUST reject the request with an error indication ‘110’ to the client.

- If STOP_TIME is earlier than START_TIME the implementation MUST reject the request with an error indication ‘110’ to the client.

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE tlrr SYSTEM "MLP_TLRR_200.DTD">

<tlrr ver="2.0.0">

<msids>

<msid>461011678298</msid>

</msids>

<interval>00003000</interval>

<start_time offset="+0300">20011003112700</start_time>

<stop_time offset="+0300">20011003152700</stop_time>

<qop>

<hor_acc>100</hor_acc>

</qop>

<geo_infocoord_sys="LL" datum="WGS-84">

<format>IDMS3</format>

</geo_info>

<pushaddr>

<url>http://location.application.com</url>

<id>LocApplID</id>

<pwd>LocApplPW</pwd>

</pushaddr>

<loc_type type="CURRENT" />

<prio type="HIGH" />

</tlrr>

### Triggered Location Reporting Answer DTD

| <!-- MLP_TLRA --> |  |  |
| --- | --- | --- |
| <!ENTITY | % extension | ""> |
| <!ENTITY | % extension.param | ""> |
| <!ELEMENT | tlra | (result, req_id?, add_info? %extension.param;)> |
| <!ATTLIST | tlra |  |
|  | ver CDATA | #FIXED "1.0"> |
| <!ENTITY | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_func.dtd; |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE plra SYSTEM "MLP_TLRA_200.DTD">

<tlra ver="2.0.0">

<result resid="0">OK</result>

<req_id>25293</req_id>

</tlra>

### Triggered Location Report DTD

| <!-- MLP_TLREP --> |  |  |
| --- | --- | --- |
| <!ENTITY | % extension | ""> |
| <!ENTITY | % extension.param | ""> |
| <!ELEMENT | tlrep | (req_id, (pos + trl pos)+, time_remaining?, result, add_info? %extension.param;)> |
| <!ATTLIST | tlrep |  |
|  | ver CDATA | #FIXED "1.0"> |
| <!ENTITY | % mlp_shape.dtd | SYSTEM "MLP_SHAPE_200.DTD"> |
| <!ENTITY | % mlp_loc.dtd | SYSTEM "MLP_LOC_200.DTD"> |
| <!ENTITY | % mlp_id.dtd | SYSTEM "MLP_ID_200.DTD"> |
| <!ENTITY | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_shape.dtd; |  |  |
| %mlp_loc.dtd; |  |  |
| %mlp_id.dtd; |  |  |
| %mlp_func.dtd; |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE tlrep SYSTEM "MLP_TLREP_200.DTD">

<plrep ver="2.0.0">

<req_id>25267</req_id>

<id>ServerID</id>

<pwd>ServerPW</pwd>

<trl pos trl_trigger=”TIMER”>

<msid>461011678298</msid>

<pd>

<time utc_off="+0300">20000813010423</time>

<shape>

<circle>

<point>

<ll_point>

<lat>301628.312</lat>

<long>451533.431</long>

</ll_point>

</point>

<rad>15</rad>

</circle>

</shape>

</pd>

</pos>

<time_remaining>00010000/time_remaining>

<result resid="0">OK</result>

</tlrep>

### Triggered Location Reporting Stop Request DTD

| <!-- MLP_TLRSR --> |  |  |  |
| --- | --- | --- | --- |
| <!ENTITY | % extension |  | ""> |
| <!ENTITY | % extension.param |  | ""> |
| <!ELEMENT | tlrsr |  | (req_id %extension.param;)> |
| <!ATTLIST | tlrsr |  |  |
|  | ver CDATA |  | #FIXED "1.0"> |
| <!ENTITY | % mlp_func.dtd |  | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_func.dtd; |  |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE plrsr SYSTEM "MLP_TLRSR_200.DTD">

<tlrsr ver="2.0.0">

<req_id>25293</req_id>

</tlrsr>

### Triggered Location Reporting Stop Answer DTD

| <!-- MLP_TLRSA --> |  |  |
| --- | --- | --- |
| <!ENTITY | % extension | ""> |
| <!ENTITY | % extension.param | ""> |
| <!ELEMENT | tlrsa | (req_id, result, add_info? %extension, param;)> |
| <!ATTLIST | tlrsa |  |
|  | ver CDATA | #FIXED "1.0"> |
| <!ENTITY | % mlp_func.dtd | SYSTEM "MLP_FUNC_200.DTD"> |
| %mlp_func.dtd; |  |  |

#### Example

<?xml version=“1.0” ?>

<!DOCTYPE plsra SYSTEM "MLP_TLRSA_200.DTD">

<tlrsa ver="2.0.0">

<req_id>25293</req_id>

<result resid="0">OK</result>

</tlrsa>

# Elements and attributes in DTD

## add_info

| Description: |  |
| --- | --- |
| A text string containing additional information about a certain result. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <add_info>EVENT</add_info> |
| Note: - |  |

## alt

| Description: |  |
| --- | --- |
| The altitude of the MS in meters in respect of the ellipsoid which is used to be define the coordinates |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [+¦-] [0-9]+ |
| Default value: | - |
| Example: | <alt>1200</alt> |
| Note: This element is present if altitude is possible to attain by the used positioning method. |  |

## alt_acc

| Description: |  |
| --- | --- |
| Accuracy of altitude in meters |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <alt_acc>200</alt_acc> |
| Note: - |  |

## angle

| Description: |  |
| --- | --- |
| Specifies the angle of rotation of an ellipse measured clockwise from north. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 0-360 |
| Default value: | - |
| Example: | <angle>60</angle> |
| Note: - |  |

## cc

| Description: |  |
| --- | --- |
| Specifies the country code. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 2 digits e.g. 44 for UK |
| Default value: | - |
| Example: | <cc>44</cc> |
| Note: This element is present if direction is possible to attain by the used positioning method. |  |

## cellid

| Description: |  |
| --- | --- |
| Identifies the Cell Identity |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 0-65535 |
| Default value: | - |
| Example: | <cellid>546</cellid> |
| Note: This element is present if direction is possible to attain by the used positioning method. |  |

## coord_sys

| Description: |  |  |
| --- | --- | --- |
| Specifies which coordinate system that should be used in the position answer |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | LL | Longitude Latitude |
|  | UTM | Universal Transverse Mercator |
| Default value: |  |  |
| Example: | <coord_sys>"UTM"</coord_sys> |  |
| Note: - |  |  |

## direction

| Description: |  |
| --- | --- |
| Specifies the direction, in degrees, that a positioned MS is moving in. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 0-360 |
| Default value: | - |
| Example: | <direction>120</direction> |
| Note: This element is present if direction is possible to attain by the used positioning method. |  |

## datum

| Description: |  |  |
| --- | --- | --- |
| Specifies a geodetic datum |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | WGS-84 | World Geodetic System 1984 |
| Default value: | - |  |
| Example: | <datum>"WGS-84"</datum> |  |
| Note: WGS-84 is the only mandatory datum supported in all implementations , however regional and implementation specific defined elements (such as BESSEL-1841) may additionally be supported. |  |  |

## easting

| Description: |  |
| --- | --- |
| Used in the UTM coordinate system. Eastings are measured from central meridian. The number of decimals provided in the response is defined by UTM_FORMAT that is provided in the request. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <easting>621160.98</easting> (if UTM_FORMAT=2) |
| Note: - |  |

## eme_event

| Description: |  |
| --- | --- |
| Specifies the events that initiated the positioning of the MS at an emergency call. |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <eme_event eme_trigger="EME_ORG"> |
| Note: - |  |

### eme_trigger

| Description: |  |  |
| --- | --- | --- |
| Specifies the trigger that initiated the positioning of the MS at an emergency call. |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | EME_ORG | An emergency service user originated an emergency call |
|  | EME_REL | An emergency service user released an emergency call |
| Default value: | - |  |
| Example: | <eme_event eme_trigger="EME_ORG"> |  |
| Note: - |  |  |

## esrd

| Description: |  |
| --- | --- |
| This element specifies Emergency Services Routing Digits (ESRD). |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <esrd>761287612582</esrd> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the origin of the ESRD |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | NA | Indicates that the ERSD is defined as the North American ESRD (NA-ERSD). NA-ESRD is a telephone number in the North American Numbering Plan that can be used to identify a North American emergency services provider and it’s associated Location Services client. The NA-ESRD also identifies the base station, cell site or sector from which a North American emergency call originates |
| Default value: | NA |  |
| Example: | <esrd type="NA">12345678</ersd> |  |
| Note: Currently only NA is specified. It is expected that other origins will be specified in the future |  |  |

## esrk

| Description: |  |
| --- | --- |
| This element specifies the Services Routing Key (ESRK). |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <esrk>928273633343</esrk> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the origin of the ESRK |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | NA | Indicates that the ERSK is defined as the North American ESRK (NA-ERSK). NA-ESRK is a telephone number in the North American Numbering Plan that is assigned to an emergency services call for the duration of the call. The NA-ESRK is used to identify (e.g. route to) both the emergency services provider and the switch currently serving the emergency caller. During the lifetime of an emergency services call, the NA-ESRK also identifies the calling subscriber. |
| Default value: | NA |  |
| Example: | <esrk type="NA">12345678</ersk> |  |
| Note: Currently only NA is specified. It is expected that other origins will be specified in the future |  |  |

## format

| Description: |  |  |  |
| --- | --- | --- | --- |
| With type="LL" This element specifies the output format of the geographical position when it is expressed in longitude and latitude. |  |  |  |
| Type: | Element |  |  |
| Format: | Char String The LL format is expressed as: [I]?[D ¦ DM ¦ DMS ¦ M ¦ MS ¦ S] [0-9] [I]? Detailed description: |  |  |
|  | String | Requested output | Example of output |
|  | D | Degrees only | 45.403 |
|  | DM | Degrees and minutes | 4515.557 |
|  | DMS | Degrees, minutes and seconds | 451533.431 |
|  | M | Minutes only | 16215.557 |
|  | MS | Minutes and seconds | 1621533.431 |
|  | S | Seconds only | 972933.431 |
|  | I | Output direction indicator The indicator has two valid positions, at the beginning or at the end of the string. If present a direction indicator (N ¦ S ¦ E ¦ W) will be added to the output. | 45.403W N16215.557 |
|  | The digit [0-9] defines the decimal precision of the output. Any trailing zero valued decimals will be deleted. |  |  |
| Defined values: | - |  |  |
| Default value: | DMS3 |  |  |
| Example: | <format type="LL">DM2I</format> |  |  |
| Note: - |  |  |  |

| Description: |  |
| --- | --- |
| With type="UTM" Specifies the output format for UTM coordinates. The digit specifies the number of decimals that should be used in the output. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9] |
| Default value: | 3 |
| Example: | <format type="UTM">5</format> |
| Note: - |  |

| Description: |  |
| --- | --- |
| With type="XY" Specifies the output format for coordinates expressed in X and Y. The digit specifies the number of decimals that should be used in the output. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9] |
| Default value: | 3 |
| Example: | <format type="XY">2</format> |
| Note: - |  |

## geo_info

| Description: |  |
| --- | --- |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <geo_info coord_sys="UTM" datum="WGS-84"> |
| Note: - |  |

## hor_acc

| Description: |  |
| --- | --- |
| Requested horizontal accuracy in meters |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <hor_acc>200</hor_acc> |
| Note: - |  |

## id

| Description: |  |
| --- | --- |
| A string defining the name of a registered user performing a location request. In an answer the string represents the name of a location server. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <id>TheTruckCompany</id> |
| Note: - This element is implementation specific. The second example illustrates an MSISDN number. |  |

## in_rad

| Description: |  |
| --- | --- |
| The inner radius of an arc in meters |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <in_rad>100</in_rad> |
| Note: If the inner radius is 0 (zero) the area described represents a circle sector. |  |

## interval

| Description: |  |  |
| --- | --- | --- |
| Specifies the interval between two responses in case of an LDR that indicates timer controlled, periodic responses. |  |  |
| Type: | Element |  |
| Format: | Char string The interval is expressed as ddhhmmss where: |  |
|  | String | Description |
|  | dd | Number of days between responses |
|  | hh | Number of hours between responses |
|  | mm | Number of minutes between responses |
|  | ss | Number of seconds between responses |
| Defined values: | - |  |
| Default value: | - |  |
| Example: | <interval>00010000</interval> |  |
| Note: - |  |  |

## lac

| Description: |  |
| --- | --- |
| Identifies the Location Area Code |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 1-65535 |
| Default value: | - |
| Example: | <lac>234</lac> |
| Note: |  |

## lat

| Description: |  |
| --- | --- |
| Specifies the geodetic latitude of a point is the angle from the equatorial plane to the vertical direction of a line normal to the reference ellipsoid. |  |
| Type: | Element |
| Format: | Char string The format is determined by the value of the LL_FORMAT element in the request. |
| Defined values: | - |
| Default value: | - |
| Example: | <lat>N301628.3</lat> (if LL_FORMAT=IDMS1) |
| Note: - |  |

## lev_conf

| Description: |  |
| --- | --- |
| This parameter indicates the probability in percent that the MS is located in the position area that is returned. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 0-100 |
| Default value: | - |
| Example: | <lev_conf>80</lev_conf> |
| Note: - |  |

## ll_acc

| Description: |  |
| --- | --- |
| Longitude and latitude accuracy in seconds. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | - |
| Default value: | - |
| Example: | <ll_acc>7.5</ll_acc> |
| Note: - |  |

## loc_type

| Description: |  |
| --- | --- |
| Defines the type of location requested. |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <loc_type type="INITIAL" /> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the type of location requested |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | CURRENT | After a location attempt has successfully delivered a location estimate the location estimate is known as the current location at that point in time. |
|  | LAST | The current location estimate is generally stored in the network until replaced by a later location estimate and is known as the last known location. The last known location may be distinct from the initial location., i.e. more recent. |
|  | INITIAL | In an originating emergency call, this is the location estimate at the commencement of the call set-up and is known as the initial location. |
| Default value: | CURRENT |  |
| Example: | <loc_type type="INITIAL" /> |  |
| Note: - |  |  |

## long

| Description: |  |
| --- | --- |
| Specifies the longitude of a point is the angle between a reference plane and a plane passing through the point, both planes being perpendicular to the equatorial plane. |  |
| Type: | Element |
| Format: | Char string The format is determined by the value of the LL_FORMAT element in the request. |
| Defined values: | - |
| Default value: | - |
| Example: | <long>W974425.2</long> (if LL_FORMAT=IDMS1) |
| Note: - |  |

## max_loc_age

| Description: |  |
| --- | --- |
| This states the maximum allowable age in seconds of a location sent in a response to a location request. |  |
| Type: | Element |
| Format: | Char string representing seconds |
| Defined values: | Maximum number of seconds (must be >= 0) |
| Default value: | Implementation specific. |
| Example: | <max_loc_age>3600</max_loc_age> |
| Note: - |  |

## mcc

| Description: |  |
| --- | --- |
| Specifies the mobile country code (MCC). |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | 3 digits, e.g. 234 for the UK |
| Default value: | - |
| Example: | <mcc>234</mcc> |
| Note: |  |

## mnc

| Description: |  |
| --- | --- |
| Specifies the mobile network code. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | Up to 3 digits e.g. 15 for Vodafone |
| Default value: | - |
| Example: | <mnc>215</mnc> |
| Note: - |  |

## ms_action

| Description: |  |
| --- | --- |
| Specifies the trigger that initiated the positioning of the MS. |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <ms_action type="MO" /> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Specifies the trigger that initiated the positioning of the MS. |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | MS_AVAIL | The positioning is triggered by the MS originating a call The positioning is triggered by the MS available notification when the MS regains radio connection with the network if the connection was previously lost. For more information refer to 3GPP TS 23.271 rel. 4. |
| Default value: | - |  |
| Example: | <ms_action type="MS_AVAIL" /> |  |
| Note: - |  |  |

## msid

| Description: |  |
| --- | --- |
| This element represents an identifier of a mobile subscriber |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <msid>460703057640</msid> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Type of identifier for the mobile subscriber |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | MSISDN | Mobile Station International ISDN Number |
|  | IMSI | International Mobile Subscriber Identity |
|  | IMEI | International Mobile station Equipment Identity |
|  | MIN | Mobile Identification Number |
|  | MDN | Mobile Directory Number |
|  | EME_MSID | Emergency MSID |
|  | IPV4 | Mobile station IP address (Version 4) |
|  | IPV6 | Mobile station IP address (Version 6) |
| Default value: | MSISDN |  |
| Example: | <msid type="IMSI"> |  |
| Note: - |  |  |

### enc

| Description: |  |  |
| --- | --- | --- |
| Type of encoding for MSID identifier for the mobile subscriber |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | ASC | Normal textual format |
|  | B64 | Base 64 encoding |
|  | CRP | Encrypted format: In some countries the Network Operator (where is placed the Location Server) isn't allowed to send to a LCS the private information of an MS like MSISDN. The Network Operator can send out to LCS the Encrypted MSID, since only the Network Operator is the only entity able to decode this information, the LCS will be never able to break the privacy of the MS. |
| Default value: | ASC |  |
| Example: | <msid type="IMSI" enc="B64"> |  |
| Note: - |  |  |

## ndc

| Description: |  |
| --- | --- |
| Specifies the network destination code. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | Up to 4 digits e.g. 7785 for Vodafone |
| Default value: | - |
| Example: | <ndc>215</ndc> |
| Note: - |  |

## nmr

| Description: |  |
| --- | --- |
| Network specific measurement result for the target MS. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | For examples see relevant standards documents. (GSM 04.08 – rel.98 section 10.5.2.20) |
| Default value: | - |
| Example: |  |
| Note: This element remains to be defined. |  |

## northing

| Description: |  |
| --- | --- |
| Used in the UTM coordinate system. Positions are measured in northings from the equator.. The number of decimals provided in the response is defined by UTM_FORMAT that is provided in the request. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <northing>3349893.53</northing> (if UTM_FORMAT=2) |
| Note: - |  |

## out_rad

| Description: |  |
| --- | --- |
| The outer radius of an arc in meters |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <out_rad>850</out_rad> |
| Note: - |  |

## prio

| Description: |  |
| --- | --- |
| Defines the priority of a location request |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <prio /> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the priority of a location request |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | NORMAL | The request is handled with normal priority |
|  | HIGH | The request is handled with high priority |
| Default value: | NORMAL |  |
| Example: | <prio type="HIGH" /> |  |
| Note: - |  |  |

## pwd

| Description: |  |
| --- | --- |
| The password for the registered user performing a location request. In an answer the string represents the password for a location server. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: |  |
| Default value: | - |
| Example: | <pwd>the5pwd</pwd> |
| Note: - |  |

## rad

| Description: |  |
| --- | --- |
| The radius of a circle in meters |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <rad>120</rad> |
| Note: - |  |

## req_id

| Description: |  |
| --- | --- |
| Unique identification of a request |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <req_id>435.23.01</req_id> |
| Note: - |  |

## resp_req

| Description: |  |
| --- | --- |
| This attribute represents response time requirement. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <resp_req type="NO_DELAY" /> |
| Note: - |  |

### type

| Description: |  |  |
| --- | --- | --- |
| This attribute represents response time requirement |  |  |
| Type: | Attribute |  |
| Format: | Char String |  |
| Defined values: | NO_DELAY | No delay: The server should immediately return any location estimate that it currently has. |
|  | LOW_DELAY | Low delay: Fulfilment of the response time requirement takes precedence over fulfilment of the accuracy requirement. |
|  | DELAY_TOL | Delay tolerant: Fulfillment of the accuracy requirement takes precedence over fulfillment of the response time requirement |
| Default value: | DELAY_TOL |  |
| Example: | <resp_req /> |  |
| Note: - |  |  |

## resp_timer

| Description: |  |  |
| --- | --- | --- |
| Defines a timer for the response time within which the current location should be obtained and returned to the LCS Client. |  |  |
| Type: | Element |  |
| Format: | Char String The time is expressed as mmss where: |  |
|  | mm | Minutes |
|  | ss | Seconds |
| Defined values: | - |  |
| Default value: | The default value is defined in the location server |  |
| Example: | <resp_timer>0010</resp_timer> |  |
| Note: - |  |  |

## result

| Description: |  |
| --- | --- |
| A text string indicating the result of the request or an individual positioning |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | See chapter 6.1 |
| Default value: | - |
| Example: | <result resid=0>OK</result> |
| Note: - |  |

### resid

| Description: |  |
| --- | --- |
| This attribute represents a numeric representation of a result message |  |
| Type: | Attribute |
| Format: | Char String |
| Defined values: | [0-9]+ See chapter 6.1 |
| Default value: | - |
| Example: | <result resid=0>OK</result> |
| Note: - |  |

## semi_major

| Description: |  |
| --- | --- |
| Specifies the length of the semi-major axis of an ellipse in meters. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <semi_major>560</semi_major> |
| Note: - |  |

## semi_minor

| Description: |  |
| --- | --- |
| Specifies the length of the semi-minor axis of an ellipse in meters. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <semi_minor>560</semi_minor> |
| Note: - |  |

## serviceid

| Description: |  |
| --- | --- |
| Specifies an id that is used by an entity to identify the service or application that is accessing the network. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | - |
| Default value: | - |
| Example: | <serviceid>0005</serviceid> |
| Note: |  |

## servicetype

| Description: |  |
| --- | --- |
| Defines the type of the service that has been requested by the ASP. |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <servicetype /> |
| Note: |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the type of the service that has been requested by the ASP |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | PASSIVE | The service is one that is not directly initiated by the user. |
|  | ACTIVE | The service is one that the user is initiating personally. |
| Default value: | PASSIVE |  |
| Example: | <servicetype type="ACTIVE" /> |  |
| Note: the default value is set to PASSIVE, as this is likely to be the one that is most restrictively defined by the user. |  |  |

## session

| Description: |  |
| --- | --- |
| This element identifies should be presented in location request when the LCS Client is making has an active session with the User Equipment, this will be either the number called by the UE or the APN on which the UE established the session. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | - |
| Default value: | - |
| Example: | <session>447073100177</session> |
| Note: This information may be required for privacy validation of the location request by the VMSC, SGSN or MSC server |  |

### type

| Description: |  |  |
| --- | --- | --- |
| Defines the type of the session that is established between the User Equipment and LCS Client |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | APN | Access Point Name. |
|  | dial | The number dialed by the user to access the LCS client. |
| Default value: | - |  |
| Example: | <session type type="dial" /> |  |
| Note: |  |  |

## sessionid

| Description: |  |
| --- | --- |
| Specifies an id that can be used by an entity to replace the need to use an ID and PWD to use the location services. In an answer it indicates the sessionid that the entity can use on subsequent requests. The Session id is a generated alphanumeric string and can be time-limited. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | - |
| Default value: | - |
| Example: | <sessionid>34eg6.876.76h4</sessionid> |
| Note: |  |

## speed

| Description: |  |
| --- | --- |
| The speed of the MS in m/s. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <speed>23</speed> |
| Note: This element is present if speed is possible to attain by the used positioning method. |  |

## start_angle

| Description: |  |
| --- | --- |
| Specifies a start angle in degrees. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | 0-360 |
| Default value: | - |
| Example: | <start_angle>60</start_angle> |
| Note: - |  |

## start_msid

| Description: |  |
| --- | --- |
| This element represents an identifier of a mobile subscriber, which is used as start of a range. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <start_msid>460703057640</start_msid> |
| Note: - |  |

## start_time

| Description: |  |  |
| --- | --- | --- |
| This element defines the absolute start time in a range of times. |  |  |
| Type: | Element |  |
| Format: | Char String The time is expressed as yyyyMMddhhmmss where: |  |
|  | String | Description |
|  | yyyy | Year |
|  | MM | Month |
|  | dd | Day |
|  | hh | Hours |
|  | mm | Minutes |
|  | ss | Seconds |
| Defined values: | - |  |
| Default value: | - |  |
| Example: | <start_time>20010630142810</start_time> |  |
| Note: - |  |  |

### utc_off

| Description: |  |
| --- | --- |
| Specifies the UTC offset in hours and minutes. Positive values indicate time zones east of Greenwich. |  |
| Type: | Attribute |
| Format: | Char string |
| Defined values: | [+¦-]0000-1400 |
| Default value: | - |
| Example: | <start_time utc_off="+0200">20000813010423</start_time> |
| Note: utc_off is specifed as 'HHMM', where 'HH' can range between 0-14 and 'MM' between '0-59'. All other values shall result in error 105, 'Format error'. |  |

## stop_angle

| Description: |  |
| --- | --- |
| Specifies a stop angle in degrees. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | 0-360 |
| Default value: | - |
| Example: | <stop_angle>180</stop_angle> |
| Note: - |  |

## stop_msid

| Description: |  |
| --- | --- |
| This element represents an identifier of a mobile subscriber, which is used as end of a range. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <stop_msid>460703057640</stop_msid> |
| Note: - |  |

## stop_time

| Description: |  |  |
| --- | --- | --- |
| This element defines the absolute stop time in a range of times. |  |  |
| Type: | Element |  |
| Format: | Char String The time is expressed as yyyyMMddhhmmss where: |  |
|  | String | Description |
|  | yyyy | Year |
|  | MM | Month |
|  | dd | Day |
|  | hh | Hours |
|  | mm | Minutes |
|  | ss | Seconds |
| Defined values: | - |  |
| Default value: | - |  |
| Example: | <stop_time>20010630142810</stop_time> |  |
| Note: - |  |  |

### utc_off

| Description: |  |
| --- | --- |
| Specifies the UTC offset in hours and minutes. Positive values indicate time zones east of Greenwich. |  |
| Type: | Attribute |
| Format: | Char string |
| Defined values: | [+¦-]0000-1400 |
| Default value: | - |
| Example: | <stop_time utc_off="+0200">20000813010423</stop_time> |
| Note: utc_off is specifed as 'HHMM', where 'HH' can range between 0-14 and 'MM' between '0-59'. All other values shall result in error 105, 'Format error'. |  |

## subclient

| Description: |  |
| --- | --- |
| Identifies the ASPs, resellers and portals in the chain of service providers between the network and the end-user |  |
| Type: | Element |
| Format: | - |
| Defined values: | - |
| Default value: | - |
| Example: | <subclient last_client="NO"> <id>TheASP</id> <serviceid>0006</serviceid> </subclient> |
| Note: - |  |

### last_client

| Description: |  |  |
| --- | --- | --- |
| Identifies whether the SUBCLIENT is the last one in the chain or not |  |  |
| Type: | Attribute |  |
| Format: | Char String |  |
| Defined values: | YES | This is the last client – the one that the end-user is actually communicating with |
|  | NO | This is not the last client |
| Default value: | NO |  |
| Example: | <subclient last_client="YES"> |  |
| Note: - |  |  |

## ta

| Description: |  |
| --- | --- |
| This Radio Access Network element that can arguably be used to offer enhanced positioning. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | 0-62 |
| Default value: | 0 |
| Example: | <ta></ta> |
| Note: Further Information regarding this element can be found in the relevant GSM Specifications |  |

## time

| Description: |  |  |
| --- | --- | --- |
| This element defines the absolute time when to perform a positioning at a timer controlled LDR. In a location answer this element indicates the time when the positioning was performed. |  |  |
| Type: | Element |  |
| Format: | Char String The time is expressed as yyyyMMddhhmmss where: |  |
|  | String | Description |
|  | yyyy | Year |
|  | MM | Month |
|  | dd | Day |
|  | hh | Hours |
|  | mm | Minutes |
|  | ss | Seconds |
| Defined values: | - |  |
| Default value: | - |  |
| Example: | <time>20010630142810</time> |  |
| Note: - |  |  |

### utc_off

| Description: |  |
| --- | --- |
| Specifies the UTC offset in hours and minutes. Positive values indicate time zones east of Greenwich. |  |
| Type: | Attribute |
| Format: | Char string |
| Defined values: | [+¦-]0000-1400 |
| Default value: | - |
| Example: | <time utc_off="+0200">20000813010423</time> |
| Note: utc_off is specifed as 'HHMM', where 'HH' can range between 0-14 and 'MM' between '0-59'. All other values shall result in error 105, 'Format error'. |  |

## time_remaining

| Description: |  |  |
| --- | --- | --- |
| Defines the time remaining until the location server terminates the current triggered location service. The time for which the service is valid is either specified by the client using start time and stop time, or is a network operator specific default value where no s stop time is defined or where the stop time exceeds the allowed value by the location server involved. |  |  |
| Type: | Element |  |
| Format: | Char String The time is expressed as ddhhmmss where: |  |
|  | String | Description |
|  | dd | Day |
|  | hh | Hours |
|  | mm | Minutes |
|  | ss | Seconds |
| Defined values: | - |  |
| Default value: | The default value is defined in the location server |  |
| Example: | <time_remaining>00010000</time_remaining> |  |
| Note: - |  |  |

## trl_trigger

| Description: |  |  |
| --- | --- | --- |
| Specifies the trigger that initiated the positioning of the MS at a triggered location report. |  |  |
| Format: | Char string |  |
| Defined values: | TIMER | The positioning is triggered by the appointed time |
|  | PERIODIC | The positioning is triggered when the periodical timer expired |
|  | MS_AVAIL | The positioning is triggered by the MS presence notification |
| Default value: | - |  |
| Example: | <MS_ACTION ms_action_type="MS_AVAIL" /> |  |
| Note: - |  |  |

## url

| Description: |  |
| --- | --- |
| Specifies the location to which a response to a LDR should be sent to |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <url>http://location.client.com/Response/</url> |
| Note: - |  |

## vlrno

| Description: |  |
| --- | --- |
| Uniquely specifies a VLR within a network. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | In GSM this is the Global Title address. The Global Title is in the same format as an E.164 number. |
| Default value: | - |
| Example: | <vlrno>1541154871</vlrno> |
| Note: |  |

## vmscno

| Description: |  |
| --- | --- |
| Uniquely specifies a VMSC within a network. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | In GSM this is the Global Title address. The Global Title is in the same format as an E.164 number. |
| Default value: | - |
| Example: | <vmscno>1541154871</vmscno> |
| Note: |  |

## x

| Description: |  |
| --- | --- |
| Used when a position is expressed in X and Y coordinates to represent a position on the X-axis of the coordinate system. The number of decimals provided in the response is defined by FORMAT that is provided in the request. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <x>33498.23</x> (if FORMAT=2) |
| Note: - |  |

## y

| Description: |  |
| --- | --- |
| Used when a position is expressed in X and Y coordinates to represent a position on the Y-axis of the coordinate system. The number of decimals provided in the response is defined by FORMAT that is provided in the request. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <y>33498.23</y> (if FORMAT=2) |
| Note: - |  |

## zone

| Description: |  |
| --- | --- |
| Identifies the zone when the UTM or other planar coordinate system is used. |  |
| Type: | Element |
| Format: | Char String |
| Defined values: | [0-9]+ |
| Default value: | - |
| Example: | <zone>14</zone> |
| Note: - |  |

## zone_des

| Description: |  |
| --- | --- |
| Identifies the zone designator when the UTM coordinate system is used. |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | - |
| Default value: | - |
| Example: | <zone_des>R</zone_des> |
| Note: - |  |

## Service attributes

### res_type

| Description: |  |  |
| --- | --- | --- |
| Defines a response type at the Standard Location Immediate Service |  |  |
| Type: | Attribute |  |
| Format: | Char string |  |
| Defined values: | PERSISTENT | An LCS Client can receive the responses at one time using persistent connection |
|  | PUSH | An LCS Client can receive the responses one by one using some connections initiated by the LCS Server |
| Default value: | PERSISTENT |  |
| Example: | <slir ver="2.0.0" res_type="PERSISTENT"> |  |
| Note: - |  |  |

### ver

| Description: |  |
| --- | --- |
| Defines the version of the location protocol. This attribute is valid for ALL messages |  |
| Type: | Element |
| Format: | Char string |
| Defined values: | [0-9].[0-9] |
| Default value: | - |
| Example: | <slia ver="2.0.0"> |
| Note: - |  |

# Result codes and error codes

## Result codes

This table defines the general result codes that indicate the result of the whole request.

| Result code | Slogan | Description |
| --- | --- | --- |
| 0 | OK | No error occurred while processing the request. |
| 1 | SYSTEM FAILURE | The request can not be handled because of a general problem in the location server or the underlying network |
| 2 | UNAUTHORIZED NETWORK | The requesting network is not allowed to access the location server |
| 3 | UNAUTHORIZED APPLICATION | The requesting location-based application is not allowed to access the location server |
| 4 | UNKNOWN SUBSCRIBER | Unknown subscriber. The user is unknown, i.e. no such subscription exists. |
| 5 | ABSENT SUBSCRIBER | Absent subscriber. The user is currently not reachable. |
| 6 | POSITION METHOD FAILURE | Position method failure. The location service failed to obtain the user’s position. |
| 101 | CONGESTION IN LOCATION SERVER | The request can not be handled due to congestion in the location server |
| 102 | CONGESTION IN MOBILE NETWORK | The request can not be handled due to congestion in the mobile network |
| 103 | INCORRECT PASSWORD | The location-based application is allowed to access the location server, but the supplied password is incorrect. |
| 104 | TOO MANY POSITION ITEMS | Too many position items have been specified in the request. |
| 105 | FORMAT ERROR | A parameter in the request has invalid format. The invalid parameter is indicated in ADD_INFO. |
| 106 | SYNTAX ERROR | The position request has invalid syntax. Details may be indicated in ADD_INFO. |
| 107 | PROTOCOL ELEMENT NOT SUPPORTED | An element specified in the position request is not supported by the Location Server. The element is indicated in ADD_INFO. |
| 108 | SERVICE NOT SUPPORTED | The requested service is not supported in the Location Server. The service may be indicated in ADD_INFO |
| 109 | ELEMENT ATTRIBUTE NOT SUPPORTED | An element attribute is not supported in the Location Server. The attribute is indicated in ADD_INFO |
| 110 | INVALID TIME RANGE | The time range specified is incorrect. The incorrect time is indicated in ADD_INFO |
| 201 | UNKNOWN SUBSCRIBER | The user is unknown, i.e. no such subscription exists |
| 202 | NOT IN PRIVACY EXCEPTION LIST | The requesting application not in the privacy exception list of the MS. |
| 203 | CALL TO USER NOT SETUP | The requesting application has not a call set up to an MS that only allows call related location requests. |
| 204 | DISALLOWED BY LOCAL REGULATIONS | The location request is disallowed by local regulatory requirements |
| 207 | MISCONFIGURATION OF LOCATION SERVER | The location server is not completely configured to be able to calculate a position. |

# References (Normative and Informative)

- Hypertext Transfer Protocol –HTTP/1.1
RFC 2068, June 1999
Available at http://www.w3c.org

- The TLS Protocol Version 1.0
RFC 2246, January 1999
Available at http://www.ietf.org

- Extensible Markup Language (XML) 1.0
W3C Recommendation: REC-xml-19980210
Available at http://www.w3c.org

- Internet Assigned Numbers Authority (IANA)
http://www.iana.org/

- US-ASCII. Coded Character Set - 7-Bit American Standard Code for Information Interchange. Standard ANSI X3.4-1986, ANSI, 1986.

- GSM 02.71: "Digital cellular telecommunications system (Phase 2+); Location Services (LCS); Service description; Stage 1".

- GSM 03.71: "Digital cellular telecommunications system (Phase 2+); Location Services (LCS); Functional description; Stage 2".

- GSM 09.02: "Digital cellular telecommunications system (Phase 2+); Mobile Application Part (MAP) specification".

- 3G TS 22.071: "Location Services (LCS); Service description, Stage 1".

- 3G TS 23.171: "Functional stage 2 description of location services in UMTS"

- Parlay Mobility Service Interface version 1.1 that is a part of version 2.1 of the Parlay API Specification.
Available on the Parlay web-site at http://www.parlay.org

# Appendix A (informative)

The terminology mapping table with 3GPP LCS Specifications

The following is the list of the terms in MLP used differently from the ones defined for 3GPP.

| Term |  | notes |
| --- | --- | --- |
| MLP | 3GPP |  |
| Location Server | LCS Server |  |
| MS (Mobile Station) | UE |  |
| MSID (Mobile Station Identifer) | Identification of the target UE |  |
| MPC (Mobile Positioning Center) | － | There is no term which is applicable to 3GPP. |

# Appendix B (informative)

The corresponding terms used for the location procedures in 3GPP LCS Definition

The following is the list of terms defined in MLP corresponding to the 3GPP LCS definition in TS23.271 for the location procedures.

| Location procedures defined with 3GPP(23.271) |  | Services defined with MLP |
| --- | --- | --- |
| Circuit Switched Mobile Terminating Location Request CS-MT-LR | LCS Service Request | Standard Location Immediate Request |
|  | LCS Service Responce | Standard Location Immediate Answer |
| CS-MT-LR without HLR Query - applicable to North America Emergency Calls only | LCS Service Request | Emergency Location Immediate Request |
|  | LCS Service Responce | Emergency Location Immediate Answer |
| Packet Switched Mobile Terminating Location Request PS-MT-LR | LCS Service Request | Standard Location Immediate Request |
|  | LCS Service Responce | Standard Location Immediate Answer |
| Network Induced Location Request NI-LR | Location Information | Emergency Location Report |
| Packet Switched Network Induced Location Request PS-NI-LR | Location Information | Emergency Location Report |
| Mobile Terminating Deferred Location Request | LCS Service Request | Triggered Location Reporting Request |
|  | LCS Service Responce (Provide Subscriber Location ack) | Triggerrd Location Reporting Answer |
|  | LCS Service Responce (Subscriber Location Report) | Triggered Location Report |
| Cancellation of a Deferred Location Request | LCS Cancel Service Request | Triggered Location Reporting Stop |
|  | LCS Cancel Service Response | Triggered Location Reporting Stop Answer |
| Mobile Originating Location Request, Circuit Switched CS-MO-LR | Location Information | Standard Location Report |
| Mobile Originating Location Request, Packet Switched PS-MO-LR | Location Information | Standard Location Report |