TR-41.3.4/99-08-xx

# Telecommunications Industry Association

# (TIA)

SUBCOMMITTEE CONTRIBUTION

Technical Subcommittee TR-41.3.4

Source:

Pingtel Corp.

Rich Schaaf, rschaaf@pingtel.com, (781) 938-5306

Dan Petrie, dpetrie@pingtel.com, (781) 938-5306

Title:	Performance and Interoperability Requirements for Voice-over-IP [VoIP] Telephones

Project:		PN-4462

Distribution:	TR-41.3.4 Attendees

- PN-4462

- Telecommunications

- Telephone Terminal Equipment

Performance and Interoperability Requirements for Voice-over-IP [VoIP] Telephones

Version 1.1+

July 28, 2026

Formulated under the cognizance of TIA Engineering Committee TR-41,

User Premises Telecom Requirements

# Revision History

| Date | Version | Description |
| --- | --- | --- |
| February 15, 1999 | 1.0 | Initial Revision, Sarasota, Florida |
| April 19, 1999 | 1.1 | Interim Meeting, Vancouver, BC - Added MGCP requirements. - Added Display and Keypad requirements. - Modified introduction and scope. - Added some definitions and abbreviations. - Reorganized acoustic performance requirements. - Changed Ethernet Requirements as per February meeting discussion and connector-type mailing list discussion. - Changed Internet Protocol Requirements for improved readability; UDP is required, TCP is optional. - Rolled in Nortel Networks’ MGCP contribution. - Made changes to bring the document in line with TIA style guidelines. |
| August 14, 1999 | 1.1+ | For Ottawa meeting: - Added SIP requirements. |

# Table of Contents

Revision History	iii

Table of Contents	1

Foreword	2

1	Introduction	3

2	Scope	4

3	Normative References	5

4	Definitions, Abbreviations and Acronyms	7

4.1	Definitions	7

4.1.1	MGCP Telephone	7

4.1.2	SIP Telephone	7

4.1.3	VoIP Telephone	7

4.2	Abbreviations and Acronyms	7

5	Technical Requirements	8

5.1	Display and Keypad Requirements	8

5.2	Acoustic Performance Requirements	8

5.3	Ethernet Requirements	8

5.4	Internet Protocol Requirements	8

5.5	Call Control Protocol Requirements	8

5.5.1	MGCP Requirements	8

5.5.2	H.323 Requirements	8

5.5.3	SIP Requirements	8

5.6	Safety	11

5.7	Electromagnetic Compatibility	11

5.8	Powering	11

5.9	Environmental Considerations	11

# Foreword

# Introduction

# Scope

  - This standard specifies requirements for the interoperability, acoustic performance, supplementary services support, safety, electromagnetic compatibility, and environmental performance of VoIP Telephones.

  - This standard applies specifically to VoIP Telephones that are connected to Ethernet networks.

  - This standard harmonizes, wherever possible, with IETF Megaco MGCP protocols for control of IP phones [xx].

  - This standard supports the use of H.323 procedures for call establishment and call control.

  - This standard supports the use of the Session Initiation Protocol (SIP) for call establishment and call control [4].

# Normative References

- IETF RFC2543, SIP: Session Initiation Protocol

- IETF draft-ietf-mmusic-sip-cc-01.txt, SIP Call Control Services

- IETF draft-ietf-pint-protocol-01.txt, The PINT Service Protocol: Extensions to SIP and SDP for IP Access to Telephone Call Services

# Definitions, Abbreviations and Acronyms

## Definitions

### MGCP Telephone

### SIP Telephone

A telephone supporting IETF SIP as described in section 5.5.3 shall be referred to as a SIP Telephone.

### VoIP Telephone

## Abbreviations and Acronyms

# Technical Requirements

## Display and Keypad Requirements

## Acoustic Performance Requirements

## Ethernet Requirements

## Internet Protocol Requirements

## Call Control Protocol Requirements

VoIP Telephones shall support at least one of the following call control protocols:

- IETF Media Gateway Control Protocol (RFCXXXX)

- ITU H.323 Protocol.

- IETF Session Initiation Protocol

Note – This standard does not specify how more than one of the above protocols may be supported simultaneously in a particular telephone. The simultaneous support of multiple protocols is for further study.

### MGCP Requirements

### H.323 Requirements

### SIP Requirements

VoIP Telephones using SIP shall support the functionality defined in the section A.1 of the SIP RFC [4] for a Redirection Capable Client.  This level of functionality set enables SIP Telephones to:

- Establish basic calls

- Forward calls

SIP Telephones should support the functionality defined in the section A.1 of the SIP RFC [4] for an Authentication Capable Client.  This level of functionality set enables SIP Telephones to:

- Establish calls through firewalls (in conjunction with a firewall that either proxies the RTP streams or opens holes in the firewall for the RTP streams).

- Participate in terminal and media negotiation

- Establish calls to parties that require caller authentication

#### Required Features

Table 1Table 1 shows the telephone capabilities that shall be supported and how those capabilities will be implemented by SIP Telephones.

Table 1 Required SIP Telephone Features and their Implementation Methods

| Feature Name | Implementation Method |
| --- | --- |
| Make a Call | As specified in the SIP RFC [4]. |
| Receive a Call | As specified in the SIP RFC [4]. |
| Call Hold | No SIP messaging is involved. The telephone keeps the RTP session alive and either sends or indicates silence to the far end of the connection. |
| Transfer | A standard implementation method for transfer does not yet exist for SIP. One possibility that is being considered by the IETF involves adding support for the “Also” and “Requested-By” header fields as specified in [31]. |
| Call Forwarding | As specified in the SIP RFC [4]. |
| Three-Way Conference | Vendors may choose to set up a three-way conference as either a bridged or fully-meshed conference call. In a bridged call, the SIP telephone setting up the conference acts as a conference bridge, mixing the audio media for the call. The other telephones in the call are either added directly or joined via consultative calls on the bridging phone. This type of three-way conference is directly supported by the SIP RFC [4]. In a fully-meshed conference call, each phone in the call sends and receives audio media from all of the other phones in the call. A standard implementation method for setting up fully-meshed conference calls does not yet exist for SIP. One possibility that is being considered by the IETF involves adding support for the “Also” header field as specified in [31]. |
| Caller ID Indication | As specified in the SIP RFC [4] using information in the “From” header field. |
| Message Waiting | A standard implementation method for message waiting does not yet exist for SIP. However, the IETF PINT working group is defining a SUBSCRIBE/NOTIFY extension to SIP [32] that could be used to implement Message Waiting. The SUBSCRIBE/NOTIFY mechanism could be used as follows: On initialization, the SIP telephone SUBSCRIBEs for the voice mail state for its user's SIP URL (e.g., the phone extension) with the relevant subscription server. The voice mail server and the subscription server may be distinct entities. The server sends the current state of the user's voice mail account (i.e., N new messages, where N is 0 or more) in a NOTIFY message. The server sends additional NOTIFY messages as the state changes (e.g., 0 new messages after the user listens to voice mail). |
| Redial | For SIP Telephones, the telephone remembers the last number that was dialed. Since the Redial feature does not require any non-local information, there are no interoperability issues and the details of the implementation of this feature are vendor-specific. |
| Speed Dial | For SIP Telephones, the set of speed dial numbers shall be stored in the telephone. Since the Speed Dial feature does not require any non-local information, there are no interoperability issues and the details of the implementation of this feature are vendor-specific. |

#### Recommended Features

Table 2 shows the telephone capabilities that should be supported and, if supported, how those capabilities will be implemented by SIP Telephones.

Table 2 Recommended SIP Telephone Features and their Implementation Methods

| Feature Name | Implementation Method |
| --- | --- |
| Transfer with Consultation | A standard implementation method for transfer with consultation does not yet exist for SIP. One possibility that is being considered by the IETF involves adding support for the “Also” and “Requested-By” header fields as specified in [31]. |
| Multiple Call Appearances | As specified in the SIP RFC [4]. The details of how additional incoming calls are brought to the attention of the user and how many call appearances a SIP Telephone can support pose no interoperability issues and are vendor-specific. |
| Call Park | A standard implementation method for call park does not yet exist for SIP. However, the IETF PINT working group is defining a SUBSCRIBE/NOTIFY extension to SIP [32] that could be used to implement Call Park. The SUBSCRIBE/NOTIFY mechanism could be used as follows: A call is already set up on a phone. The user wishes to park the call. The first step is to put the call on hold. Another entity (i.e., phone or server) which is interested in receiving parked call notifications for that phone, SUBSCRIBEs to the phone. As a result, that entity will receive notifications of current or future parked call states. As a call is currently parked at the phone the subscribing entity(s) is sent a NOTIFY message containing callId, To, From information and call state. |
| Call Pickup of a Ringing Call | Supported under the SIP RFC [4] as follows. Call Pickup for Station A phone is ringing. A user at another phone indicates the desire to pickup the ringing call at a specific phone. The user’s phone REGISTERs using the ringing phone's SIP URL (e.g., extension). The registry server (if different from the proxy server) notifies the proxy server for the ringing phone of the registration. The proxy server forks (or cancels the INVITE to the ringing phone) and INVITEs the user's phone to the call. Call Pickup for Group For the group case, the user's phone REGISTERs the group name (as opposed to the user/extension name for the call at the ringing phone). The registry and/or proxy server must understand the mapping (through configuration or other means) from group name to user name in the SIP URL. The rest of the call setup is the same as for the ringing station case. |
| Call Pickup of a Parked Call | A standard implementation method for call pickup of a parked call does not yet exist for SIP. However, the IETF PINT working group is defining a SUBSCRIBE/NOTIFY extension to SIP [32] that can be used along with “Replaces” header field as defined in the call control extension [31] to implement Call Pickup of a Parked Call. The SUBSCRIBE/NOTIFY mechanism could be used as follows: Call Pickup For Station When a user requests a call pickup for a specific SIP URL (e.g., extension), the user’s phone SUBSCRIBEs for calls parked at the designated URL. The user’s phone receives a NOTIFY message containing the data (callId, To, From, etc.) from the parked call. The user’s phone then sends an INVITE to the phone with the parked call using the same callId, as the parked call. The call control extension field “Replaces” [31] is used to indicate that the user’s phone should be added to the call and the phone with the parked call should be dropped from the call. Call Pickup for Group For the group case, the user requests a call pickup from a group of phones that contain the parked call. This case requires that an entity (park service) aggregate the NOTIFY messages for parked calls and works as follows: - On startup, the park service subscribes to all of the phones in the park group. - To pickup a call in a group, the user’s phone SUBSCRIBEs with the park service for the relevant group. The park service responds to the subscription request with the URLs of all of the calls that are currently parked for the group. The user’s phone selects one of these calls and then uses the “Call Pickup for Station” procedure (detailed above) to pickup that call. |

## Safety

## Electromagnetic Compatibility

## Powering

## Environmental Considerations

The contributors grant a free, irrevocable, license to the Telecommunications Industry Association (TIA) to incorporate text contained in this contribution and any modifications thereof in the creation of a TIA standards publication; to copyright in TIA's name any standards publication even though it may include portions of this contribution; and at TIA's sole discretion to permit others to reproduce in whole or in part the resulting TIA standards publication.